3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2102721

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1744 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on Active TCI state switching for NR-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2020-12-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The issues for the case when THARQ > TRRCProcessing for RRC based TCI switch has been discussed in Rel-15 session in the previous meeting. The clarification was added that longer delay is allowed when THARQ > TRRCProcessing. The similar changes are needed for RRC-based TCI switch for NR-U. A NR-U specific issue is whether to consider the CCA failure for THARQ . To avoid unnesssary inter-layer operation issue, it should only consider the case as NR without extension due to CCA failure to keep the same requirements when no CCA failure happens. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the clarification that a longer switching delay is allowed when THARQ > TRRCProcessing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10A.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

8.10A.5	RRC based TCI state switch delay

If the target TCI state is known, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing  +TOk*(Tfirst-SSB + TSSB-proc+ TSSB*LRRC,known)) / NR slot length, where TRRC_processing is the RRC processing delay defined in Clause 12 of TS38.331 [2], Tfirst-SSB, TSSB-proc,TOk, TSSB are as defined in clause 8.10A.3. The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period.

Tfirst-SSB is time to first SSB transmission occasion after RRC processing by the UE, during which some of the SSB occasions may not be availabledue to DL CCA failures;

The SSB shall be the QCL-TypeA or QCL-TypeC to target TCI state;

LRRC,known≤ LRRC,known,max is the corresponding number of SSB occasions not available at the UE;

LRRC,known,max =2 for TSSB≤40 ms, LRRC,known,max =1 for TSSB>40 ms.

If the target TCI state is unknown, upon receiving PDSCH carrying RRC activation command at slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing  + TOuk*(Tfirst-SSB+ TSSB-proc+TSSB*LRRC,unknown) ) / NR slot length, where TRRC_processing is the RRC processing delay defined in Clause 12 of TS38.331 [2], TOuk,  TSSB-proc, TSSB are as defined in clause 8.10A.3. The UE is not required to receive PDCCH/PDSCH/CSI-RS or transmit PUCCH/PUSCH until the end of switching period.

Tfirst-SSB is time to first SSB transmission occasion after L1-RSRP measurement when TCI state switching involves QCL-TypeD, during which some SSB occasions may not be available at the UE due to DL CCA failures;

Tfirst-SSB is time to first SSB transmission occasion after RRC processing time at the UE for other QCL types, during which some SSB occasions may not be available at the UE due to DL CCA failures;

LRRC,unknown≤LRRC,unknown,max is the corresponding number of SSB occasions not available at the UE;

LRRC,unknown,max = 2  for TSSB ≤40 ms, LMAC,unknown,max = 1 for TSSB>40 ms.

The requirements for RRC based TCI state switch delay apply when only 1 TCI state is configured in RRC TCI state list. When  THARQ> TRRC_processing a longer switching delay is allowed. Where THARQ is the time between DL data transmission and acknowledgement as specified in TS 38.213 [3].

<Start of Change 1>