3GPP TSG-RAN4 Meeting #98-e	R4-2102735

# Electronic Meeting, Jan 25 – February 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7062 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to idle more requirements in 36133 R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-12-30 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 4.2.2 defines the requirements for high priority carrier seach when UE is in LTE Idle mode, and it includes E-UTRA high priority carriers and high priority carriers of other RATs. However, NR RAT is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NR as one of the inter-RAT targets for high priority carrier seach when UE is in LTE Idle mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Number of NR high priority carriers is not counted in Thigher_priority_search. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.2.2	Requirements

The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds when the UE is not configured with eDRX_IDLE cycle, and at least every Thigher_priority_search = MAX(60 * Nlayers, one eDRX_IDLE cycle) when UE is configured with eDRX_IDLE cycle, where Nlayers is the total number of configured higher priority E-UTRA, UTRA FDD, UTRA TDD, CDMA2000 1x, HRPD and NR carrier frequencies and is additionally increased by one if one or more groups of GSM frequencies is configured as a higher priority.

In the requirements of Section 4.2.2 for the UE capable of CA, the applicable exceptions for side conditions are specified in Annex B, Section B.4.2.

For a UE which supports increased carrier monitoring E-UTRA or increased carrier monitoring UTRA, the reselection performance for different carriers may be configured by higher layers to be either normal or reduced. The following definitions are used in the requirements:

Kcarrier : Total number of interfrequency carriers in the neighbour cell list

Kcarrier,normal =Kcarrier- Kcarrier,reduced: Number of interfrequency carriers to be monitored in the normal performance group

Kcarrier,normal,FDD : Number of interfrequency FDD carriers to be monitored in the normal performance group

Kcarrier,normal,TDD : Number of interfrequency TDD carriers to be monitored in the normal performance group

Kcarrier,reduced : Number of interfrequency carriers to be monitored in the reduced performance group

NUTRA_carrier: Total number of configured UTRA FDD carriers in the neighbour cell list

NUTRA_carrier,normal= NUTRA_carrier - NUTRA_carrier,reduced: Number of UTRA FDD carriers to be monitored in the normal performance group

NUTRA_carrier,reduced: Number of UTRA FDD carriers to be monitored in the reduced performance group

NUTRA_carrier_TDD : Total number of configured UTRA TDD carriers in the neighbour cell list

NUTRA_carrier_TDD,normal= NUTRA_carrier_TDD - NUTRA_carrier_TDD,reduced: Number of UTRA TDD carriers to be monitored in the normal performance grop

NUTRA_carrier_TDD,reduced: Number of UTRA TDD carriers to be monitored in the reduced performance group

The minimum performance requirements for a UE which supports Increased UE carrier monitoring E-UTRA [2, 31] are calculated as defined in section 4.2.2.4 provided that Kcarrier,normal ≤3 for a UE capable of either FDD E-UTRA carrier monitoring or TDD E-UTRA carrier monitoring or Kcarrier,normal ≤6  for a UE capable of both FDD and TDD E-UTRA carrier monitoring provided Kcarrier,normal,FDD ≤3  and Kcarrier,normal,TDD ≤3 and the minimum performance requirements for a UE which supports Increased UE carrier monitoring UTRA [2, 31] are calculated as defined in section 4.2.2.5 provided that NUTRA_carrier_normal≤3 and NUTRA_carrier_TDD,normal≤3. In case the limits for the number of normal performance carriers is exceeded considering the broadcast neighbour cell list and the bands supported by the UE, the UE which supports Increased UE carrier monitoring E-UTRA shall measure at least 3 interfrequency carriers with normal performance and the UE which supports Increased UE carrier monitoring UTRA shall measure at least 3 UTRA carriers with normal performance. For a UE capable of monitoring E-UTRAN FDD and TDD carriers, in case the limits for the number of normal performance carriers is exceeded considering the broadcast neighbour cell list and the bands supported by the UE, the UE shall measure at least 3 FDD and 3 TDD E-UTRAN interfrequency carriers with normal performance. Additionally, reduced performance requirements shall be met for carriers for which the Reduced measurement performance IE is indicated, up to the UE measurement capability in section 4.2.2.9a.The minimum performance requirements for a UE which does not support Increased UE carrier monitoring E-UTRA [2,31] are calculated assuming all E-UTRA carriers required to be monitored for such UE, are having normal performance and are in normal performance group, i.e. Kcarrier,normal =Kcarrier and Kcarrier,reduced=0. The minimum performance requirements for a UE which does not support Increased UE carrier monitoring UTRA [2,31] are calculated assuming all UTRA carriers required to be monitored for such UE, are having normal performance and are in normal performance group, i.e. NUTRA_carrier,normal= NUTRA_carrier, NUTRA_carrier_TDD,normal= NUTRA_carrier_TDD  and NUTRA_carrier,reduced =0 and NUTRA_carrier_TDD,reduced =0. No reduced performance carrier requirement applies to a UE not supporting Increased UE carrier monitoring E-UTRA or UTRA [2, 31].  Capabilities for number of carriers to monitor for a UE which does not support Increased carrier monitoring E-UTRA or Increased carrier monitoring UTRA are specified in section 4.2.2.9

<End of Change 1>