3GPP TSG-RAN WG4 Meeting#98             	R4-2103084

E-meeting, 21st Jan – 5th Feb, 2021

Title:	TP to TR 36.717-03-02: Addition of CA_n25-n41-n71

Source:	Nokia, T-Mobile USA

Agenda item:	9.11.2

Document for:	Approval

# 1	Introduction

This is a TP into TR 36.717-03-02 to introduce CA_n25A-n41A-n71A, CA_n25A-n41(2A)-n71A and CA_n25A-n41C-n71A.

************************************* Start of TP*****************************************

### 5.X.1	CA_n25-n41-n71

#### 5.X.1.1		Operating bands for CA

Table 5.X.1.1-1: CA band combination of band n25+n41+n71

| NR Band | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n25-n41-n71 | n25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 5.X.1.2		Channel bandwidths per operating band for CA

Table 5.X.1.2-1: Supported channel bandwidths per CA configuration for band n25+n41+n71

| NR CA configuration | Uplink CA configuration | NR Band | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n25A-n41A-n71A | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 |  | 10 | 15 | 20 |  | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41(2A)-n71A | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 | See CA_n41(2A) bandwidth combination set 1 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
| CA_n25A-n41C-n71A | CA_n25A-n41A CA_n41A-n71A CA_n25A-n71A | n25 | 5 | 10 | 15 | 20 | 25 | 30 | 40 |  |  |  |  |  |  | 1 |
|  |  | n41 | See CA_n41C bandwidth combination set 1 in Table 5.5A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n71 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |

#### 5.X.1.3	Co-existence studies

For 3DL/2UL NR CA, only the IMD issues due to dual uplink operation of two bands falling into the DL of the third band shall be verified.

The co-existence studies for dual uplink operation of two bands, i.e. CA_n25A-n41A, CA_n25A-n71A and CA_n41A-n71A have been captured in TR38.716-02-00, where:

- IMD2 and IMD5 products produced by CA_n25A-n41A impact the reference sensitivity of NR band n71.

- IMD2 and IMD4 products produced by CA_n25A-n71A impact the reference sensitivity of NR band n41.

- IMD2 products produced by CA_n41A-n71A impact the reference sensitivity of NR band n25.

#### 5.X.1.4	REFSENS requirements

According to clause 5.X.1.3, MSD specification is necessary. MSD values are re-used from similar band combinations. IMD2 is taken from DC_1A_n28A-n41A, DC_1A-7A_n28A, DC_2A-71A_n38A and IMD5 from DC_1A-28A_n7A. It is proposed not to specify MSD for IMD4 as with real channel allocations only part of the IMD hits victim band.

Table 5.X.1.4-1: MSD

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n25A-n41A-n71A CA_n25A-n41(2A)-n71A CA_n25A-n41C-n71A | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n41 | 2525 | 10 | 50 | 2525 | N/A | TDD | N/A |
|  | n71 | 691 | 5 | 25 | 645 | 29.3 | FDD | IMD2 |
|  | n25 | 1880 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n41 | 2571 | 10 | 50 | 2571 | 30 | TDD | IMD2 |
|  | n71 | 691 | 5 | 25 | 645 | N/A | FDD | N/A |
|  | n25 | 1860 | 5 | 25 | 1940 | 26 | FDD | IMD2 |
|  | n41 | 2620 | 10 | 50 | 2620 | N/A | TDD | N/A |
|  | n71 | 680 | 5 | 25 | 634 | N/A | FDD | N/A |
|  | n25 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n41 | N/A | N/A | N/A | N/A | N/A | N/A | IMD4 |
|  | n71 | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
|  | n25 | 1885 | 5 | 25 | 1960 | N/A | FDD | N/A |
|  | n41 | 2510 | 10 | 50 | 2510 | N/A | TDD | N/A |
|  | n71 | 681 | 5 | 25 | 635 | 4.5 | FDD | IMD5 |

************************************* End of TP*****************************************