3GPP TSG-RAN WG4 Meeting#98-e                                 R4-2103107

E-meeting,25th Jan – 5th Feb, 2021

Agenda item:	10.1.3

Source: 	ZTE Corporation,Nokia, Nokia Shanghai Bell, CATT, Qualcomm

Title: 	TP to TR 38.921 summary of simulation results

Document for:	Approval

Introduction

Since RAN4#98e meeting should be last meeting for this ITU-R related WID based the latest status report RP-202275. Until this meeting, extensive simulation results among interested companies have already been submitted, therefore this TP is to capture the summary of simulation results in the TR.

Discussion

TP to TR 38.921

< START OF CHANGE>

4.3	Co-existence simulation results

### 4.3.1	Urban Macro scenario

#### 4.3.1.1	Downlink

For Urban macro scenarios, simulation results for DL throughput loss with baseline assumption ACLR=45dBc at BS and ACS=33dBc at UE are summarized in Table 4.3.1.1-1/2/3/4.

Table 4.3.1.1-1: DL throughput loss of victim UE for 6425-7125MHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016134 | AAS based BS | Average throughput loss in % (7GHz) | 1.65 | 1.89 | 2.10 | 2.33 | 2.63 | 2.99 |
|  |  | 5%-tile throughput loss in % (7GHz) | 4.39 | 5.18 | 5.98 | 7.07 | 7.87 | 9.06 |
| Nokia R4-2014476 | AAS based BS | Average throughput loss in % (7GHz) | 1.11 | 1.28 | 1.48 | 1.70 | 1.94 | 2.21 |
|  |  | 5%-tile throughput loss in % (7GHz) | 6.22 | 6.23 | 6.24 | 7.51 | 8.99 | 9.23 |
| Huawei R4-2015678 | AAS based BS | Average throughput loss in % (7GHz) | 1.04 | 1.39 | 1.47 | 1.77 | 2.30 | 2.39 |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.00 | 4.63 | 4.96 | 6.37 | 6.78 | 7.16 |

Table 4.3.1.1-1a: DL throughput loss of victim UE for 6425-7125MHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015897 | Throughput lost at Average | 2.7 | 2.4 | 2.1 | 1.8 | 1.6 | 1.4 | 1.2 | 1.0 | 0.9 | 0.8 | 0.7 |
|  | Throughput lost at 5%-tile | 13.1 | 10.6 | 10.0 | 8.3 | 6.6 | 6.1 | 5.5 | 4.7 | 3.1 | 2.2 | 2.1 |
| CATT R4-2016777 | Throughput lost at Average | 2.945446 | 2.615785 | 2.318919 | 2.050108 | 1.808206 | 1.592518 | 1.399160 | 1.226961 | 1.073971 | 0.936971 | 0.814444 |
|  | Throughput lost at 5%-tile | 11.345302 | 10.038643 | 8.745891 | 7.593149 | 6.594121 | 5.813514 | 5.023313 | 4.235969 | 3.589495 | 3.063706 | 2.576903 |
| Qualcomm R4-2016236 | Throughput lost at Average |  |  | 3.0285 | 2.6700 | 2.3630 | 2.0876 | 1.8414 | 1.6318 | 1.4200 |  |  |
|  | Throughput lost at 5%-tile |  |  | 12.3347 | 10.6516 | 9.2041 | 7.9086 | 6.7639 | 5.8122 | 4.9061 |  |  |

Table 4.3.1.1-2: DL throughput loss of victim UE for 10-10.5GHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016134 | AAS based BS | Average throughput loss in % (7GHz) | 1.52 | 1.68 | 1.93 | 2.12 | 2.31 | 2.62 |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.36 | 3.79 | 4.79 | 5.78 | 6.41 | 7.63 |
| Nokia R4-2014476 | AAS based BS | Average throughput loss in % (7GHz) | 1.03 | 1.20 | 1.38 | 1.60 | 1.83 | 2.09 |
|  |  | 5%-tile throughput loss in % (7GHz) | 4.38 | 4.38 | 4.86% | 5.23 | 6.01 | 7.40 |
| Huawei R4-2015678 | AAS based BS | Average throughput loss in % (7GHz) | 1.21 | 1.22 | 1.30% | 1.72 | 1.99 | 2.44 |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.04 | 3.33 | 4.14% | 5.06 | 6.41 | 8.67 |

Table 4.3.1.1-2a: DL throughput loss of victim UE for 10-10.5GHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015897 | Throughput lost at Average | 2.1 | 1.9 | 1.7 | 1.4 | 1.3 | 1.1 | 0.9 | 0.8 | 0.7 | 0.6 | 0.5 |
|  | Throughput lost at 5%-tile | 11.8 | 10.0 | 9.6 | 9.4 | 7.8 | 6.3 | 4.8 | 4.7 | 4.2 | 3.9 | 3.5 |
| CATT R4-2016777 | Throughput lost at Average | 2.874567 | 2.547846 | 2.252212 | 1.986030 | 1.748135 | 1.537007 | 1.348903 | 1.181463 | 1.032390 | 0.899829 | 0.782843 |
|  | Throughput lost at 5%-tile | 9.975828 | 8.600968 | 7.468747 | 6.454207 | 5.566782 | 4.750499 | 4.068893 | 3.494199 | 2.988947 | 2.552896 | 2.076688 |
| Qualcomm R4-2016236 | Throughput lost at Average |  |  | 2.788 | 2.4497 | 2.1611 | 1.9031 | 1.6734 | 1.4787 | 1.2825 |  |  |
|  | Throughput lost at 5%-tile |  |  | 11.0936 | 9.6118 | 8.3690 | 7.2691 | 6.2895 | 5.4504 | 4.5726 |  |  |

In addition, coexistence simulation results for UE with NF 13dB is summarized in Table 4.3.1.1.3 and 4.3.1.1.4.

Table 4.3.1.1-3: 7GHz ACIR with 13dB NF [Qualcomm, R4-2016236]

| ACIR [dB] | 31 | 30 | 29 | 28 | 27 | 26 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 1.3706% | 1.5769% | 1.7811% | 2.0213% | 2.2902% | 2.5902% | 2.9408% |
| 5%-tile throughput loss (7GHz) | 4.5693% | 5.4639% | 6.3517% | 7.4031% | 8.5919% | 9.9359% | 11.5323% |

Table 4.3.1.1-4: 10GHz ACIR with 13dB NF [Qualcomm, R4-2016236]

| ACIR [dB] | 31 | 30 | 29 | 28 | 27 | 26 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (10GHz) | 1.2320% | 1.4223% | 1.6112% | 1.8344% | 2.0853% | 2.3663% | 2.6961% |
| 5%-tile throughput loss (10GHz) | 3.2501% | 3.9646% | 4.7371% | 5.6903% | 6.7960% | 8.0620% | 9.5717% |

Based on the above simulation results, the required DL ACIR for 6.425-7.125GHz and 10.0-10.5 GHz for urban macro scenarios are in summarized in Table 4.3.1.1-5.

Table 4.3.1.1-5: DL simulation results for 6.425-7.125GHz and 10.0-10.5 GHz

|  | ACIR (dB) |  |
| --- | --- | --- |
| Company | 6.425-7.125GHz | 10.0-10.5GHz |
|  | Urban macro uncoord. | Urban Macro uncoord. |
| CATT | 29.5 | 28 |
| Nokia | 32.7 | 30.7 |
| Huawei | 30.7 | 29.7 |
| Ericsson | 30 | 29 |
| ZTE | 31.7 | 29.7 |
| Qualcomm | 30.9 | 30.5 |
| Average | 30.9 | 29.6 |
| Average after removing highest and lowest values | 30.9 | 29.5 |

#### 4.3.1.2	Uplink

For Urban macro scenarios, simulation results for UL throughput loss with baseline assumption ACLR=30dBc at UE and ACS=46dBc at BS are summarized in Table 4.3.1.2-1/2/3/4.

Table 4.3.1.2-1: UL throughput loss of victim BS for 6425-7125MHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016135 | AAS based BS | Average throughput loss in % (7GHz) | 0.99 | 1.09 | 1.23 | 1.40 | 1.55 | 1.76 |
|  |  | 5%-tile throughput loss in % (7GHz) | 2.72 | 2.84 | 5.28 | 6.01 | 6.71 | 7.20 |
| Nokia R4-2014477 | AAS based BS | Average throughput loss in % (7GHz) | 1.08 | 1.24 | 1.43 | 1.65 | 1.89 | 2.15 |
|  |  | 5%-tile throughput loss in % (7GHz) | 1.01 | 1.68 | 3.14 | 3.98 | 5.76 | 8.08 |
| Huawei R4-2015679 | AAS based BS | Average throughput loss in % (7GHz) | 0.64 | 0.69 | 0.69 | 0.92 | 1.01% | 1.07 |
|  |  | 5%-tile throughput loss in % (7GHz) | 3.8 | 4.3 | 5.0 | 6.2 | 7.6 | 8.7 |

Table 4.3.1.2-1a: UL throughput loss of victim BS for 6425-7125MHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015898 | Throughput lost at Average | 1.5 | 1.3 | 1.1 | 1.0 | 0.8 | 0.7 | 0.6 | 0.5 | 0.5 | 0.4 | 0.3 |
|  | Throughput lost at 5%-tile | 6.3 | 6.2 | 5.5 | 4.3 | 2.3 | 1.6 | 1.5 | 1.4 | 0.8 | 0.6 | 0.5 |
| CATT R4-2016778 | Throughput lost at Average | 1.489821 | 1.301164 | 1.134213 | 0.986689 | 0.856389 | 0.741773 | 0.641199 | 0.553419 | 0.476770 | 0.409968 | 0.351849 |
|  | Throughput lost at 5%-tile | 7.283069 | 6.142583 | 5.227730 | 4.495608 | 3.745409 | 3.051767 | 2.429319 | 1.621495 | 0.968716 | 0.671695 | 0.526234 |
| Qualcomm R4-2016601 | Throughput lost at Average | 1.5 | 1.2 | 1.0 | 0.9 | 1.0 | 0.9 | 0.8 | 0.6 | 0.5 |  |  |
|  | Throughput lost at 5%-tile | 4.3 | 3.4 | 2.4 | 2.2 | 2.4 | 2.2 | 1.8 | 1.5 | 1.3 |  |  |

Table 4.3.1.2-2: UL throughput loss of victim BS for 10-10.5GHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2016135 | AAS based BS | Average throughput loss in % (7GHz) | 0.79 | 0.87 | 1.11 | 1.23 | 1.41 | 1.59 |
|  |  | 5%-tile throughput loss in % (7GHz) | N/A | N/A | N/A | N/A | N/A | N/A |
| Nokia R4-2014477 | AAS based BS | Average throughput loss in % (7GHz) | 0.90 | 1.04 | 1.21 | 1.39 | 1.60 | 1.84 |
|  |  | 5%-tile throughput loss in % (7GHz) | 1.98 | 2.27 | 2.28 | 3.90 | 4.00 | 6.41 |
| Huawei R4-2015679 | AAS based BS | Average throughput loss in % (7GHz) | 0.53 | 0.59 | 0.61 | 0.76 | 0.91 | 1.05 |
|  |  | 5%-tile throughput loss in % (7GHz) | 1.7 | 2.3 | 3.3 | 4.0 | 5.3 | 6.4 |

Table 4.3.1.2-2a: UL throughput loss of victim BS for 10-10.5GHz [Absolute ACIR value]

|  | ACIR (dB) | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2015898 | Throughput lost at Average | 1.3 | 1.1 | 1.0 | 0.8 | 0.7 | 0.6 | 0.5 | 0.5 | 0.4 | 0.3 | 0.3 |
|  | Throughput lost at 5%-tile | 3.0 | 2.3 | 1.3 | 1.0 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.8 | 0.2 |
| CATT R4-2016778 | Throughput lost at Average | 1.535229 | 1.345764 | 1.177367 | 1.027196 | 0.894277 | 0.777219 | 0.673935 | 0.582684 | 0.502769 | 0.432740 | 0.371237 |
|  | Throughput lost at 5%-tile | 9.496269 | 7.816610 | 6.444894 | 5.273296 | 4.180206 | 3.433665 | 3.082907 | 2.691289 | 2.325946 | 2.024768 | 1.625941 |
| Qualcomm R4-2016601 | Throughput lost at Average | 1.5 | 1.2 | 1.0 | 0.9 | 0.8 | 0.6 | 0.5 | 0.4 |  |  |  |
|  | Throughput lost at 5%-tile | 4.0 | 3.5 | 2.9 | 2.6 | 2.2 | 2.0 | 1.6 | 1.4 |  |  |  |

In addition, coexistence simulation results for UE with Tx power 20dBm is summarized in Table 4.3.1.2.3 and 4.3.1.2.4.

Table 4.3.1.2-3: ACIR results at 7GHz with 20dBm UE max Tx power [Qualcomm, R4-2016601]

| ACIR [dB] | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7 GHz) | 0.4% | 0.5% | 0.6% | 0.8% | 0.9% | 1.0% | 1.2% | 1.5% | 1.8% |
| 5%-tile throughput loss (7 GHz) | 0.8% | 1.2% | 1.6% | 1.8% | 2.2% | 2.5% | 3.4% | 4.2% | 5.0% |

Table 4.3.1.2-4: ACIR results at 10GHz with 20dBm UE max Tx power [Qualcomm, R4-2016601]

| ACIR [dB] | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (10GHz) | 0.3% | 0.4% | 0.5% | 0.6% | 0.7% | 0.8% | 1.0% | 1.2% | 1.4% | 1.6% |
| 5%-tile throughput loss (10GHz) | 0.8% | 1.2% | 1.4% | 1.8% | 2.1% | 2.3% | 3.2% | 4.0% | 4.7% | 5.6% |

Based on the above simulation results, the required UL ACIR for 6.425-7.125GHz and 10.0-10.5 GHz for urban macro scenarios are summarized in Table 4.3.1.2-5.

Table 4.3.1.2-5: UL simulation results for 6.425-7.125GHz and 10.0-10.5 GHz

|  | ACIR (dB) |  |
| --- | --- | --- |
| Company | 6.425-7.125GHz | 10.0-10.5GHz |
|  | Urban macro uncoord. | Urban Macro uncoord. |
| CATT | 26 | 26.2 |
| Nokia | 26.9 | 25.9 |
| Huawei | 27.9 | 25.9 |
| Ericsson | 27 | 23 |
| ZTE | 27.9 | 24.9 |
| Qualcomm | 221 | 22 / 21.51, 2 |
| Average value | 26.3 | 24.6 |
| Average after removing highest and lowest values | 27 | 24.9 |
| Note 1: 3 UL UEs are assumed, where the same antenna gain derived by the BS beam forming pattern modelling described in subclause 8.1.1 is used for all UEs. Note 2: 22 with 23dBm UE maximum Tx power and 21.5 with 20dBm UE maximum Tx power. |  |  |

### 4.3.2	Indoor scenario

#### 4.3.2.1	Downlink

For indoor scenarios, simulation results for DL throughput loss with baseline assumption ACLR=45dBc at BS and ACS=33dBc at UE are summarized in Table 4.3.2.1-1/1a/2/2a/3/3a/4/4a.

Table 4.3.2.1-1: DL throughput loss of victim UE for 6425-7125MHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101951 | Omni based BS | Average throughput loss in % (7GHz) | 0.1371 | 0.1724 | 0.2168 | 0.2726 | 0.3425 | 0.4302 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.1429 | 0.1798 | 0.2263 | 0.2847 | 0.3123 | 0.3427 |
| Nokia R4-2101794 | Omni based BS | Average throughput loss in % (7GHz) | 0.13 | 0.17 | 0.21 | 0.26 | 0.33 | 0.41 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.07 | 0.09 | 0.11 | 0.14 | 0.20 | 0.33 |

Table 4.3.2.1-1a: DL throughput loss of victim UE for 6425-7125MHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102154 | Throughput lost at Average |  |  |  |  |  | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.4 |
|  | Throughput lost at 5%-tile |  |  |  |  |  | 0.9 | 1.1 | 1.4 | 1.8 | 2.3 | 2.8 |
| CATT R4-2100490 | Throughput lost at Average |  |  |  |  |  |  | 3.198061 | 3.945391 | 4.852136 | 5.946106 | 7.258307 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 1.970774 | 2.470131 | 3.106864 | 3.901198 | 4.918388 |
| Qualcomm R4-2102498 | Throughput lost at Average | 0.59 | 0.76 | 0.9 | 1.1 | 1.4 | 1.8 | 2.3 | 2.9 | 3.6 | 4.4 | 5.4 |
|  | Throughput lost at 5%-tile | 0.31 | 0.39 | 0.44 | 0.53 | 0.66 | 0.8 | 1.1 | 1.4 | 1.9 | 2.5 | 3.1 |

Table 4.3.2.1-2: DL throughput loss of victim UE for 6425-7125MHz with AAS antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101951 | AAS based BS | Average throughput loss in % (7GHz) | 0.3043 | 0.3817 | 0.4775 | 0.5947 | 0.7378 | 0.9123 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0054 | 0.0067 | 0.0085 | 0.0107 | 0.0135 | 0.1421 |
| Huawei R4-2101497 | AAS based BS | Average throughput loss in % (7GHz) | 0.15 |  | 0.30 |  | 0.36 |  |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.02 |  | 0.06% |  | 0.07 |  |
| Nokia R4-2101794 | AAS based BS | Average throughput loss in % (7GHz) | 0.27 | 0.33 | 0.42 | 0.52 | 0.65 | 0.81 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.05 | 0.06 | 0.08 | 0.10 | 0.12 | 0.16 |

Table 4.3.2.1-2a: DL throughput loss of victim UE for 6425-7125MHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102154 | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT R4-2100490 | Throughput lost at Average |  |  |  |  |  |  | 4.311783 | 5.036284 | 5.868329 | 6.803309 | 7.846610 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 0.988785 | 1.284747 | 1.673493 | 2.221040 | 2.921901 |
| Qualcomm R4-2102498 | Throughput lost at Average | 1.27 | 1.57 | 1.88 | 2.27 | 2.72 | 3.24 | 3.87 | 4.52 | 5.29 | 6.16 |  |
|  | Throughput lost at 5%-tile | 0.17 | 0.26 | 0.34 | 0.46 | 0.60 | 0.77 | 1 | 1.2 | 1.6 | 1.9 |  |

Table 4.3.2.1-3: DL throughput loss of victim UE for 10-10.5GHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101951 | Omni based BS | Average throughput loss in % (7GHz) | 0.1561 | 0.1956 | 0.2447 | 0.3058 | 0.3815 | 0.4750 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.1096 | 0.1380 | 0.1736 | 0.2369 | 0.3323 | 0.4823 |
| Nokia R4-2101794 | Omni based BS | Average throughput loss in % (7GHz) | 0.13 | 0.17 | 0.21 | 0.26 | 0.33 | 0.41 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.09 | 0.11 | 0.13 | 0.16 | 0.19 | 0.23 |

Table 4.3.2.1-3a: DL throughput loss of victim UE for 10-10.5GHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102154 | Throughput lost at Average |  |  |  |  |  | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.4 |
|  | Throughput lost at 5%-tile |  |  |  |  |  | 0.9 | 1.1 | 1.4 | 1.8 | 2.3 | 2.8 |
| CATT R4-2100490 | Throughput lost at Average |  |  |  |  |  |  | 3.122018 | 3.855086 | 4.745063 | 5.820303 | 7.109105 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 1.938403 | 2.462824 | 3.109281 | 3.943454 | 4.927987 |
| Qualcomm R4-2102498 | Throughput lost at Average | 0.59 | 0.76 | 0.94 | 1.18 | 1.4 | 1.85 | 2.3 | 2.9 | 3.6 | 4.46 | 5.4 |
|  | Throughput lost at 5%-tile | 0.5 | 0.66 | 0.7 | 0.83 | 0.9 | 1.18 | 1.4 | 1.9 | 2.4 | 3.0 | 3.72 |

Table 4.3.2.1-4a: DL throughput loss of victim UE for 10-10.5GHz [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101951 | AAS based BS | Average throughput loss in % (7GHz) | 0.3188 | 0.3979 | 0.4950 | 0.6133 | 0.7587 | 0.9337 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0481 | 0.0553 | 0.0645 | 0.0760 | 0.0969 | 0.1978 |
| Huawei R4-2101497 | AAS based BS | Average throughput loss in % (7GHz) | 0.13 |  | 0.21 |  | 0.32 |  |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.01 |  | 0.07 |  | 0.09 |  |
| Nokia R4-2101794 | AAS based BS | Average throughput loss in % (7GHz) | 0.27 | 0.34 | 0.43 | 0.54 | 0.67 | 0.82 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.00 | 0.00 | 0.00 | 0.01 | 0.01 | 0.01 |

Table 4.3.2.1-4a: DL throughput loss of victim UE for 6425-7125MHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102154 | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT R4-2100490 | Throughput lost at Average |  |  |  |  |  |  | 4.235205 | 4.964440 | 5.793027 | 6.735404 | 7.788678 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 1.414406 | 1.703417 | 2.085369 | 2.595632 | 3.227734 |
| Qualcomm R4-2102498 | Throughput lost at Average | 1.2 | 1.5 | 1.8 | 2.2 | 2.6 | 3.1 | 3.8 | 4.4 | 5.2 | 6.0 |  |
|  | Throughput lost at 5%-tile | 0.26 | 0.3 | 0.307 | 0.31 | 0.312 | 0.314 | 0.4 | 0.64 | 0.93 | 1.28 |  |

Based on the above simulation results, the required DL ACIR for 6.425-7.125GHz and 10.0-10.5 GHz for indoor hotpot scenarios are in summarized in Table 4.3.2.1-5.

Table 4.3.2.1-5: DL simulation results for 6.425-7.125GHz and 10.0-10.5 GHz

| DL simulations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ACIR (dB) for Indoor |  |  |  | Can we keep agreed BS ACLR (38-37) and UE ACS (32-31) for indoor? |
|  | 6.425-7.125GHz |  | 10.0-10.5GHz |  |  |
| ACIR for macro BS | 31.0 |  | 30.0 |  |  |
|  | Omni | AAS | Omni | AAS |  |
| CATT | [18] | [20] | [18] | [19] | Yes |
| Huawei |  | <25.9 |  | <25.9 | Yes |
| Nokia | <ACIR -5 | <ACIR -5 | <ACIR -5 | <ACIR -5 | Yes |
| ZTE | <ACIR -5 | <ACIR -5 | <ACIR -5 | <ACIR -5 | Yes |
| Ericsson | 17 |  | 17 |  | Yes |
| Qualcomm | 16 | 18 | 16 | 18 | Yes |

#### 4.3.2.2	Uplink

For indoor scenarios, simulation results for UL throughput loss with baseline assumption ACLR=30dBc at UE and ACS=46dBc at BS are summarized in Table 4.3.2.2-1/1a/2/2a/3/3a/4/4a.

Table 4.3.2.2-1: UL throughput loss of victim BS for 6425-7125MHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101952 | Omni based BS | Average throughput loss in % (7GHz) | 0.1433 | 0.1794 | 0.2245 | 0.2805 | 0.3499 | 0.4356 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0078 | 0.0098 | 0.0123 | 0.0155 | 0.0212 | 0.0246 |
| Nokia R4-2101795 | Omni based BS | Average throughput loss in % (7GHz) | 0.30 | 0.37 | 0.46 | 0.58 | 0.73 | 0.91 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.05 | 0.08 | 0.10 | 0.12 | 0.19 | 0.27 |

Table 4.3.2.2-1a: UL throughput loss of victim BS for 6425-7125MHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102155 | Throughput lost at Average | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.3 | 6.6 | 8.1 | 9.9 |  |  |
|  | Throughput lost at 5%-tile | 1.2 | 1.5 | 1.9 | 2.4 | 3.1 | 3.9 | 4.8 | 5.8 | 7.4 |  |  |
| CATT R4-2100491 | Throughput lost at Average |  | 2.392580 | 2.954531 | 3.639646 | 4.471517 | 5.476805 | 6.684997 | 8.127856 | 9.843106 | 11.858804 | 14.204583 |
|  | Throughput lost at 5%-tile |  | 1.391922 | 1.752115 | 2.163126 | 2.636267 | 3.216128 | 3.893275 | 4.758722 | 6.027059 | 7.603577 | 9.503131 |
| Qualcomm R4-2102499 | Throughput lost at Average |  | 2.5 | 3.0 | 4.0 | 5.0 | 6.0 | 7.0 | 8.5 | 11.0 | 13.0 | 15.0 |
|  | Throughput lost at 5%-tile |  | 0.8 | 1.0 | 1.5 | 1.8 | 2.0 | 2.5 | 3.0 | 4.5 | 5.5 | 6.5 |

Table 4.3.2.2-2: UL throughput loss of victim BS for 6425-7125MHz with AAS antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101952 | AAS based BS | Average throughput loss in % (7GHz) | 0.1414 | 0.1770 | 0.2211 | 0.2759 | 0.3437 | 0.4275 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0033 | 0.0042 | 0.0053 | 0.0378 | 0.0880 | 0.1511 |
| Nokia R4-2101795 | AAS based BS | Average throughput loss in % (7GHz) | 0.16 | 0.20 | 0.24 | 0.30 | 0.38 | 0.47 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.04 | 0.05 | 0.07 | 0.08 | 0.10 | 0.13 |

Table 4.3.2.2-2a: UL throughput loss of victim BS for 6425-7125MHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102155 | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT R4-2100491 | Throughput lost at Average |  |  |  |  |  |  | 3.165202 | 3.814549 | 4.575053 | 5.458766 | 6.477323 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 2.151940 | 2.644977 | 3.314275 | 4.158239 | 5.164969 |
| Qualcomm R4-2102499 | Throughput lost at Average |  | 2.0 | 2.5 | 3 | 3.5 | 4.5 | 5.0 | 5.5 | 7.5 | 9.0 |  |
|  | Throughput lost at 5%-tile |  | 0.7 | 1.0 | 1.5 | 1.8 | 2.0 | 2.4 | 3.0 | 4.0 | 5.0 |  |

Table 4.3.2.2-3: UL throughput loss of victim BS for 10-10.5GHz with omni antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101952 | Omni based BS | Average throughput loss in % (7GHz) | 0.1510 | 0.1892 | 0.2367 | 0.2957 | 0.3689 | 0.4593 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0214 | 0.0347 | 0.0362 | 0.0382 | 0.0407 | 0.1147 |
| Nokia R4-2101795 | Omni based BS | Average throughput loss in % (7GHz) | 0.30 | 0.38 | 0.47 | 0.59 | 0.74 | 0.92 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.07 | 0.09 | 0.12 | 0.19 | 0.37 | 0.59 |

Table 4.3.2.2-3a: UL throughput loss of victim BS for 10-10.5GHz with omni antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102155 | Throughput lost at Average | 1.8 | 2.2 | 2.8 | 3.5 | 4.3 | 5.3 | 6.6 | 8.1 | 9.9 |  |  |
|  | Throughput lost at 5%-tile | 1.2 | 1.5 | 1.9 | 2.4 | 3.1 | 3.9 | 4.8 | 5.8 | 7.4 |  |  |
| CATT R4-2100491 | Throughput lost at Average |  | 2.260160 | 2.793478 | 3.445105 | 4.238153 | 5.198868 | 6.356444 | 7.742539 | 9.400835 | 11.352258 | 13.623966 |
|  | Throughput lost at 5%-tile |  | 1.537452 | 1.949692 | 2.435648 | 2.994580 | 3.642921 | 4.428238 | 5.338709 | 6.554238 | 8.002240 | 9.693387 |
| Qualcomm R4-2102499 | Throughput lost at Average |  | 2.5 | 3.0 | 4.0 | 5.0 | 6.0 | 7.0 | 9 | 11.0 | 13.0 | 15.0 |
|  | Throughput lost at 5%-tile |  | 0.8 | 1.0 | 1.5 | 1.8 | 2.2 | 2.5 | 3.5 | 5 | 5.5 | 7.0 |

Table 4.3.2.2-4: UL throughput loss of victim BS for 10-10.5GHz with AAS antenna [Relative ACIR value]

| Company | Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| ZTE R4-2101952 | AAS based BS | Average throughput loss in % (7GHz) | 0.1536 | 0.1920 | 0.2395 | 0.2983 | 0.3709 | 0.4602 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.0900 | 0.0904 | 0.0910 | 0.0918 | 0.0927 | 0.0938 |
| Nokia R4-2101795 | AAS based BS | Average throughput loss in % (7GHz) | 0.16 | 0.20 | 0.25 | 0.31 | 0.38 | 0.48 |
|  |  | 5%-tile throughput loss in % (7GHz) | 0.00 | 0.01 | 0.01 | 0.01 | 0.02 | 0.02 |

Table 4.3.2.2-4a: UL throughput loss of victim BS for 10-10.5GHz with AAS antenna [Absolute ACIR value]

|  | ACIR (dB) | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson R4-2102155 | Throughput lost at Average |  |  |  |  |  |  |  |  |  |  |  |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  |  |  |  |  |  |
| CATT R4-2100491 | Throughput lost at Average |  |  |  |  |  |  | 3.302668 | 3.975666 | 4.761144 | 5.670780 | 6.715829 |
|  | Throughput lost at 5%-tile |  |  |  |  |  |  | 2.197258 | 2.860650 | 3.679917 | 4.658938 | 5.877432 |
| Qualcomm R4-2102499 | Throughput lost at Average |  | 2.0 | 2.5 | 3.0 | 4.0 | 4.5 | 5.0 | 6.0 | 7.5 | 9.0 | 10.5 |
|  | Throughput lost at 5%-tile |  | 0.5 | 1.0 | 1.5 | 1.8 | 2.0 | 2.4 | 3.0 | 4.0 | 4.8 | 5.5 |

Based on the above simulation results, the required UL ACIR for 6.425-7.125GHz and 10.0-10.5 GHz for indoor hotpot scenarios are in summarized in Table 4.3.2.1-5.

Table 4.3.2.2-5: UL simulation results for 6.425-7.125GHz and 10.0-10.5 GHz

| UL simulations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ACIR (dB) for Indoor |  |  |  | Can we keep agreed UE ACLR (26-24) and BS ACS (42-40) for indoor? |
|  | 6.425-7.125GHz |  | 10.0-10.5GHz |  |  |
| Macro urban | 25.9 |  | 23.9 |  |  |
|  | Omni | AAS | Omni | AAS |  |
| CATT | [17] | [13] | [17] | [13] | Yes |
| Huawei |  | <23.9 |  | <23.9 | Yes |
| Nokia | <ACIR -5 | <ACIR -5 | <ACIR -5 | <ACIR -5 | Yes |
| ZTE | <ACIR -5 | <ACIR -5 | <ACIR -5 | <ACIR -5 | Yes |
| Ericsson | 17 |  | 17 |  | Yes |
| Qualcomm | 17 | 15 | 17 | 15 | Yes |

< End OF CHANGE>