3GPP TSG-RAN WG4 Meeting # 98-e 										                   R4-2103298

Electronic Meeting, 25 January – 5 February 2021

Agenda item:			4.2.3, 7.5.1, 15.2

Source:	Ericsson

Title:	Email discussion summary for [98e][108] LTE_NR_DC_CA_enh_RF

Document for:	Information

# Introduction

This email discussion concerns three topics

- Maintenance of 38.101-1, 38.101-3 and 38.307 (EN-DC and NR-DC)

- Reply LS to RAN2 on P-Max for FR2

- Single uplink operation (including action upon the LS from RAN in RP-202622)

List of candidate target of email discussion for 1st round and 2nd round

- 1st round:

  - decide on which of the maintenance CRs to pursue

  - decide on which one of the submitted draft RAN2 reply LSs to send

  - decision on how to capture single-UL only (SUO) in the RAN4 specifications and need for an LS to RAN/RAN2

- 2nd round:

  - decide on which of the maintenance CRs to pursue

  - agree on a Reply LS to RAN2 on p-NR-FR2 in R4-2103143

  - agree on a LS to RAN2 on LS to RAN2 on single-uplink operation in more than one band pair of a band combination in R4-2103144

  - decide on how to capture single-UL only (SUO) in the RAN4 specifications

    - Option 1: clarification in RAN4 specification as proposed in R4-2101718 (SUO part), no LS to RAN2

    - Option 2: LS to RAN2 on single-uplink operation in R4-2103145

# Topic #1: Maintenance of 38.101-1, 38.101-3 and 38.307

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100798 | MediaTek Inc. | MSD due to wider BW evaluation for DC_28_n5 (Background to CR in R4-2100844) |
| R4-2100844 | MediaTek Inc. | Title: CR for 38.101-3 Correction on EN-DC MSD due to cross band isolation for DC_28_n5 (R16) CR to 38.101-3 Rel-16 (Cat F) Reason for change: MSD due to introducing wider CBW was further discussed and more IMD impairment shall be considered due to wider NR channel BW. There is MSD due to CIM5 for DC_28_n5. Corrections are needed. Summary of change: Add MSD due to CIM5 due to NR wider CBW according to R4-2100798 |
| R4-2102207 | ZTE Corporation, CHTTL | Title: CR to TS 38.307 on the definition of the duplex-mode for the band configurations CR to 38.307 (Rel-15) Reason for change: In current 38.307 spec, there are no definitions for the ‘duplex-mode’ in the table. Due to there are lots of types of band configurations including ENDC, NR-CA, SUL, etc, it is necessary to add the NOTE in the table to describe the meaning of the ‘duplex-mode’ for a certain type of band configuration, especially more and more types of configurations will be added in future. Also, several ‘FDD and TDD’ inter-band ENDC for PC3 are defined in Rel-15. Summary of change: By using the similar method of TS36.307, the NOTE for each ‘duplex-mode’ in the table is added. Also duplex mode of ‘FDD and TDD’ is added for PC3 inter-band ENDC. Moderator comment: related CR in R4-2101989 under agenda 4.2.1 [102] |
| R4-2102408 | Qualcomm Incorporated | Title: Missing parent clause for NR-DC PCMAX CR to 38.101-1 (Rel-16) Reason for change: Sub-clauses 6.2B.4.1 and 6.2B.4.2 without parent clause 6.2B.4. Summary of change: Add missing parent clause |

## Open issues summary

No open issues listed, the CRs submitted are for ‘close-to-finalized Rel-16’ work (comments in sub-clause 1.3.2).

### Sub-topic 1-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 1-1: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

### Sub-topic 1-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

Comments on the CRs in the next sub-clause.

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2100844 TS 38.101-3 CR 449 Rel-16 Cat-F | Qualcomm: we need more time to analyze to check, at least defer to 2nd round. We need to investigate the filtering options. Same comment applies to discussion R4-2100798. |
|  | Skyworks: we recognize the reality of the issue, we would need to crosscheck what other implementations performance may result into. At least square brackets would be useful but we will try to provide feedback in round 2 |
|  | Huawei: Two MSD values for cross band isolation may cause ambiguity. As we discussed in last meeting, the MSD due to counter intermodulation can be used for this case. It can be captured into the mirror CR of R4-2102094 CR quality control: Work item code (NR_newRAT-Core) and Release (Rel-15) are not correct for this agenda. And the current release is 16.6.0. |
|  | Apple: we would like more time to check |
| R4-2102207 TS 38.307 CR 0055 Rel-15 Cat-F | Ericsson: overlaps with R4-2101989 (proposes to remove the duplex modes) |
|  | ZTE: We think the duplex mode is important information for band combination release independence due to there are no release independence information in the WID. Actually the duplex modes for the band combination in each release 38.307 spec are aligned with the each release 38.101 specs. |
|  | Qualcomm: we are fine with this change. CHTTL: We support this change. Nokia: There is alternative proposal from Huawei to remove duplex-mode information in R4-2101989 which would reduce maintenance need. Decision is needed whether to keep duplex mode information or not. NTT DOCOMO, INC.: We are OK with the clarification on the meaning of duplex mode, but in response to this clarification, could you add “FDD and TDD” in the following table? - Table 5.2.2-1: NR interband CA within FR1 - Line of “DL” - NOTE: The corresponding band combinations have been introduced in Rel-15 specification such as n3-n77. - Table 8.1.2.3-1: EN-DC interband configurations including FR1 and FR2 - Line of “DL FR1” - Line of “UL FR1” NOTE: The corresponding band combinations have been introduced in Rel-15 specification such as DL/UL 1_n77-n257. And we wonder it would be better to specify the definition of duplex mode in general section instead of putting NOTEs in every table. Huawei: The indication for duplex mode is unnecessary for the band combinations with mixing duplex mode, since RAN4 never discuss the requirements or capabilities based mixing duplex mode for the band combination. We may still lost the mixing duplex mode, such as “SDL and FDD” for inter-band CA. If so, there is no need to add these notes. CR quality control: This agenda and WI code “LTE_NR_DC_CA_enh-Core” is for Rel-16. However, this CR is for Rel-15 and it’s based on the current version “15.7.0”. Apple: we support the proposal to clarify the meaning of duplex mode in the configuration tables; whether this is done in notes or a general description is fine for us |
| R4-2102208 TS 38.307 CR 0056 Rel-16 Cat-A |  |
| R4-2102209 TS 38.307 CR 0057 Rel-17 Cat-A |  |
| R4-2102408 TS 38.101-1 CR 0689 Rel-16 Cat-F | Ericsson: agreed. |
|  | Huawei: The Work item code NR_newRAT-Core is for Rel-15 instead of Rel-16 |
|  | Apple: we support this correction |
| R4-2102409 TS 38.101-1 CR 0690 Rel-17 Cat-A |  |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2100844 TS 38.101-3 CR 449 Rel-16 Cat-F | Return toSkyworks: as discussed in other threads, we do not see that additional line should be added for increased UL BW. we recognize the need to revise the MSD but this could be done by using the full allocation of the new largest BW. |
| R4-2102207 TS 38.307 CR 0055 Rel-15 Cat-F | Return to |
| R4-2102408 TS 38.101-1 CR 0689 Rel-16 Cat-F | Agreeable (WI code to be changed) |

## Discussion on 2nd round

### CRs/TPs comments collection

Moderator comment: note that there is a CR related R4-2102207 in R4-2101989 under agenda 4.2.1 [102]

| CR/TP number |  | Comments collection |  |
| --- | --- | --- | --- |
| R4-2100844 TS 38.101-3 CR 449 Rel-16 Cat-F |  | Company A |  |
|  |  | Huawei: We propose to specify the MSD due to CIM. This case can be merged in my Rel-16 CR R4-2102095. The link for draft CR can be found as below. https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/TSGR4_98_e/Inbox/Drafts/%5B98e%5D%5B104%5D%20NR_NewRAT_UE_RF_Part_3/Draft%20CR/draft%20R4-2102095%20CR%20for%2038.101-3%20to%20introduce%20a%20new%20MSD%20due%20to%20the%20counter%20intermodulation%20interference%20(Rel-16).docx |  |
|  | Qualcomm: |
|  | We need to comeback and revisit this with our own analysis. We do not need see the need to have a reduction in the UL configuration, but we do recognize that MSD could be an issue depending on implementation and available filter technology given the number of antennas used in the band combination. |
| R4-2102207 TS 38.307 CR 0055 Rel-15 Cat-F |  | Huawei: Based on the TS 38.306, for band combinations’ capabilities, the TDD-FDD Diff is N/A. Both RAN2 and RAN4 never use the “mixed duplex” to distinguish UE requirements or capabilities. Seems that we can also follow the same principle to drop them. This information is redundant. As Nokia said, it may help to reduce the maintenance workload if we removing them. Besides, we may still lost the mixing duplex mode, such as “SDL and FDD” for inter-band CA. Totally, it’s recommend to agree R4-2101989. ZTE: Disagree with Huawei’s comments. According to the 1st round discussion, 5 companies (QC, CHTTL, NTT DoCoMo, Apple, ZTE) agreed to add the note, rather than deleting it. We think it is no problem for the maintenance workload, the maintenance workloads are largely smaller than the other specs, actually we have already done some maintenance work in the past meetings for Rel-16 for the missing duplex-mode, Also the combinations in each 38.307 release are aligned with each 101 spec release, for example the combinations in Rel-15 101-1 spec are reflected in Rel-15 38.307, the combinations in Rel-16 101-1 spec are reflected in Rel-16 38.307. If lost the duplexer information for these combination, it may lead some confusion. For example: In Rel-15 101-1, only duplex-mode of TDD intra-band contiguous CA combinations are supported, however if we delete the duplex-mode, then it will change the meaning that FDD intra-band contiguous CA combinations are also supported in Rel-15, but it is not true. Another example is for inter-band NR CA, only TDD for both band and TDD-FDD are supported for UL in Rel-15 101-1 spec, however if we delete the duplex-mode, then it will change the meaning that FDD-FDD are supported for UL in Rel-15, also it is not true. In our understanding, deleting the duplex-mode will lost the relationship between 38.101-1/2/3 spec and TS38.307, which violate the original intention of TS38.307. For the mixing “SDL and FDD” for inter-band CA, it can be added to the spec easy.. For capabilities, we didn’t seen any problem, for example the “mixed duplex” of “TDD and FDD” means CA configuration including both FDD and TDD bands with the explanation in our CR, it can be easy understood as FDD+TDD band combination, which is aligned with the RAN2 capabilities. Also in TS36.307, such column is also existed and kept for more than 10 years, we never heard of there exists capabilities problem.. |  |
|  |  | Company B |  |
| R4-2103142 (revision of R4-2102408) TS 38.101-1 CR 0689r1 Rel-16 Cat-F |  | Company A |  |
|  |  | Company B |  |

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2100844 TS 38.101-3 CR 449 Rel-16 Cat-F | To be noted |
| R4-2102207 TS 38.307 CR 0055 Rel-15 Cat-F | To be noted |
| R4-2103142 TS 38.101-1 CR 0689r1 Rel-16 Cat-F | Agreeable |

# Topic #2: Reply LS to RAN2 on P-Max for FR2

RAN4 has received a LS from RAN4 on p-NR-FR2, a UE-specific P-Max on a cell group in FR2. RAN4 has not agreed upon inclusion of a UE-specific P-Max in the configured maximum output power in the Rel-16 specification.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102044 | Ericsson | Title: LS reply to RAN2 on power control for NR-DC |
| R4-2102711 | vivo | Title: Discussion and reply LS on p-NR-FR2 Observation 1: RAN1 defined p-NR-FR1 and p-NR-FR2 for a certain frequency range as maximum power for uplink power control in MR-DC. P-NR-FR1 is defined for FR1 in EN-DC/NE-DC/NR-DC, while p-NR-FR2 is only defined for FR2 in NR-DC. Observation 2: RAN4 use p-NR-FR1 in EN-DC/NE-DC/NR-DC in the calculation of configured transmitted power, while did not use p-NR-FR2. Observation 3: For FR2, the configured transmitted power is indirectly restricted using Pumax and PTmax, and no place to use p-NR-FR2. Observation 4: For FR2, there is currently no definition and requirements for NR-DC. Observation 5: p-NR-FR2 is similar to p-UE-FR2 in that EIRP and TRP control feasibility problem persists. Proposal: Not using p-NR-FR2 at least in Rel-16, based on similar reason to p-UE-FR2, and also no NR-DC requirements in Rel-16 RAN4. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Reply LS to RAN2

- Proposals

  - Option 1: Reply in accordance with R4-2102044

  - Option 2: Reply in accordance with R4-2102711

  - Option 3: other (specify what)

- Recommended WF

  - Choose one of these two available drafts (possibly modified) for a reply this meeting, RAN4 has not agreed to include a UE-specific P-Max for Rel-16.

### Sub-topic 2-2

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 2-2: TBA

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |
| OPPO | Issue 2-1: Reply LS to RAN2 Option 2. |
| Ericsson | Issue 2-1: Option 1, the same intent as Option 2 but answers both questions, |
| ZTE | Same answers for both option 1&2. slight prefer to Option 2. |
| vivo | Issue 2-1: Option 2. Option2 is different from option 1 in that the 2nd question was not really answered, because the pre-condition of question 2 “If no concern, …” was not satisfied, since RAN4’s question on the question 1 is negative which is also aligned with another candidate LS. In another word, RAN4 should just answer the first question and question 2 is not appropriate to be answered, RAN2 can make adjustment by their own based on RAN4’s feedback on question 1. |
| MTK | Support Option 2 But also suggest to mention to RAN2 that question is no longer needed to be discussed in order to avoid any different/strange interpretation in RAN2. |
| NTT DOCOMO, INC. | Prefer to use option 1 as baseline. Option 1 seems to describe the current situation more clearly. But, since there is a concern on the sentence of “RAN4 confirms the description of p-NR-FR2 provided by RAN2, but”, we are OK to remove this part from R4-2102044. And, in our understanding, for the answer to Q1, “no” should be added. “Hence there is “no” action upon reception of p-NR-FR2 by a UE compliant with Rel-16.” |
| Huawei | Issue 2-1: Reply LS to RAN2 Option 3. The problem is: if we agree that RAN4 do not define Pmax for FR2 and also for NR-DC with FR2 when calculate Pcmax, why network need to configure P-NR-FR2 to UE that is actually useless? We propose to request RAN2 remove the definition P-NR-FR2 at least in Rel-16. Issue 2-2: Open issues For P-NR-FR2, we have 2 option2: - There is no definition in FR2, network should not configure this parameter to the UE - Network still could configure this parameter to UE, but UE will ignore the configuration when power control calculation. |
| Apple | We prefer option 2 and the associated simplicity of the response, since RAN4 has not defined any requirements associated with this signaling in Rel-16 |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | Tentative agreements: Issue 2-1: Option 2 Candidate options: Recommendations for 2nd round: Revise the LS in R4-2102711 |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round

### Sub-topic 2-1 Reply LS to RAN2

Discussion on the Reply LS to RAN2 on p-NR-FR2 (R4-2102711 revised), comments provided on the e-mail reflector or below.

| Tdoc number | Comments collection |
| --- | --- |
| R4-2103143 (Draft Reply LS) | Company A |
|  | Company B |

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103373 (LS to RAN2, revision of R4-2103143) | No disagreement on the technical issue. Agreeable. |

# Topic #3: Single-uplink operation

RAN4 has received an LS from RAN on single-uplink only (SUO) operation in RP-202622 with the following action.

ACTION: 	RAN respectfully requests RAN2/RAN4 to check if any specification clarification is needed to ensure there is no inter-operability issue between the UE side and network side, considering the report of singleUL-Transmission as described in RP-202622.

Moreover, a potential signaling issue for band combinations with several parts allowing single-UL transmission has been identified, and discussions on a SUO capability for a roaming situation will be continued (from RAN4#97-e)

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101144 | MediaTek Inc. | Title: Discussion on the reply to LS on single UL operation Observation 1: Current feature set reporting allows UE to indicate which pair(s) of CCs that UE can support UL transmission in a single band combination. Observation 2: Single singleUL-Transmission but is not sufficient for UE to indicate dual UL in one UL CC pair and single UL in another CC pair in one band combination. Proposal 1: In Rel-15, to indicate a different singleUL-Transmission capability for a particular UL pair in a high level band combination, UE may additionally report the corresponding fallback band combination with a different singleUL-Transmission capability. Proposal 2: Reply LS to RAN2 with RAN4’s suggestions, but leave it to RAN2 on whether to resolve this issue in R15 or in later releases. |
| R4-2101718 | Ericsson | Title: Correction to applicability of simultaneous RX/TX and single-UL transmission CR to 38.101-3 (Rel-15) Reason for change (SUO part only): 2. The applicability of single-switched uplink is unclear (LS to RAN4 from RAN in RP-202932). Summary of change (SUO part only) 2. Clause 5.3B.1.3 and 5.5B.1 (general): The statement that “only single-switched UL is supported” (not the scope of 38.101-3) is replaced by statements that minimum requirements are only specified for single-switched UL and it is clarified that the UE may include the field singleUL-Transmission. Clause 5.5B, 6.2B.1.2 and 7.3B.2.2: the applicability specified in BCS band combination tables. Annex H: the applicability for the DC_3-n3 configuration removed (specified in the band combination tables). |
| R4-2101820 | Huawei, HiSilicon | Title: Further discussion on RF requirements about Multi-RAT Dual-Connectivity Proposal 1: There are some specific situation such as roaming in which UE could report “singleUL-Transmission”, which can bring benefits in business application. Proposal 2: To introduce a new UE capability for specific ENDC band combinations in roaming situation. This capability can be reported to network with existing capability “singleUL-Transmission” together or separately. In this situation, the UE can report roaming indication to clarify the roaming status to the base station together or separately. |
| R4-2102387 | Huawei, HiSilicon | Title: On SUO for intra-band EN-DC Proposal: It is proposed to send LS to RAN2 asking some clarification in RAN2 spec that that for intra-band EN-DC combinations which support only single switched UL, the capability singleUL-Transmission must be reported. |

## Open issues summary

### Sub-topic 3-1 Clarification of SUO specification

Sub-topic description: the action in RP-202622

Open issues and candidate options before e-meeting:

Issue 3-1-1: Clarification is needed to ensure there is no inter-operability issue between the UE side and network side (RAN LS in RP-202622)

- Proposals

  - Option 1: Clarification as proposed in R4-2101718 (specify the cases in which the UE includes the single-UL Transmission), no LS needed

  - Option 2: Reply LS to RAN/RAN2 that for intra-band EN-DC combinations which support only single switched UL, the capability single-UL Transmission must be reported as proposed in R4-2102387, no change to RAN4 specifications

  - Option 3: other (specify which)

Issue 3-1-2: Single UL allowed for several band pairs part of an indicated band combination

- Proposals

  - Option 1: LS to RAN2 with recommendations according to proposals in R4-2101144

  - Option 2: LS to RAN2 describing the identified issue only

  - Option 3: no need for any changes (no LS)

  - Option 4: other (specify which)

- Recommended WF

  - TBA

### Sub-topic 3-2 Single UL in a roaming scenario

Open issues and candidate options before e-meeting:

Issue 3-2-1: UE capability for specific ENDC band combinations in roaming situation

- Proposals

  - Option 1: introduce a SUO capability for specific ENDC band combinations in roaming situation as proposed in R4-2101820 (Correction by moderator)

  - Option 2: do not introduce a SUO capability for specific ENDC band combinations in a roaming situation

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 2-1: Sub topic 2-2: …. Others: |
| OPPO | Issue 3-1-1: Clarification is needed to ensure there is no inter-operability issue between the UE side and network side (RAN LS in RP-202622) Option 2, to make the same understanding among groups. |
| Ericsson | Issue 3-1-1: Clarification is needed to ensure there is no inter-operability issue between the UE side and network side (RAN LS in RP-202622) Option 1. SUO is an incapability that should not be mandated in the absence of RAN4 requirements for a band combination. The 38.306 specifies that “The UE may only include this field [singleUL-Transmission] for certain band combinations defined in TS 38.101-3 [4].” The 38.101-3 should specify when the UE may include (or when it includes) the singleUL-Transmission. No RAN2 changes needed. Issue 3-1-2: Single UL allowed for several band pairs part of an indicated band combination Option 2, describing the issue without proposing signalling changes. On the R4-2101144, we do not agree with the proposal Proposal 1: In Rel-15, to indicate a different singleUL-Transmission capability for a particular UL pair in a high level band combination, UE may additionally report the corresponding fallback band combination with a different singleUL-Transmission capability. The capability of a top-level band combination should not be dependent on that of any included fallback combination, this would break the capability signalling. Issue 3-2-1: UE capability for specific ENDC band combinations in roaming situation Option 2. |
| Qualcomm | Sub topic 3-1-1: Prefer option 1, SUO part of R4-2101718. Sub topic 3-1-2: Tend to agree with the observations in R4-2101144. The proposals look ok from RAN4 perspective, but would be good to get (informal) RAN2 feedback since fallback configurations are intended to be implicit. Sub topic 3-2-1: Option 2. The UE should support requirements in both the home network and roaming network. |
| CHTTL | Issue 3-1-1 Support Option 2. Option 1 introduces a huge change on the Rel.15 spec, and the change removes the extension support for dual uplink in the later release, which are not acceptable to us. And it seems the issue that raised in R4-2102387 is unsolved. |
| MTK | Issue 3-1-1: We think an LS is anyway needed to avoid RAN2 keep waiting for RAN4 input. Option 2 is preferred which seems to be an easier solution. Issue 3-1-2: Option 2 is preferred. The issue is that the singleUL-transmission capability is reported per band combination, but in a band combination, there could be multiple possible UL CC pairs, e..g, DC_2A_n66A, DC_7A_n66A, and DC_66A_n66A in DC_2A-7A-66A_n66A. Current signalling structure implies all UL CC pairs have to share the same capability. This may not be sufficient for the case that UE can support dual UL in one CC pair but single UL in another. Since this is a Rel-15 issue, we preferred to minimize the spec change as much as possible. The suggestion in R4-2101144 is to ask network to further check all corresponding fallback band combo (if reported) to identify the capability of each individual CC pair. We understand Ericsson’s concern that the network currently does not need to check all fallback combinations in order to determine the capability of the top-level band combination. Therefore, we prefer to inform RAN2 this issue. And RAN2 should be the correct WG to judge if the suggestion is feasible or not as well as whether and how to resolve it in Rel-15. For Rel-16, the easiest solution is to introduce a new capability in feature set. But we also prefer to leave it to RAN2 to decide. |
| Nokia | Issue 3-1-1: Option 2 UE is mandated to indicate the singleUL-Transmission for band combinations where single UL is the only specified operation mode. Otherwise, not required. Issue 3-1-2: Option 3 No need for any changes. In RAN2 we would like to keep Rel-15 approach and not to add new pairs signalling for marking such combinations. UE can signal fallback based on current principles. No need to change anything here. Issue 3-2-1 Option 2 UE can identify it is roaming and report different capabilities. The home network does not need to know what the UE is reporting to a roaming network when it goes out, and the roaming network doesn’t care what the UE reported when it was in the home network. |
| Skyworks | Issue 3-2-1 Option 2, from the contribution provided, the roaming situation is too vague, roaming between continents is not the same thing than roaming within Europe so UE should meet requirements at home or in roaming situation. |
| NTT DOCOMO, INC. | Issue 3-2-1: Option 2: Basically, we want UEs to support dual UL for all band combination except for some problematic band combinations. Question for our understanding is: In our understanding, if UE supports a certain EN-DC band combination, for example, a band combination with low band + high band, then UE basically can support other low band + high band combinations, assuming that UE have PAs for low bands and high bands, respectively, and that RF component is designed to achieve sufficient isolation between low and high bands. So, we are not sure if it is beneficial that single UL operation is allowed in roaming case. |
| Huawei | Issue 3-1-1: Clarification is needed to ensure there is no inter-operability issue between the UE side and network side (RAN LS in RP-202622) Option 2. The changes in options still cause ambiguity for a band combination which may have different capability or applicable requirements in different releases. We think some clarification is needed in RAN2 spec. Issue 3-1-2: Single UL allowed for several band pairs part of an indicated band combination Option 1. Agree with the observations in R4-2101144. Whether there is a spec impact can be further discussed. Issue 3-2-1: UE capability for specific ENDC band combinations in roaming situation Option 1. As for UE in roaming network, It will have different performance due to different implement between different edition. So we think it is reasonable and profitable to support UE capability for specific ENDC band combinations in roaming situation. |

### CRs/TPs comments collection

For R4-2101718, comments only on the SUO part.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2101718 TS 38.101-3 CR 0461 Rel-15 Cat-F | CHTTL: this CR introduce a huge change on the Rel.15 spec. and the change remove the extension support for dual uplink in the later release, which will cause inconsistency to the later release. |
|  | Company B |
| R4-2101719 TS 38.101-3 CR 0462 Rel-16 Cat-A | CHTTL same comment above. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Tentative agreements: Issue 3-1-1: no consensus, consider an LS to RAN2 on SUO clarification as proposed in R4-2102387 subject to further discussions in the 2nd round Issue 3-1-2: Option 2 (LS to RAN2 describing the identified issue only) Candidate options: Recommendations for 2nd round: Discussions on the two LS above (see below) |
| Sub-topic#3-2 | Tentative agreements: Option 2 (five companies against Option 1, one supporting) Candidate options: Recommendations for 2nd round: No further discussion |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | LS to RAN2 on single-uplink operation in more than one band pair of a band combination | MTK |
| #2 | LS to RAN2 on single-uplink operation | Huawei |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101718 TS 38.101-3 CR 0461 | To be revised (this CR is anyway “to be revised” in thread [104]) |

## Discussion on 2nd round

### Sub-topic 3-1 Clarification of SUO specification

Issue 3-1-1: Clarification is needed to ensure there is no inter-operability issue between the UE side and network side (RAN LS in RP-202622)

- Proposals (options remaining for 2nd round)

  - Option 1: Clarification as proposed in R4-2101718 (specify the cases in which the UE includes the single-UL Transmission), no LS needed

  - Option 2: Reply LS to RAN/RAN2 that for intra-band EN-DC combinations which support only single switched UL, the capability single-UL Transmission must be reported as proposed in R4-2103145, no change to RAN4 specifications

Continued discussion on the CR in R4-2101718 (Option 1) and/or an LS to RAN2 on single uplink operation (R4-2103145).

| CR/LS number | Comments collection |
| --- | --- |
| R4-2101718 TS 38.101-3 CR 0461 (SUO part only) | Company A |
|  | Company B |
| R4-2103145 Draft LS to RAN2 | Qualcomm: We do not agree with the latest version of the draft LS (_EAB_NOK_HW). We are not convinced that an LS is needed, but if so, we prefer to further discuss the revision marked as _EAB. Huawei: As commented by QC in the reflector, the main concern is sending LS and ask RAN2 to mandate SUO reporting. But in the LS, it just provide the requirements status in RAN4 and how to solve the issue identified by RAN4 is left to RAN2. |
|  | Company B |

Issue 3-1-2: Single UL allowed for several band pairs part of an indicated band combination

- Proposals (options remaining for 2nd round)

  - Option 2: LS to RAN2 describing the identified issue only

Comments on the Draft LS to RAN2 in R4-2103144 provided on the e-mail reflector or below.

| Tdoc number | Comments collection |
| --- | --- |
| R4-2103144 (Draft LS to RAN2) | Company A |
|  | Company B |

## Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2101718 TS 38.101-3 CR 0461 (SUO part only) | To be noted |
| R4-2103144 (LS to RAN2) | No comments neither above nor on the reflector. Agreeable. |
| R4-2103365 (LS to RAN2 revision of R4-2103145) | No consensus. To be returned to. |