3GPP TSG-RAN WG4 Meeting #98-e	R4-2103347

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0637 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1: corrections on exceptions for out-of-band blocking for inter-band CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The combinations of CA_n5-n77, CA_n5-n78 and CA_n28-n78 also meet the exception condition for out-of-band blocking, which should be included in the corresponding table 7.6A.3.3-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the combinations of CA_n5-n77, CA_n5-n78 and CA_n28-n78 into the exception table 7.6A.3.3-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may cause stringent requirement when testing out-of-band requirements for those band combinations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

--------------Start of change-------------

7.6A.3.3	Out-of-band blocking for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band, the out-of-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For NR CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.2-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in clause 7.6.3 for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with component carriers in operating bands < 2.7GHz including n48, and for FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz, the appropriate adjacent channel selectivity and in-band blocking requirements in the respective clauses 7.5 and 7.6.2 shall be applied for carrier j. For inter-band carrier aggregation with component carriers in operating bands > 2.7GHz excluding n48, and for FDL_Low(j) – 3* BWchannel ≤ f ≤ FDL_High(j) + 3* BWchannel, the appropriate adjacent channel selectivity and in-band blocking requirements in the respective clauses 7.5 and 7.6.2 shall be applied for carrier j. FDL_Low(j) and FDL_High(j) denote the respective lower and upper frequency limits of the operating band containing carrier j, j = 1,…,X, with carriers numbered in increasing order of carrier frequency and X the number of component carriers in the band combination. BWchannel denotes the channel bandwidth of the wanted signal component carrier j. If CW interferer falls in a gap between FDL_High(j) and FDL_Low(j+1) where the corresponding OOB ranges 1 and 2 overlap, then the lower level interferer limit of the overlapping OOB ranges applies.

For inter-band carrier aggregation with uplink assigned to two NR bands, the out-of-band blocking requirements specified in clause 7.6.3 shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,f,c  for each serving cell c.

For the UE which supports inter-band CA configuration in Table 7.3A.3.2.1-1, Pinterferer power defined in Table 7.6.3-2 and 7.6.3-4 is increased by the amount given by ΔRIB,c in Table 7.3A.3.2.1-1.

For inter-band CA combination listed in Table 7.6A.3.3-1, exceptions to the requirement specified in Table 7.6A.3.3-2 are allowed when the second order intermodulation product of the lower frequency band UL carrier and the CW interfering signal fully or partially overlaps with the higher frequency band DL carrier.

Table 7.6A.3.3-1: CA band combination with exceptions allowed

CA band combination

CA_n5-n77

CA_n5-n78

CA_n5-n79

CA_n8-n78

CA_n8-n79

CA_n20-n78

CA_n28-n77

CA_n28-n78

CA_n78-n92

Table 7.6A.3.3-2: Requirement for out-of-band blocking exceptions

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -441 |
| NOTE 1: The requirement applies when fInterferer± fULLB- fDLHB≤(BWULLB+ BWDLHB)/2, where fULLB and fDLHB are the carrier frequencies for lower frequency band UL and higher frequency band DL, respectively. BWULLB and BWDLHB are the channel bandwidths configured for lower frequency band UL carrier and higher frequency band DL carrier in MHz, respectively. |  |  |

For all interferer frequency ranges specified in clause 7.6.3 a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of   MHz with NRB the number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.

The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

7.6A.4	Narrow band blocking for CA

-------------End of change-------------