3GPP TSG-RAN WG4 Meeting #98-e	R4-2103758

Electronic Meeting, January 25th – February 5th, 2021

Agenda item:			7.3.6.1, 7.3.6.3

Source:	Moderator (Intel Corporation)

Title:	Email discussion summary for [98e][319] V2X_Demod_Part2

Document for:	Information

# Introduction

The scope of this email thread is Rel-16 V2X multi-link performance requirements.

Email discussion targets for the 1st round and 2nd round

- 1st round:

  - Discussion of open issues for all considered tests.

  - Collection of comments for Draft CRs (focus on comments which are not related to open issues, i.e. structure, list of test parameters etc.)

- 2nd round:

  - WF preparation

  - Collection of comments for Draft CRs.

# Topic #1: Power imbalance test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100410 | CATT, GOHIGH | Simulation results of multiple link demodulation test cases based on the simulation assumptions |
| R4-2100629 | Qualcomm, Inc. | Proposal 1: Reuse LTE ICS value (-27dBc) for NR. |
| R4-2100662 | LG Electronics Inc. | Proposal 1: 10RB separation between two sidelink and -27dBc ICS level for power imbalance test can be considered as in LTE V2X test case. |
| R4-2101237 | Intel Corporation | Proposal 1: Use the following simulation assumptions for Rel-16 V2X Power imbalance requirements: - ICS level = -27 dBc - Distance between the two links is 20 PRBs (i.e. PRBs for second UE are 30-39) - 2nd stage SCI configuration: β = 2.5 |
| R4-2101357 | Huawei, HiSilicon | Draft CR: Introduction of Power imbalance requirements for NR V2X |
| R4-2101358 | Huawei, HiSilicon | Proposal 1: In order to minimum the impact of RF impairment (minimize the TX general IBE, avoid the IQ image and carrier leakage), use 10 RBs sub-channel size and distance between two links should be 20RBs and following allocation can be used: - Option 1: sub-channel 1(RB index: 0~9) for UE1 and sub-channel 4(RB index: 30~39) for UE 2. - Option 2: sub-channel 2(RB index:10~19) for UE1 and sub-channel 5(RB index:40~49) for UE 2. Observation 1: For ICS=-27dBc, required SNR of weak link derived from explicit modelling is 0.8dB lower than required SNR derived from single-link Proposal 2: Use ICS=-27dBc Proposal 3: Use Osci2=35, Betta-offset=3.5 for 2nd SCI configuration. |

## Open issues summary

Issue 1-1: ICS level

- Proposals

  - Option 1 (QC, LGE, Intel, Huawei, MTK, CATT): Use -27dBc ICS level

- Recommended WF

  - Check whether Option 1 is acceptable for all interested companies

Issue 1-2: Distance between the two links and UEs allocation

- Proposals

  - Option 1 (LGE, MTK): 10 PRBs, UE1 PRBs 0-9, UE2 PRBs 20-29

  - Option 2 (Intel, Huawei, QC, LG, MTK, CATT): 20 PRBs, UE1 PRBs 0-9, UE2 PRBs 30-39

  - Option 3 (Huawei, QC, LG, MTK, CATT): 20 PRBs, UE1 PRBs 10-19, UE2 PRBs 40-49

- Recommended WF

  - Collect comments on options above

Issue 1-3: SCI format 2-A configuration

- Proposals

  - Option 1 (Intel, Huawei, QC, CATT)

    - Payload = 35 Bits

    - βoffset = 3.5

- Recommended WF

  - Check whether Option 1 is acceptable for all interested companies

## Companies views’ collection for 1st round

### Open issues

| Company | Comments collection |
| --- | --- |
| Huawei, HiSilicon | Issue 1-1: ICS level Option 1 Based on our analysis and evaluations, ICS=-27dB is feasible if 20 RBs distance is used. Issue 1-2. Distance between the two links and UEs allocation Based on our analysis, 20RBs distance should be considered. Option 2 and Option 3 is essentially same. Option 2 can be selected based on majority’s view. Issue 1-3: SCI format 2-A configuration Option 1. Based on our simulation results, Option 1 is feasible. |
| QC | Issue 1-1 Option 1 is good Issue 1-2 Option 2 and 3 are both fine for us Issue 1-3 Option 1 is good |
| LG | Issue 1-1: ICS level Support option 1. Issue 1-2: Distance between the two links and UEs allocation We are fine with 20 PRB distance between two links. Both option 2 and option 3 are fine. Issue 1-3: SCI format 2-A configuration Support option 1 |
| MTK | Issue 1-1: ICS level Support option 1. Issue 1-2: Distance between the two links and UEs allocation We slight prefer option 1, but option 2 and option 3 with larger distance are also fine with us. |
| Intel | Issue 1-2: Distance between the two links and UEs allocation In figures below we provide IBE requirements for two options. Option 2.; Option 3. We can observe that for option 3, one PRB will overlap with IQ image, and for Option 2 we don’t have such overlap. Therefore, we prefer Option 2. |
| CATT | Issue 1-1: ICS level Support option 1. Issue 1-2: Distance between the two links and UEs allocation Both option 2 and option 3 are OK with us. Issue 1-3: SCI format 2-A configuration Support option 1 |

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2101357 | QC: Noc unit: dBm/30kHz, value: -95 |
|  | LG: need resource pool configuration |
|  | Intel: Based on our understanding, information about Noc can be removed from test configuration because this information is defined in Section 4.4.3 of 38.101-4 and applicable to all requirements in the specification. |

## Summary for 1st round

### Open issues

Status summary

Agreements Issue 1-1: ICS level - Use -27dBc ICS level Issue 1-3: SCI format 2-A configuration - Payload = 35 Bits - βoffset = 3.5 GTW agreements Issue 1-2: Distance between the two links and UEs allocation - 20 PRBs, UE1 PRBs 0-9, UE2 PRBs 30-39 No issues for 2nd round discussion.

WF and Simulation assumptions assignment (documents will capture agreements for all topics under this e-mail thread)

|  | WF doc Title | Assigned Company, WF lead |
| --- | --- | --- |
| #1 | WF on multiple link tests for NR V2X demodulation performance | Intel |
| #2 | Simulation assumptions for NR V2X multiple link test case | Huawei |

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101357 | To be revised |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #2: HARQ buffer soft combining test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100408 | CATT | Proposal 1: Configure 15 PRB sub-channel size for HARQ buffer soft combining test. Proposal 2: Configure MCS 28 for HARQ buffer soft combining test. |
| R4-2100410 | CATT, GOHIGH | Simulation results of multiple link demodulation test cases based on the simulation assumptions |
| R4-2100629 | Qualcomm, Inc. | Observation 1: Peak MCS with all first Tx failed is not a likely scenario happened in practice. Observation 2: Since no capability defined for HARQ buffer size, the NR HARQ soft combing test is different from LTE soft buffer test. Proposal 2: Set MCS 13 for HARQ processes test. |
| R4-2100630 | Qualcomm, Inc. | Draft CR: Introduction of HARQ buffer soft combining requirements for NR V2X |
| R4-2101238 | Intel Corporation | Proposal 1: Use the following simulation assumptions for Rel-16 V2X HARQ soft buffer combing test: - Sub-channel size: 10 PRBs - MCS 28 - 2nd stage SCI configuration: β = 2.5 - SNR 14 dB |
| R4-2101359 | Huawei, HiSilicon | Proposal 1: Add the following test to verify UE support receive 48 HARQ processes per slot: ” For verification of n48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The next 17 UEs transmit signal one by one circularly for every slot and in the second subchannel” Proposal 2: Consider 600Hz frequency offset and 12Ts time offset at receiving offset at Rx side for simulation alignment. Observation 1: Imperfect synchronization has very subtle impact on performance for cases with n32, n48, n64. Proposal 3: Define only one requirement for different UEs supporting different maximum receiving HARQ processes. Proposal 4: Use 10 sub-channel size. Proposal 5: Use MCS 28 for soft buffer test. Observation 2: For MCS=28, the BLER for 2nd SCI is lower than 10-4 when SNR larger than 8dB which is required SNR for soft buffer test. For MCS=13, the BLER for 2nd SCI is lower than 10-5 when SNR larger than 1dB which is required SNR for soft buffer test. Proposal 6: It is feasible to use Payload = 35 Bits, βoffset = 2.5 for soft buffer test. |

## Open issues summary

Issue 2-1: Test design for UE supporting n48 HARQ processes

- Proposals

  - Option 1 (Huawei, QC, MTK, Intel, CATT): Add the following test to verify UE support receive 48 HARQ processes per slot: “For verification of n48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The next 17 UEs transmit signal one by one circularly for every slot and in the second subchannel”

- Recommended WF

  - Collect comments on option above.

Issue 2-2: Sub-channel size

- Proposals

  - Option 1 (CATT): 15 PRBs

  - Option 2 (Intel, Huawei, QC, MTK, CATT): 10 PRBs

- Recommended WF

  - Collect comments on options above

Issue 2-3: MCS

- Proposals

  - Option 1 (CATT, Intel slight preference, Huawei, MTK slight preference, CATT): MCS 28

  - Option 2 (QC, MTK, Intel): MCS 13

- Recommended WF

  - Collect comments on options above.

Issue 2-4: SCI format 2-A configuration

- Proposals

  - Option 1 (Intel, Huawei, QC, CATT)

    - Payload = 35 Bits

    - βoffset = 2.5

- Recommended WF

  - Check whether Option 1 is acceptable for all interested companies

Issue 2-5: Rx assumptions for simulation alignment

- Proposals

  - Option 1 (Huawei): Consider 600Hz frequency offset and 12Ts time offset at receiving offset at Rx side

  - Option 2 (QC, MTK, CATT): Use TO and FO equal to 0.

  - Option 3 (Intel): Leave it up to individual company assumptions.

- Recommended WF

  - Collect comments on option above

Issue 2-6: SNR for requirements definition

- Proposals

  - Option 1 (Intel): 14 dB for MCS 28

- Recommended WF

  - Postpone this discussion and come back after Issue 2-3 is resolved and after collection of alignment results

Issue 2-7: How to define the requirements for different supported max number of HARQ process

- Proposals

  - Option 1 (Huawei, QC, Intel, CATT): One set of performance requirements based on the results of n64

- Recommended WF

  - Depend on the discussion on Issue 2-5 and the performance difference among different max number of HARQ process

## Companies views’ collection for 1st round

### Open issues

| Company | Comments collection |
| --- | --- |
| Huawei, HiSilicon | Issue 2-1: Test design According to TS 38.331, capability that UE can support maximum number of receiving HARQ process: But according to the WF agreed in last meeting, n48 is missing, we added this case to keep consistent with core specification. Issue 2-2: Sub-channel size Slightly prefer option 2. From our simulation results, imperfect synchronization has little effect on performance for different HARQ process if 10 RB sub-channel size is used, so only one set of requirement can be defined for supported different number of HARQ process. If we use 15 RBs sub-channel size, ICI impact should be evaluated whether it is feasible to define one set of requirements for different supported HARQ processes. Issue 2-3: MCS Option 1 Based on our simulation results, the performance difference between first transmission and second transmission is 3~4 dB for MCS 13 and 9dB for MCS 28. The test purpose is to make all first transmissions fail and 95% 2nd transmission pass the test. Obviously, MCS 28 is more feasible since performance gap is more obvious between 1st transmission and 2nd transmission. Issue 2-4: SCI format 2-A configuration Option 1. Based on our simulation results, option 1 is feasible. Issue 2-5: Rx assumptions for simulation alignment Option 1. We can’t control the CFO and CTO at receiving side. Therefore, imperfect synchronization should be considered for simulation alignment. Issue 2-6: SNR for requirements definition The averaged impairments results from all interesting companies should be taken into account. Issue 2-7: How to define the requirements for different supported max number of HARQ process Option 1 Based on our simulation results, there are only 0.2dB performance gap between n16 (without ICI) and n64 (Maximum ICI) (If Option1 in issue 2-5 is agreed), hence only one set of requirements for different supported max number of HARQ processes could be defined. Meanwhile, considering definition of the minimum requirements, we propose to define the performance requirements based on the results of n64 |
| QC | Issue 2-1: Option 1 is good Issue 2-2: Support option 2 Issue 2-3: Support option 2 As we explained in our contribution, option 1 is not a practical scenario where HARQ buffer is used. Since almost all the TBs are failed in first Tx and pass only at second Tx, UE can easily achieve better throughput when slightly lower MCS is transmitted with successful decoding for first Tx. Hence peak MCS is not practical scenario for this test setup, in AWGN gNB is expected to slightly lower MCS, which can pass first Tx, from peak MCS, to enhance network throughput. Simulation results from other companies show a 3-4dB gap between first and second Tx for MCS 13, which is smaller than MCS 28 case but is sufficient, since RAN4 typically has span range of 2.5dB, which is smaller than the gap. Issue 2-4: Option 1 is good. Issue 2-5: We don’t agree with option 1, TO and FO should both be 0. This is a capability test, test purpose is to verify if UE can support that many HARQ processes. Effect of TO/FO should be tested in single link test, not capability test Issue 2-6: Agree to postpone the discussion Issue 2-7: Agree with one set of performance requirement. Number of HARQ processes supported shouldn’t affect performance given that the channel is AWGN. |
| MTK | Issue 2-1: Support this proposal for adding n48 HARQ process case. Issue 2-2: Support option 2. As compromised in single link test case configuration, unified sub-channel structure is more preferable. Issue 2-3: Slightly prefer option 1, but open with option 2. Issue 2-5: Not support Option 1. We have the similar view with QC, it doesn’t need to introduce the CFO/CTO for HARQ buffer soft combining test since we have agreed that the main test purpose is to verify HARQ buffer combining in last RAN4 meeting. Issue 2-6: Support Recommended WF and postpone this discussion. |
| Intel | Issue 2-1: Test design for UE supporting n48 HARQ processes Option 1 is fine for us. Issue 2-2: Sub-channel size Support Option 2 because it is the most typical configuration for 20 MHz CBW and 30 kHz SCS. Issue 2-3: MCS Option 1 and 2 are fine for us. Same time, option 1 is slightly preferred because it has the higher gap between performance with and without soft combining. Issue 2-5: Rx assumptions for simulation alignment We think that is up to each company which assumptions are considered for results preparation (i.e. similar to procedures which are consider for all other WI). Therefore, we prefer not to define any certain assumptions for Rx part. Issue 2-7: How to define the requirements for different supported max number of HARQ process Support Option 1. Requirements will be defined for AWGN conditions. Therefore, we don’t expect big performance difference for scenarios with different number of HARQ process. |
| CATT | Issue 2-1: Test design for UE supporting n48 HARQ processes Support option 1. Issue 2-2: Sub-channel size We propose option 1 because 15PRB sub-channel size enables higher spectral efficiency when three sub-channels are occupied for n64 case. Nevertheless, we can accept option 2 as well. Issue 2-3: MCS Support option 1. Issue 2-4: SCI format 2-A configuration Support option 1. Issue 2-5: Rx assumptions for simulation alignment Prefer to not consider CFO and TO for HARQ buffer soft combining test. Issue 2-6: SNR for requirements definition Support the recommended WF. Issue 2-7: How to define the requirements for different supported max number of HARQ process Support Option 1. Based on our simulation results, no obvious performance difference is observed for different supported maximum number of HARQ process. |

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2100630 | Intel: Based on our understanding, information about Noc can be removed from test configuration because this information is defined in Section 4.4.3 of 38.101-4 and applicable to all requirements in the specification. |

## Summary for 1st round

### Open issues

Status summary

Agreements Issue 2-1: Test design for UE supporting n48 HARQ processes - Add the following test to verify UE support receive 48 HARQ processes per slot: “For verification of n48, the first 31 UEs transmit signal one by one circularly for every slot and in the first subchannel. The next 17 UEs transmit signal one by one circularly for every slot and in the second subchannel” Issue 2-2: Sub-channel size - 10 PRBs Issue 2-4: SCI format 2-A configuration - Payload = 35 Bits - βoffset = 2.5 Issue 2-7: How to define the requirements for different supported max number of HARQ process - One set of performance requirements based on the results of n64 Issues for discussion in the 2nd round Issue 2-3: MCS - Option 1 (CATT, Intel slight preference, Huawei, MTK slight preference, CATT): MCS 28 - Option 2 (QC, MTK, Intel): MCS 13 Issue 2-5: Rx assumptions for simulation alignment - Option 1 (Huawei): Consider 600Hz frequency offset and 12Ts time offset at receiving offset at Rx side - Option 2 (QC, MTK, CATT): Use TO and FO equal to 0. - Option 3 (Intel): Leave it up to individual company assumptions.

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2100630 | To be further discussed to collect more comments |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: PSFCH decoding capability test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100408 | CATT | Proposal 5: Configure 20MHz CBW/30kHz SCS for PSFCH decoding capability test. Proposal 6: Configure one sub-channel for full channel bandwidth for PSCCH/PSSCH decoding capability test. |
| R4-2100410 | CATT, GOHIGH | Simulation results of multiple link demodulation test cases based on the simulation assumptions |
| R4-2100629 | Qualcomm, Inc. | Proposal 4: Set 40MHz channel bandwidth and 10 subchannels for PSFCH detection capability test. |
| R4-2100662 | LG Electronics Inc. | Proposal 3: Use 30MHz or 40MHz channel bandwidth configuration to perform test for all UE capability under the same conditions for PSFCH decoding capability test. |
| R4-2101239 | Intel Corporation | Proposal 1: Use the following simulation assumptions for Rel-16 V2X PSFCH decoding capability test - CBW/SCS configuration: 40 MHz/30 kHz - Number of sub-channels: 10 |
| R4-2101360 | Huawei, HiSilicon | Draft CR: Introduction of PSFCH decoding capability requirements for NR V2X |
| R4-2101361 | Huawei, HiSilicon | Proposal 1: Use 20MHz for PSFCH decoding capability test. Observation 1: When no external noise is input (SNR is infinite), the probability of ACK miss for 2 CS pairs multiplexed and 1 CS pair are close to 0. Proposal 2: It is feasible to use 2 CS pairs multiplexed for PSFCH decoding capability test. |

## Open issues summary

Issue 3-1: CBW/SCS

- Proposals

  - Option 1 (CATT, Huawei): 20 MHz, 30 kHz

  - Option 2 (QC, LGE, Intel): 40 MHz, 30 kHz

  - Option 3 (LG): 30 MHz, 30 KHz

- Recommended WF

  - Collect comments on options above with detailed description of UE capability

Issue 3-2: Resource pool configuration

- Proposals

  - Option 1 (CATT, HW): 1 sub-channel with full BW allocation

  - Option 2 (QC, Intel): 10 subchannels of size 10 PRBs (for 40 MHz/30kHz case)

- Recommended WF

  - Collect comments on options above taking into account discussion on Issue 3-1

## Companies views’ collection for 1st round

### Open issues

| Company | Comments collection |
| --- | --- |
| Huawei, HiSilicon | Issue 3-1: CBW/SCS The number of PSFCH resource that the UE can receive in a slot: {n5, n15, n25, n32, n35, n45, n50, n64} Some companies concern that 20MHz bandwidth will lead to the situation that 2 CS pairs are multiplexed on one RB for psfch-RxNumber=64 and the performance maybe impact and evaluation is needed, actually this test is conducted under static channel without external noise, no issue for UE to decode the PSFCH during the testing; Also if we use 40MHz to verify other number of psfch-RxNumber than n64, it will cause serious waste of resources. For example, only 5RBs are effectively used for verification of psfch-RxNumber=5 but with total number of 106 RBs used during the testing. Issue 3-2: Resource pool configuration Prefer Option 1 that is applicable for different channel bandwidth. |
| QC | Issue 3-1: We support option 1. Since this is capability test, it is preferred to verify UE can handle maximum number of PSFCH spread out across the entire maximum possible BW. For Huawei’s comment above, we don’t see why larger configured BW with smaller number of PSFCH Rx capability is a waste of resources, since BW configuration is not based on PSFCH Rx capability. Issue 3-2: We support option 2, same reason as issue 3-1. |
| LG | Issue 3-1: CBW/SCS Support option 2. To test all UE capability for PSFCH decoding under the same condition, 30MHz or 40MHz CBW with 30kHz SCS is reasonable. |
| Intel | Issue 3-1: CBW/SCS Support Option 2, because it allows to simplify test setup. Also, based on our understanding, 40 MHz is mandatory for UE which support bands with PC5 which include 40 MHz CBW. Issue 3-2: Resource pool configuration Support Option 2, because configuration with multiple sub-channels is more typical for scenarios with multiple PSFCHs. |
| CATT | Issue 3-1: CBW/SCS Prefer option 1. Based on our simulation results, there is no evident performance difference between one PRB with one PSFCH transmission and two PSFCH transmissions. So it is slightly preferred to configure 20MHz CBW/30kHz SCS for PSFCH decoding capability test. We can introduce 40MHz CBW for PSSCH/PSCCH decoding capability test. Issue 3-2: Resource pool configuration Prefer option 1 also open to option 2. |

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2101360 | LG: need to revise the title of sub-section (11.1.X) |
|  | Intel: Based on our understanding, information about Es can be removed, because it is defined in Section 4.4.4 and applicable to all requirements in TS. |

## Summary for 1st round

### Open issues

Status summary

No agreements in the first round Issues for discussion in the 2nd round Issue 3-1: CBW/SCS - Option 1 (CATT, Huawei): 20 MHz, 30 kHz - Option 2 (QC, LGE, Intel): 40 MHz, 30 kHz - Option 3 (LG): 30 MHz, 30 kHz Issue 3-2: Resource pool configuration - Option 1 (CATT, HW): 1 sub-channel with full BW allocation - Option 2 (QC, Intel): Multiple subchannels of size 10 PRBs (i.e. 10 for 40 MHz case, 7 for 40 MHz case, 5 for 20 MHz case)

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101360 | To be further discussed to collect more comments |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: PSCCH/PSSCH decoding capability test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101065 | MediaTek inc. | Proposal 8: 40MHz CBW should be configured for PSCCH/PSSCH decoding capability test. Proposal 9: Not to verify 2*floor(NRB /10 RBs) capability for PSCCH/PSSCH decoding capability test. Proposal 10: Configure PSFCH within every slot in PSCCH/PSSCH decoding capability test. |
| R4-2100408 | CATT | Proposal 3: To verify the worst case of PSCCH/PSSCH decoding capability test, the maximum CBW 40MHz is proposed to be tested. Proposal 4: Configure PSFCH with 1 periodicity for PSCCH/PSSCH decoding capability test. |
| R4-2100410 | CATT, GOHIGH | Simulation results of multiple link demodulation test cases based on the simulation assumptions |
| R4-2100629 | Qualcomm, Inc. | Observation 3: UE passing PSCCH and PSSCH decoding capability test with 40MHz is expected to fulfill the corresponding requirement with bandwidth reduced to 20MHz. Observation 4: All the channel bandwidth options in a band have to be mandatorily supported by an SL UE, if the UE declares the support of the band. Observation 5: The capability test is supposed to verify UE decoding capability for the maximum bandwidth among the bands it supports, instead of verify UE decoding capability of a particular deployment scenario. Proposal 3: Set 40MHz channel bandwidth for PSCCH and PSSCH decoding capability test. |
| R4-2100662 | LG Electronics Inc. | Observation: depending on capability of psfch-TxNumber, PSFCH feedback cannot be used for PSCCH/PSSCH decoding capability test. Proposal 2: Select one alternative for PSCCH/PSSCH decoding capability test - Alt1: use AT command for only PSCCH/PSSCH decoding capability test - Alt2: use PSFCH feedback for PSCCH/PSSCH decoding capability test and introduce different test configuration depending on capability of psfch-TxNumber |
| R4-2101240 | Intel Corporation | Proposal 1: Use 40 MHz CBW for Rel-16 V2X PSCCH/PSSCH decoding capability test Proposal 2: Do not verify pscch-RxSidelink = 2*floor(NRB /10 RBs) capability in Rel-16 V2X PSCCH/PSSCH decoding capability test Proposal 3: Don’t configure PSFCH feedback and use AT commands for Rel-16 V2X PSCCH/PSSCH decoding capability test |
| R4-2101362 | Huawei, HiSilicon | Draft CR: Introduction of PSSCH/PSCCH decoding capability requirements for NR V2X |
| R4-2101363 | Huawei, HiSilicon | Proposal 1: Only test 20MHz bandwidth. Proposal 2: Set PSFCH periodicity to 1. Proposal 3: Not to verify 2*floor(NRB /10 RBs) capability |

## Open issues summary

Issue 4-1: CBW/SCS

- Proposals

  - Option 1 (CATT, QC, Intel, LG): 40 MHz, 30 kHz

  - Option 2 (Huawei): 20 MHz, 30 kHz

  - Option 3 (LG, Huawei as compromise, MTK): Flexibly CBW depending on psfch-TxNumber capability:

    - 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4.

- Recommended WF

  - Collect comments on options above with detailed description of UE capability

Issue 4-2: PSFCH configuration

- Proposals

  - Option 1 (MTK, CATT, Huawei): Set PSFCH periodicity to 1.

  - Option 2 (LGE, Intel, QC): Use AT command for only PSCCH/PSSCH decoding capability test

  - Option 3 (LGE): Use PSFCH feedback for PSCCH/PSSCH decoding capability test and introduce different test configuration depending on capability of psfch-TxNumber

    - For example, 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4.

- Recommended WF

  - Collect comments on options above

Issue 4-3: Verification of 2*floor(NRB /10 RBs) capability

- Proposals

  - Option 1 (MTK, Intel, Huawei, QC, LG, CATT): Not to verify 2*floor(NRB /10 RBs) capability

- Recommended WF

  - Check whether Option 1 is acceptable for all interested companies?

## Companies views’ collection for 1st round

### Open issues

| Company | Comments collection |
| --- | --- |
| Huawei, HiSilicon | Issue 4-2: CBW/SCS Considering the test of psfch-TxNumber =n16, CBW 40MHz should be used with floor(NRB /10 RBs) capability. We can compromise to option 3 of 40MHz for psfch-TxNumber =n16; 20MHz for psfch-TxNumber =n8 and 10MHz for psfch-TxNumber =n4. Thus UE supporting different number of psfch-TxNumber can be tested based on UE capability. Issue 4-3: Verification of 2*floor(NRB /10 RBs) capability We support Option 1. |
| QC | Issue 4-1 We support option 1. We don’t see how UE can declare support of n47 but not 40MHz in n47, this has been verified in previous meeting that when UE declares support of n47, it must support 40MHz. If PSFCH Tx capability is a concern, AT command can be used. Since 50MHz is left in n47, it is possible that 40MHz CBW being allocated. Issue 4-2 We support option 2. For option 3, the PSCCH capability testing depends on the capability of PSFCH Tx, but feedback is not necessary to be configured for all transmission, therefore, bundling PSCCH/PSCH capability test with PSFCH Tx capability is problematic. Issue 4-3 Option 1 is good. |
| LG | Issue 4-1: CBW/SCS, Issue 4-2: PSFCH configuration Depending on capability of psfch-TxNumber, channel bandwidth configuration can be different. For example, 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4. Otherwise, RAN4 needs to consider AT command for this test. Issue 4-3: Verification of 2*floor(NRB /10 RBs) capability Support option 1, but 2*floor(NRB /10 RBs) capability can be discussed in future release. |
| MTK | Issue 4-1/ Issue 4-2 From our understanding, the current specification doesn’t define UE dedicated signalling to inform UE which channel bandwidth should be supported. Besides, we have agreed that PSFCH feedback instead of AT commands can be used to test NR V2X UE’s performance. In order to unify the test procedure, we don’t support use AT commands in this case. Considering the concern from other companies about PSFCH capability, LG’s suggestion can be as a baseline and further discuss. Issue 4-3 Support Option 1. |
| Intel | Issue 4-1: CBW/SCS Support Option 1, because based on our understanding, 40 MHz CBW is mandatory in case UE support PC5 bands. Issue 4-2: PSFCH configuration Support Option 2, because it is the only way to verify PSCCH/PSSCH decoding capability for the large CBW configuration which is the main target of this test. If Option 3 will be considered, then we will have dependency on psfch-TxNumber capability and we will not be able to verify PSCCH/PSSCH decoding capability fully for DUTs with n4 or n8 capability. |
| CATT | Issue 4-1: CBW/SCS Support option 1. Configure 40MHz is to verify the worst case of PSCCH/PSSCH decoding capability test. If 40MHz CBW/30kHz SCS could pass the test, the performance of 20MHz CBW/30kHz SCS can be guaranteed. Issue 4-2: PSFCH configuration Prefer option 1 and open to further discuss option 3. Issue 4-3: Verification of 2*floor(NRB /10 RBs) capability Support option 1. |

### CRs comments collection

| CR number | Comments collection |
| --- | --- |
| R4-2101362 | QC: Come back after all issues are agreed |
|  | LG: need to revise the title of sub-section (11.1.X) |
|  | Intel: Based on our understanding, information about Es can be removed, because it is defined in Section 4.4.4 and applicable to all requirements in TS. |

## Summary for 1st round

### Open issues

Status summary

Agreements Issue 4-3: Verification of 2*floor(NRB /10 RBs) capability - Not to verify 2*floor(NRB /10 RBs) capability in Rel-16 V2X Issues for discussion in the 2nd round (based on GTW discussion) - Further discuss the feedback mode for PSCCH/PSSCH decoding capability test: - Option1: AT command and 40 MHz, 30 kHz. - Option2: Using PSFCH with PSFCH with periodicity 1 and flexibly CBW depending on psfch-TxNumber capability: - 40MHz channel bandwidth is for the UE supporting psfch-TxNumber =n16, 20MHz/30MHz channel bandwidth is for the UE supporting psfch-TxNumber =n8, and 10MHz channel bandwidth is for the UE supporting psfch-TxNumber =n4. - Further check with TE vendors to get their feedback regarding feasibility and test complexity.

### CRs

| CR number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2101362 | To be further discussed to collect more comments |

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #5: SDR with active sidelink test

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101065 | MediaTek inc. | Proposal 11: Not to define SDR with active sidelink test case. |
| R4-2100408 | CATT | Proposal 7: To verify the UE supporting con-current operation between Uu and SL, it is proposed to introduce SDR test with active sidelink. |
| R4-2101364 | Huawei, HiSilicon | Proposal 1: Not introduce the SDR test with active sidelink. |
| R4-2101943 | Intel Corporation | Proposal 1: Define SDR requirements with active sidelink in the scope of Rel-16 V2X. Proposal 2: Further discuss the methodology for verification of SL processing on PC5 CC for SDR requirements with active sidelink and consider the following options: - Option 1: Use HARQ soft buffer test. - Option 2: Use PSCCH/PSSCH decoding capability test - Option 3: Define new single link test with full CBW allocation and configurable MCS based on maximum data rate equation. |

## Open issues summary

Issue 5-1: Whether to define the SDR test with active sidelink

- Proposals

  - Option 1 (MTK, Huawei, QC, LG): No

  - Option 2 (CATT, Intel): Yes

- Recommended WF

  - Collect more comments on this issue with detailed justification why one or another option is supported.

Issue 5-2: Methodology for verification of SL processing on PC5 CC (in case requirements will be defined)

- Proposals

  - Option 1 (Intel): Use HARQ soft buffer test.

  - Option 2 (Intel): Use PSCCH/PSSCH decoding capability test

  - Option 3 (Intel): Define new single link test with full CBW allocation and configurable MCS based on maximum data rate equation.

- Recommended WF

  - Collect views from interested companies.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments collection |
| --- | --- |
| Huawei, HiSilicon | Issue 5-1: Whether to define the SDR test with active sidelink Option 1. This feature is an optional with capabiliy signalling feature. Considering so many tests have been introduced and this is not typical scenario, we propose not to introduce the concurrent SDR test. |
| QC | Issue 5-1 We support option 1. |
| LG | Issue 5-1: Whether to define the SDR test with active sidelink Option 1. RAN4 has been discussing this test cases for a couple of meetings, but there have no further progress. So we suggest to postpone this issue in future release. |
| MTK | Issue 5-1: Whether to define the SDR test with active sidelink Support Option 1. |
| Intel | Issue 5-1: Whether to define the SDR test with active sidelink Support Option 2. In Rel-16 we don’t have any requirements for scenarios with concurrent operation. This test is one of important requirements for these scenarios. If it is fine for all companies not to cover scenarios with concurrent operation in Rel-16 then we can compromise to come back with this topic in the next release. |
| CATT | Issue 5-1: Whether to define the SDR test with active sidelink Prefer option 2. In Rel-16, con-current operation is considered as one of primary scenarios for NR V2X. Multiple concurrent band combinations were also introduced in Rel-16. Considering the majority companies have concern on workload, it is acceptable to us that SDR test will be introduced in future release. |

### CRs comments collection

N/A

## Summary for 1st round

### Open issues

Status summary

GTW agreements - Not define the SDR test cases with active sidelink in Rel-16 V2X WI. - RAN4 also realize with such agreements, the co-current operation has not be verified by Rel-16 RAN4 demodulation requirements. RAN4 can further discuss and define corresponding test cases if needed in Rel-17 timeframe i.e. Rel-17 side-link enhancement WI.

### CRs

N/A

## Discussion on 2nd round (if applicable)

## Summary on 2nd round (if applicable)

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |