3GPP TSG-RAN WG4 Meeting #98-e	R4-2103998

Electronic Meeting, 25th Jan. – 5th Feb., 2021

Title: 	Simulation assumption for NTN co-existence study

Source: 	CATT

Agenda item:	11.8.3.1

Document for:	Discussion

- Introduction

This document captures initial simulation assumptions for the NTN coexistence study.

- Discussion

2.1 Co-existence simulation scenarios

In [3], the proposed scenarios for coexistence study are duplicated in the following table.

Table 2.1-1 Scenarios for NTN-NTN/TN co-existence

| FR1: 2GHz |  |  | Set 1 |  |  | Set 2 |  |  | HAPS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | GEO | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |  |
| NR / NB-IoT | Rural |  | X | X | X | X | X | X | FFS |
|  | Urban macro |  | X | X | X | X | X | X | FFS |
|  | Dense Urban |  | X | X | X | X | X | X | FFS |
|  | Indoor |  | X | X | X | X | X | X |  |
| NTN | GEO | Set 1 | X | X | X | N/A | N/A | N/A | FFS |
|  | LEO 1200km |  | X | X | X | N/A | N/A | N/A | FFS |
|  | LEO 600km |  | X | X | X | N/A | N/A | N/A | FFS |
|  | GEO | Set 2 | N/A | N/A | N/A | X | X | X | FFS |
|  | LEO 1200km |  | N/A | N/A | N/A | X | X | X | FFS |
|  | LEO 600km |  | N/A | N/A | N/A | X | X | X | FFS |
| Note 1: Start with Earth Fixed beam first, Earth Moving Beams could be further discussed Note 2: Set 1 and Set 2 could be found in Table 6.1.1.1-6 of TR 38.821. FFS if one set would be more stringent and so, if all simulations would be needed for both sets. Note 3: GEO and LEO only operate at adjacent channel. |  |  |  |  |  |  |  |  |  |

The aggressor and victim combination is list in Table 2.1-2.

Table 2.1-2 Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL |  |
| 2 | TN with NTN | TN UL | NTN UL |  |
| 3 | TN with NTN | NTN DL | TN DL |  |
| 4 | TN with NTN | NTN UL | TN UL |  |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S band, e.g. coexistence with Band 34 TDD. |
| 7 | NTN with NTN | NTN DL | NTN DL | LEO-LEO or GEO-GEO |
|  |  | NTN UL | NTN UL | LEO-LEO or GEO-GEO |

The proposed frequency and bandwidth are listed as table 2.1-3.

Table 2.1-3.  Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| Rural | 2 GHz | TBD | FDD, TDD | [1] |
| Urban macro | 2 GHz | TBD | FDD, TDD | [1] |
| Dense Urban | 2 GHz | TBD | FDD, TDD | [1] |
| GEO | 2 GHz | 30 MHz for FR1 | FDD | [2] or [3] |
| LEO | 2 GHz | 30 MHz for FR1 | FDD | [2] or [3] |
| HAPS | 2 GHz | TBD | FDD | [1] |

2.2. Network layout model

Cellular cell structure is considered for both NTN and TN network layout.

Co-existence between NTN and TN

For co-existence between NTN and TN, it is proposed to consider [TBD] satellite(s) and the layout is FFS. The number of TN IMT BS should be large enough to emulate the interference seen by the satellite from the IMT systems. It is FFS on exact range of TN BS deployment based on simulations.

Co-existence between NTN and NTN

For co-existence between NTN and NTN, the following 2 cases are considered as [candidate options].

- One satellite carries two neighbour carriers, where the footprints of the 2 carriers are the same and coordinated see figure 2.2-1.

- Two satellites (GEO and LEO) operate on two neighbour carriers but at different height, see figure 2.2-2. The number of LEO satellite and footprints are FFS.

Co-existence between HAPS and TN

For co-existence between HAPS and TN, the exact layout is FFS.

Figure2.1-1 Layout for coexistence between NTN and TN (TBD)

Figure 2.2-2 Layout for coexistence between NTN systems

Figure 2.2-3 Layout for coexistence between NTN systems (different height satellites)

2.3. Simulation parameters

Two sets of satellite parameters are proposed in Table 2.3-1 and Table 2.3-2 according to TR 38.821.

Table 2.3-0 Simulation assumptions of TN respectively based on NB-IoT and NR

|  | NB-IoT standalone | NR |
| --- | --- | --- |
| Carrier frequency in GHz | 2 | 2 |
| Size of each nominal channel BW in MHz | 0.2 | 20 |
| Transmission bandwidth in MHz | 0.18 | 9 |
| Environment | Urban macro Sub-urban Rural | Urban macro Sub-urban Rural |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 500 for 2GHz band for UMA | 500 for 2GHz band for UMA |
| System loading and activity | Full buffer 100% | Full buffer 100% |
| Network location | FFS | FFS |
| DL subcarrier spacing | 15kHz | 15kHz |
| UL | See RP-152284 | OFDMA |
| DL power control | No | No |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | 36.942 section 5.1.1.6 (set=1) |
| Frequency reuse | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | 3 |
| BS antenna height in meter | 30 | 30 |
| BS max TX power in dBm | 43dBm/200kHz | 46 |
| BS antenna gain including feeder loss in dBi | 15 | 15 |
| BS antenna pattern | Horizontal (36.942) | [8x4x2 AAS BS] |
| BS antenna front-back ratio in dB | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 |
| Building penetration loss | 45.820 Annex D.1 | n/a |
| Cell selection margin in dB | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 |
| BS noise figure in dB | 5 | 5 |
| UE noise figure in dB | 9 | 9 |
| BS-UE path-loss model | TR36.942 macro urban | TR38.803 |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  | section 2.9 |
| Evaluation metrics | SINR vs ACS (as victim) | SINR and throughput loss vs standalone NB-IoT ACLR (as victim); |

Table 2.3-1 Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite Tx max Gain |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| Satellite beam diameter |  | 250 km | 90 km | 50 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  | 51 dBi | 30 dBi | 30 dBi |

Table 2.3-2 Set-2 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for DL transmissions |  |  |  |  |
| Satellite EIRP density | 2GHz | 53.5 dBW/MHz | 34 dBW/MHz | 28 dBW/MHz |
| Satellite Tx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |
| 3dB beamwidth |  | 0.7353 deg | 8.8320 deg | 8.8320 deg |
| Satellite beam diameter |  | 450 km | 190 km | 90 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| G/T | 2 GHz | 14 dB K-1 | -4.9 dB K-1 | -4.9 dB K-1 |
| Satellite Rx max Gain |  | 45.5 dBi | 24 dBi | 24 dBi |

Table 2.3-3 HAPS parameters for co-existence study (TBD)

Table 2.3-4 UE characteristics for co-existence study

| Characteristics | Handheld |
| --- | --- |
| Frequency band | 2 GHz |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |

Table 2.3-5 ACLR/ACS for TN  (2GHz)

|  |  | NR | NB-IOT |
| --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB |
|  | ACS | 45 dB | 45 dB |
| UE | ACLR | 30dB (ACLR1) 43dB (ACLR2) | 37 |
|  | ACS | 33 | 28 |

Table 2.3-6: Other simulation parameters for NR

| Parameters | NR | NB-IOT | NTN |
| --- | --- | --- | --- |
| Carrier frequency | 2GHz | 2GHz | 2GHz |
| Channel bandwidth | 20MHz | 200kHz | 5MHz |
| Scheduled channel bandwidth per UE (DL) |  |  |  |
| Scheduled channel bandwidth per UE (UL) |  |  |  |
| The number of active UE (DL) (Note 1) | 1 | 1 | 1 |
| The number of active UE (UL) (Note 1) | 1/3 | 1/3 | 1/3 |
| Traffic model | Full buffer | Full buffer | Full buffer |
| DL power control | NO | NO | NO |
| UL power control | YES | YES | TBD |
| BS max TX power in dBm | 43 | 43 | TBD |
| UE max TX power in dBm | 23 | 23 | 23 |
| UE min TX power in dBm | -33 | -33 | -33 |
| BS Noise figure in dB | 5 (@2GHz) | 5 (@2GHz) | TBD |
| UE Noise figure in dB | 9 | 9 | 7 |
| Handover margin | 3dB | 3dB | 3dB |
| Note 1: Further down scope between 1 and 3 will be done based on later simulations. |  |  |  |

  - Antenna and beam forming pattern modelling

Satellite and UE Antenna and beam forming pattern modelling of satellite could be referred to section 6.1.1.1 in TR 38.821 [5].

Antenna and beam forming pattern modelling of TN BS and UE could be referred to TR 38.803 [6].

The antenna and beam forming pattern modeling for HAPS is FFS.

2.5. Propagation model

Propagation model between NTN and UE could be referred to section 6.6 in TR 38.811 [5].

Propagation model between TN BS and UE could be referred to section 5.2.2 in TR 38.803 [6].

Propagation model between NTN BS and TN BS should reference to TS 38.811 which is used for DL-UL cross link interference for S band.

Propagation model between HAPS BS and UE is defined in TR 38.811 [5]

2.6. Transmission power control model

For downlink scenario, no power control scheme is applied.

For uplink scenario, TPC model specified in Section 9.1 TR 36.942 [7] could be applied for TN with following parameters.

Where, Pmax = 23dBm, Rmin = TBD dB, CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y,

where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario, TPC model for NTN is FFS.

2.7. Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

Where,

RX_PWR is the received power

TX_PWR is the transmitted power

G_TX is the transmitter antenna gain (directional array gain)

G_RX is the receiver antenna gain (directional array gain).

2.8. Performance metric

For NR,

The average throughput loss and 5%-ile throughput loss should be less than 5%.

For NB-IOT,

The average throughput loss and SNR loss, 5%-ile throughput loss and SNR loss should be according to 36.802

For NTN,

FFS

2.9. Throughput ~ SNR mapping

Use section 5.2.7 of TR 38.803.

- Conclusion

It is proposed to use the simulation assumptions in this paper as the starting point for NTN co-existence study.

- Reference

[1] R4-2100486

[2] R4-2100904

[3] R4-2101105

[4] R4-2101812

[5] R4-2101859

[6] R4-2101964

[7] R4-2102174

[8] R4-2102508