3GPP TSG-RAN WG4 Meeting #98-e	R4-2100012

e-Meeting, January 25th – February 5th, 2021

3GPP TSG-RAN WG1 Meeting #103-e	R1-2009676

e-Meeting, October 26th – November 13th, 2020

# Title:	LS on uplink Tx switching

# Response to:

# Release:	Rel-16

# Work Item:	NR_RF_FR1

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:	TSG RAN WG4

Contact Person:

Name:	Jianchi Zhu

E-mail Address:	zhujc@chinatelecom.cn

1. Overall Description:

RAN1 discussed the supported max data rate for uplink Tx switching and has reached the following agreements in RAN1#103-e meeting:

Agreement:

- Adopt the following TP (38.306, Section 4.1.2).

< unchanged text omitted> ### 4.1.2 Supported max data rate For NR, the approximate data rate for a given number of aggregated carriers in a band or band combination is computed as follows. wherein J is the number of aggregated component carriers in a band or band combination Rmax = 948/1024 For the j-th CC, is the maximum number of supported layers given by higher layer parameter maxNumberMIMO-LayersPDSCH for downlink and maximum of higher layer parameters maxNumberMIMO-LayersCB-PUSCH and maxNumberMIMO-LayersNonCB-PUSCH for uplink. is the maximum supported modulation order given by higher layer parameter supportedModulationOrderDL for downlink and higher layer parameter supportedModulationOrderUL for uplink. is the scaling factor given by higher layer parameter scalingFactor and can take the values 1, 0.8, 0.75, and 0.4. is the numerology (as defined in TS 38.211 [6]) is the average OFDM symbol duration in a subframe for numerology , i.e. . Note that normal cyclic prefix is assumed. is the maximum RB allocation in bandwidth with numerology , as defined in 5.3 TS 38.101-1 [2] and 5.3 TS 38.101-2 [3], where is the UE supported maximum bandwidth in the given band or band combination. is the overhead and takes the following values 0.14, for frequency range FR1 for DL 0.18, for frequency range FR2 for DL 0.08, for frequency range FR1 for UL 0.10, for frequency range FR2 for UL NOTE 1: Only one of the UL or SUL carriers (the one with the higher data rate) is counted for a cell operating SUL. NOTE 2: For UL Tx switching between carriers in cell(s), only the supported MIMO layer combination across carriers that results in the highest combined data rate is counted for the cell(s) in the supported maximum UL data rate. < unchanged text omitted>

2. Actions:

To RAN WG2:

ACTION:

RAN1 respectfully asks RAN2 to take the above information into account.

3. Date of Next RAN WG1 Meetings:

TSG RAN WG1 Meeting #104-e            25 Jan. - 05 Feb., 2021            e-meeting

TSG RAN WG1 Meeting #104-bis-e      12 Apr. - 20 Apr., 2021              e-meeting