3GPP RAN WG4 Meeting #98-e		R4-2100517

Online, 25th January – 05th February 2021

Agenda item:	9.22.3

Source:	Apple

Title:	A-MPR Proposal for n2 and 35MHz CBW

WI/SI:	NR_FR1_35MHz_45MHz_BW-Core

Release:	Rel-17

Document for:	Approval

# 1	Introduction

The latest revision of the WID introduces 35MHz CBW to band n2 [1]. Simulations have been done for 35MHz channel bandwidth and NS_03.  This paper proposes dedicated A-MPR for NS_03 35MHz CBW.

# 2  Discussion

###### 2.1 Simulation assumptions

The following assumptions for the simulations were used:

- Power Class 3

- Calibration: 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB

- Carrier Leakage: 28dBc

- Image: 28dBc

- CIM3: 60dBc

- Coexistence requirements of n2

The simulations were done for 35MHz channel bandwidth with center frequency of 1867.5MHz and 1892.5MHz with NS_03. The simulations suggest that for 35MHz channel bandwidth also inner allocations require additional power reduction. This is especially true for DFT-s-OFDM with PI/2 BPSK and QPSK.

Table 1

| Channel BW | Carrier Frequency Fc | Waveform | Modulation | Outer | Inner |
| --- | --- | --- | --- | --- | --- |
| 35MHz | 1867.5<=Fc <= 1892.5 | DFT-s-OFDM | PI/2 BPSK | 2.5 | 0.5 |
|  |  |  | QPSK | 3.0 | 0.5 |
|  |  |  | 16QAM | 3.0 | 1.0 |
|  |  |  | 64QAM | 3.5 | 2.5 |
|  |  |  | 256QAM | 5.5 | 4.5 |
|  |  | CP-OFDM | QPSK | 4.5 | 2.0 |
|  |  |  | 16QAM | 4.5 | 2.5 |
|  |  |  | 64QAM | 4.5 | 3.5 |
|  |  |  | 256QAM | 7.5 | 6.5 |

Proposal: Use A-MPR proposed in Table 1 for 35MHz CBW NS_03.

# 3  Simulations

# Conclusions

This paper presents A-MPR simulation results for n2 with 35MHz CBW for NS_03.

Proposal: Use A-MPR proposed in Table 1 for 35MHz CBW NS_03.

Proposal: Use A-MPR proposed in Table 1 for 35MHz CBW NS_03.

# 4	References

- RP-202607, “Introduction of channel bandwidths 35MHz and 45MHz for NR FR1”, Huawei, HiSilicon, 3GPP TSG RAN Meeting #90e