3GPP RAN WG4 Meeting #98e		R4-2100520

Electronic meeting, 25th January – 5th February 2021

Agenda item:	12.2.4

Source:	Apple Inc.

Title:	Regulatory status for the frequency range 52.6 GHz to 71 GHz

WI/SI:	FS_NR_52_to_71GHz

Release:	Rel-17

# 1	Introduction

As a first step for enabling even higher frequency ranges for the NR radio system, 3GPP RAN was studying frequency bands above 52.6GHz and up to 114.25GHz [1], for which the corresponding TR 38.807 [2] captured global spectrum availability, regulatory requirements, and potential use cases. After RAN#86 meeting, a new SI was agreed that aims at studying more specific technical aspects on how the NR technology can be adopted to the 60GHz frequency range [3]. Referring to the corresponding SI description, the scope has been narrowed down to the operation between 52.6 GHz and 71 GHz. It is worth mentioning that even though TR 38.807 was capturing the 60GHz regulatory status in different regions and countries, it was not updated after the corresponding SI was formally completed in December 2019. Since that time more regulatory decisions were made outcome of which it would be preferable to capture to facilitate further discussions on the band plan.

This issue was discussed during the RAN4#97 meeting, during which some companies expressed the preference to capture/update the latest regulatory status for the 60GHz frequency range [4][5], but no conclusion was reached where exactly it should be captured. The same problem was raised again during the RAN#98 meeting, during which it was concluded that it is possible to bring the CR to TR 38.807.  Thus, in this discussion paper we present the content of a draft TR 38.807 CR [6] that introduces further regulatory updates.

# 2	60GHz regulatory status

As already mentioned in the Introduction part, 3GPP RAN plenary already conducted a study that aimed at identifying available spectrum in higher frequency ranges, outcome of which was captured in TR 38.807. However, after that more decisions were made. Table 2-1 below presents a short overview of the latest licensing situation in different regions and countries for the frequency range of 52.6-71GHz.

Table 2-1: Licensing mode for various countries between frequency 52.6GHz and 71GHz.

| Region | Country | Frequency (GHz) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 52.6-54.25 | 54.25-55.78 | 55.78-56.9 | 56.9-57 | 57-58.2 | 58.2-59 | 59-59.3 | 59.3-64 | 64-65 | 65-66 | 66-71 |
| ITU Region 1 | Europe/CEPT |  |  |  |  | U |  |  |  |  |  |  |
|  | Israel |  |  |  |  |  |  |  |  |  |  |  |
|  | South Africa |  |  |  |  | U |  |  |  |  |  |  |
| ITU Region 2 | USA |  |  |  |  | U |  |  |  |  |  |  |
|  | Canada |  |  |  |  | U |  |  |  |  |  |  |
|  | Brazil |  |  |  |  | U |  |  |  |  |  |  |
|  | Mexico |  |  |  |  | U |  |  |  |  |  |  |
| ITU Region 3 | China |  |  |  |  | L |  |  |  |  |  |  |
|  | Japan |  |  |  |  | U |  |  |  |  |  |  |
|  | Korea |  |  |  |  | U |  |  |  |  |  |  |
|  | India |  |  |  |  |  |  |  |  |  |  |  |
|  | Taiwan |  |  |  |  | U |  |  |  |  |  |  |
|  | Singapore |  |  |  |  | U |  |  |  |  |  |  |
|  | Australia |  |  |  |  | U |  |  |  |  |  |  |

Based on the feedback from different companies expressed during the RAN4#97 meeting and the RAN#98 guidance, RAN WG4 could discuss and prepare updates for the 60GHz frequency range that will be captured in TR 38.807 (e.g. through the corresponding LS sent from RAN WG4 to TSG RAN). The corresponding draft CR [6], which is submitted to the RAN#98e meeting, contains some general updates as well as more specific technical to the ITU region 1 based on the latest EC/CEPT agreements.

Proposal:	Agree the content of the latest regulatory updates for the 52-71GHz frequency range and endorse the TR 38.807 CR to the RAN#99 meeting.

# 3	Conclusions

3GPP RAN has completed study on requirements for NR beyond 52.6 GHz in December 2019, whereupon the latest regulatory status for frequency ranges from 52.6GHz up to 114.25GHz was captured in TR 38.807. Since then, more regulatory decisions were made and it would be preferable to capture them to facilitate further 3GPP discussions on bands and band plan. The corresponding changes are presented in the draft CR [6].

Proposal:	Agree the content of the latest regulatory updates for the 52-71GHz frequency range and endorse the TR 38.807 CR to the RAN#99 meeting.

# 4	References

RP-182861, "Study on NR design above 52.6GHz (spectrum & regulatory)", Intel

TR 38.807, "Study on requirements for NR beyond 52.6 GHz"

RP-193259, "New SID: Study on supporting NR from 52.6GHz to 71 GHz", Intel

R4-2016982, "Email discussion summary for [97e][141] FS_NR_52_to_71GHz_Part_2", Moderator (Intel)

R4-2014894, "Regulatory overview and input for the 60GHz frequency range", Apple Inc.

R4-2100521, "Regulatory update for the 60GHz frequency range", Apple Inc.