3GPP TSG-RAN WG4 Meeting#98-e	R4- 2100541

Online, 25th Jan – 5th Feb, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on UE Power Saving Test Cases

Agenda item:	7.6.2

Document for:	Discussion

- Introduction

In the last meeting, WF [1] listed different options for PDCCH WUS parameters. In this paper, we discuss our views on test cases for UE power saving tests.

- PDCCH WUS Parameters

Based on simulation assumptions in WF [1], below are our simulation results for SNR at 1% Total PDCCH BLER.

Table 1: SNR in dB at 1% Total PDCCH BLER

| Case | PDCCH WUS Configuration | SNR in dB at 1e-2 Total PDCCH BLER | Impairment SNR in dB |
| --- | --- | --- | --- |
| FDD: 1x2 | 12 bits, AL8 | 4.19 | 6.19 |
| FDD: 1x2 | 36 bits, AL16 | 4.13 | 6.13 |
| FDD: 1x4 | 12 bits, AL8 | -1.72 | 0.28 |
| FDD: 1x4 | 36 bits, AL16 | -1.72 | 0.28 |
| FR1 TDD: 1x2 | 12 bits, AL8 | 0.88 | 2.88 |
| FR1 TDD: 1x2 | 36 bits, AL16 | 0.88 | 2.88 |
| FR1 TDD: 1x4 | 12 bits, AL8 | -2.64 | -0.64 |
| FR1 TDD: 1x4 | 36 bits, AL16 | -2.64 | -0.64 |
| FR2 TDD: 1x2 | 12 bits, AL8 | 0.62 | 2.62 |
| FR2 TDD: 1x2 | 36 bits, AL8 | 0.62 | 2.62 |

Based on above results, overall PDCCH performance doesn’t change much due to PDCCH WUS configuration. We also see that PDCCH WUS is very reliable at above SNRs for both configurations. Therefore, we propose the following.

Proposal 1: Use 12 bits, AL8 for PDCCH WUS for defining FR1 and FR2 UE power saving requirements.

- Conclusions

This paper discusses the parameters for PDCCH WUS. Following has been proposed:

Proposal 1: Use 12 bits, AL8 for PDCCH WUS for defining FR1 and FR2 UE power saving requirements.

# References

[1] R4-2017677, “WF on power saving demodulation”, 3GPP TSG RAN WG4 Meeting #97-e, Online, Nov 2 - Nov 13, 2020.