3GPP TSG-WG RAN4 Meeting #98-e	R4-2100730

# Online, 25th January – 5th February, 2021

Source: 				Nokia, Nokia Shanghai Bell, [Bell Mobility]

Title: 				TP to TR 36.717-02-01 CA_25-66

Agenda Item: 	13.1.3 [LTE_CA_R17_2BDL_1BUL]

Document for:	Approval

# Introduction

In this contribution, a text proposal to complete E-UTRA CA configurations, CA_25-66A and CA_25A-25A-66A, is provided.

# TP to TR 36.717-02-01

## 5.x	CA_25-66

### 5.x.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |

Table 5.x.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_25A-66A | - | 25 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| CA_25A-25A-66A | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  | 60 | 0 |
|  |  | 66 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

### 5.x.2 	Co-existence studies

Table 5.x.2-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_25-66.

Table 5.x.2-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| 25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | - | - |
| 66 | 1710 | 1780 | 2110 | 2200 | 3420 | 3560 | 5130 | 5340 | - | - |

Table 5.x.2-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 25 | 1850 | 1915 | 1930 | 1995 | 3700 | 3830 | 5550 | 5745 | - | - |
| 66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | - | - |

There is no harmonic or harmonic mixing issue for this band combination.

### 5.x.3	∆TIB and ∆RIB values

Relaxation values for this band combination is based on the similar combination, CA_2-4.

Table 5.x.3-1: IB,c

| CA_25-66 | 25 | 0.5 |
| --- | --- | --- |
|  | 66 | 0.5 |

Table 5.x.3-2: R IB,c

| CA_25-66 | 25 | 0.3 |
| --- | --- | --- |
|  | 66 | 0.3 |

### 5.x.4 	REFSENS

REFSENS exceptions are not required for this band combination due to no harmonic or harmonic mixing issue.