3GPP TSG-RAN WG4 Meeting #98-e	R4-2101046

Electronic Meeting, 25 Jan. – 5 Feb., 2021

Agenda item:	11.4.2.3

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on PUCCH SCell Activation/Deactivation delay requirements

Document for:	Discussion

- Introduction

The objective of this work item is to specify NR RRM further enhancement requirements for REL-17

- SRS antenna port switching [RAN4]

  - Specify RRM requirement for SRS antenna port switching, e.g.

    - Interruption requirement

    - Potential impact on other RRM requirements

- HO with PSCell [RAN4]

  - Determine the scenarios for HO with PSCell for which RRM requirements are to be specified

    - from NR SA to EN-DC

    - from EN-DC to EN-DC

    - from NE-DC to NE-DC

    - from NR-DC to NR-DC

  - Study the UE behavior for HO with PSCell

    - Existing requirements for HO and PSCell addition as baseline

    - Timeline and interaction between HO and PSCell addition

  - Specify RRM requirements for HO with PSCell based on agreed UE behavior

- PUCCH SCell activation/deactivation [RAN4]

  - Specify SCell Activation Delay Requirement for Deactivated PUCCH SCell (including valid TA and invalid TA)

  - Specify SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells (including valid TA and invalid TA)

  - Specify SCell Deactivation Delay Requirement for Activated PUCCH SCell

  - Specify SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells.

At the RAN #89e meeting, three new topics of Rel-17 NR RRM further enhancement are approved [1]. The objective of core part WI as follows:

In this contribution, we focus on the 3rd topic of PUCCH SCell activation/deactivation and provide our view.

- Discussion

According to the agreed work plan at RAN4 #97e meeting [2], we would like to focus on the single SCell PUCCH scenario firstly. In TS 36.133, for the SCell with valid TA, the activation delay requirement is same between normal SCell and PUCCH SCell. On the other hand for the SCell with invalid TA, the activation delay requirement for deactivated PUCCH SCell is relaxed by considering TA alignment time. For NR, the same manner should be applied and relaxation factor should be reconsidered.

Proposal 1: For NR, the same manner as LTE SCell activation delay requirement for deactivated PUCCH SCell should be applied and relaxation factor should be reconsidered.

Proposal 2: SCell activation delay requirement for deactivated PUCCH SCell with valid TA should be same as that of SCell activation delay requirement for deactivated SCell.

For SCell deactivation delay, if there are no special reasons, the delay requirement should be same as that of for the normal SCell deactivation.

Proposal 3: SCell deactivation delay requirement for activated PUCCH SCell should be same as that of for the normal SCell if there are no special reasons.

- Conclusion

In this contribution, we proposed our views on PUCCH SCell Activation/Deactivation delay requirements.

Proposal 1: For NR, the same manner as LTE SCell activation delay requirement for deactivated PUCCH SCell should be applied and relaxation factor should be reconsidered.

Proposal 2: SCell activation delay requirement for deactivated PUCCH SCell with valid TA should be same as that of SCell activation delay requirement for deactivated SCell.

Proposal 3: SCell deactivation delay requirement for activated PUCCH SCell should be same as that of for the normal SCell if there are no special reasons.

References

[1] 3GPP, RP-202053, Apple, Intel, “	WID of REL-17 NR RRM further enhancement”.

[2] 3GPP, R4-2014286, Apple, “Work plan for R17 Further RRM enhancement for NR and MR-DC WI”