3GPP TSG-RAN WG4 Meeting #98-e	R4-2101350

# Electronic Meeting, Jan. 25th – Feb. 5th, 2021

Title: 	Simulation results on NR-U PRACH performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.7.4.4

Document for:	Discussion

Background

At last meeting, RAN 4 agreed to define the performance requirements for wideband PRACH [1]. In this paper, we provide our initial simulation results.

Simulation results

Simulation results are shown in Figure 2-1 to 2-3 and summarized in Table 2-1 and simulation assumptions derived from [1] are shown in Appendix.

Figure 2-1: Simulation results for Format A2

Figure 2-2: Simulation results for Format B4

Figure 2-3: Simulation results for Format C2

Table 2-1: Summary of PRACH simulation results

| Test metric: SNR@ 1% of PRACH miss detection probability. | AWGN |  | TDLC-300-100 |  | TDLA-30-10 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LRA=1151 | LRA=571 | LRA=1151 | LRA=571 | LRA=1151 | LRA=571 |
| A2 | -23.36 | -20.34 | -17.4 | -13.9 | -18.3 | -14.7 |
| B4 | -27.02 | -24.0 | -20.6 | -17.5 | -21.1 | -18.4 |
| C2 | -23.7 | -20.6 | -17.3 | -14.0 | -18.1 | -14.9 |

Conclusion

In this paper, we provide the initial simulation results for NR-U PRACH.

Reference

[1]   Way forward on NR-U PRACH demodulation requirements. Nokia, Nokia Shanghai Bell

Appendix

Table 5-1: Simulation assumptions for wideband PRACH

| Parameters | Values |
| --- | --- |
| Format | A2, B4,C2 |
| SCS | 15kHz and 30kHz |
| Bandwidth | 20MHz |
| Ncs | 127 for LRA=1151 and 63 for LRA=571 |
| Antenna configuration | 1x2 |
| Propagation conditions | AWGN, TDLA-30-10,TDLC-300-100 |
| v | 0 |
| Logical sequence index | 0 |
| Time estimation error tolerance | For AWGN: 0.52 us for 15 kHz; 0.26 us for 30kHz. For TDLC: 2.03us for 15 kHz; 1.77us for 30kHz. For TDLA: 0.88 us for 15 kHz; 0.62 us for 30kHz. |
| Test metric | 0.1% of false alarm probability. 1% of PRACH detection error and time estimation error probability |