3GPP TSG-RAN WG4 Meeting #98	R4-2101370

Electronic Meeting, 25 Jan. – 5 Feb., 2021

Agenda item:	11.6.3.2

Source: 		NTT DOCOMO, INC.

Title: 	Views on HST CA tests for FR1

Document for:	Discussion

1. Introduction

Rel.17 WI on high-speed train (HST) scenario was approved [1]. This contribution presents our views on HST tests for Rel.17 NR.

2. Discussion

In Rel.16 NR, RAN4 introduced single carrier tests. In Rel.17 NR, RAN4 discusses to extend single carrier tests to carrier aggregation tests. We believe that this extension should at least include the FDD 15kHz and TDD 30kHz, which are considered for Rel-16 NR.

Proposal 1: For FDD, consider at least FDD 15kHz

Proposal 2: For TDD, consider at least TDD 30kHz

At the same time, in Rel.17 WI, RAN4 agreed to discuss the same target speed (up to 500km/h) and carrier frequency (up to 3.6 GHz) as Rel-16 WI. Since RAN4 spent a lot of time deciding on the maximum Doppler frequency under HST-SFN scenario in Rel.16 NR, we prefer to use the same maximum Doppler frequency in Rel.17 NR.

Proposal 3: Target maximum Doppler frequency under HST-SFN scenario in the tests are as follows.

  - For FDD 15kHz : 870Hz

  - For TDD 30kHz : 1667Hz

3. Conclusion

In this contribution, we present our views on HST tests for Rel.17 NR. Our proposals are summarized below.

Proposal 1: For FDD, consider at least FDD 15kHz

Proposal 2: For TDD, consider at least TDD 30kHz

Proposal 3: Target maximum Doppler frequency under HST-SFN scenario in the tests are as follows.

  - For FDD 15kHz : 870Hz

  - For TDD 30kHz : 1667Hz

References

- RP-202106, New WID on enhancement for NR high speed train scenario in FR1, CMCC