3GPP TSG-RAN WG4 #98-e	R4-2101460

Online, January 25th – February 5th, 2021

Agenda item:	7.19.1

Source: 	Qualcomm Incorporated

Title: 	Short Transient Period Testing

Document for:	Discussion

# Introduction

The discussion on the introduction of a shorter transient period has been ongoing for many meetings. A draft CR  introducing the shorter transient capabilities of 2,4 and 7us was agreed [1]. Some of the requirements such as EVM measurement and testing are still under discussion. In this paper we further clarify some testing aspects. This paper contains multiple sections that were already presented in [8] but not have been discussed yet.

# Discussion

## Previous discussions

The discussions in the previous meetings was centered around the open issues that were summarized in [2]. Most of those issues were addressed in [3] and captured in the e-mail discussion from Ran4#96e [4]. We would like to consider those issues closed since there were no further open issue brought up during the discussion.

One of the main open issues previously discussed was that the power change of ~20dB is not enough. This was clearly addressed in [3] and [4] from a system level and no further evidence of typical scenarios where the power change would be higher was presented. In the previous meeting a company commented that such large power changes (~55dB) is still possible because UE to UE distance was not considered or because IBE is normally better than 30dBc. It should be noted that UE to UE distance is not related to this discussion since the power control parameters set the target received power at the gNB, the power difference implied by these parameters is agnostic of the distance between UEs. Furthermore, the claim that IBE for all UEs would be much better than the actual specification is unfounded(no data was presented) and it seems very unlikely that the gNB would make such an assumption when configuring the power control parameters. As was previously explained, it should be further investigated if fractional power control is in use or not.

In [5] a new placement of the transient window was proposed, we discuss this proposal in the section below.

## Transient Period Placement

In [5] it was argued that placing the transient period asymmetric relative to the symbol boundary will improve the performance because the transient period will be cut out by the gNB receiver.

The likely gNB receiver window placement is depicted in Figure 1 below. The figure shows the UE Tx signal and multiple copies of the same signal received at the base station considering multipath fading. For a better intuitive understanding, the scale is intended to show a tp of 2us with 30kHz SCS.

It can be seen from the picture that if the tp is shorter, it is beneficial to place the transient within the CP or make it overlap with the part of the symbol that is somewhat distorted by WOLA(actual signal will be distorted anyway). If the tp is placed asymmetrically, the largest portion of the transient that falls within the BS processing window will come from the late arriving paths which should contain less energy than the earlier arriving paths.

Figure 1. BS Rx window for 2us tp and 30kHz SCS

Considering the above, the proposal in Table 1 from [5] is a good WF for defining the placement of the shorter transient period.

## EVM Measurement and Testing

The actual test metric for the short transient period is still under discussion. The previous proposal on how to measure the EVM shown in [3] was focused on re-using the currently defined EVM measurement windows to minimize the impact on the test equipment. However, some issues related to not having the same metric for each transient period capability and not having the ability to use any SCS for any transient period were brought up. The simplest way to avoid all these problems is to introduce new measurement windows that are defined specifically for each transient period. This approach was already discussed in [6],[7]. This is a new feature for Rel.16 and changing the location of the EVM measurement window is straightforward to implement.

The measurement procedure is further explained below. The location of the transient and the definition of the measurement window is depicted in Figure 2.

For full slot transmissions, the transient occurs in the last and first symbol of a slot. The new measurement window is designed to capture all the samples that are outside the transient period. These are depicted as EVMh_tp for the last symbol of the slot (transient occurs at the end of this symbol) and EVMl_tp for the first symbol of the slot(transient occurs at the beginning of this symbol). By defining these new windows, the transient period can be clearly isolated and the EVM on the samples outside the transient can be precisely measured. This new window definition also enables the use of the same metric and any SCS, for any of the newly introduced transient periods. Also, this approach is compatible with any tp placement(symmetric or asymmetric relative to the symbol boundary) so it can accommodate the tp placement discussed in Section 2.2 .

The test metric and applicability is shown in Table 1.

Fig. 2 EVM measurement window

Table 1. Test metric and applicability for the new transient period capability

| Reported transient capability (us) | EVM definition | SCS |
| --- | --- | --- |
| 2 | EVM0=max⁡(EVMl_new,EVMh) EVMlast=max⁡(EVMl,EVMh_new) | 15kHz or 30kHz |
| 4 | EVM0=max⁡(EVMl_new,EVMh) EVMlast=max⁡(EVMl,EVMh_new) | 15kHz or 30kHz |
| 7 | EVM0=max⁡(EVMl_new,EVMh) EVMlast=max⁡(EVMl,EVMh_new) | 15kHz or 30kHz |
| NOTE 1: EVMl , EVMh, EVMl_new,and EVMh_neware defined in Annex F NOTE 2: EVM0 is the EVM for a symbol right after a transition; EVMlast is the EVM for a symbol right before a transition |  |  |

The EVM should be measured on the last and first symbol and averaged over multiple instances. Also, EVM can be measured on all other symbols against the legacy values based on the legacy measurement windows. For a transient period/placement that exceeds the CP length, the transient period will be longer than the CP and a similar approach with the current LTE methodology has to be used (only DFT-s-OFDM can be used and some time domain samples have to be removed).

## Other testing considerations

From an overall testing procedure, this transient period can be tested at the slot boundary by measuring the EVMs on the last symbol of a slot and the first symbol of the subsequent slot. A PUSCH allocation (e.g. type A) with DMRS not allocated on the 1st symbol can be used. Since the channel is static, the DMRS on a different symbol can be used for the channel estimation/equalization process by the TE. This would also ensure that the DMRS is “clean” and is not influenced by the transient period.

So far the discussion has centered around using DFTs-OFDM for the test, however, our understanding is that CP-OFDM can also be used if the transient period is shorter than the CP. This is the case for all the transient period and SCS combinations but 7us with 30kHz SCS.

Furthermore, mini slots could also be used if they are supported by the UE. This could allow for further testing enhancements in the future.

# Conclusion

In this paper we further discussed the testing issues related to the newly introduce transient periods. We propose to define new EVM measurement windows that are designed specifically for each transient period. This will enable precise testing with minimum impact on the test equipment.

A text proposal for the requirements is provided in the Annex.

# References

- R4-2010914, “Draft CR on introduction of shorter Transient Period Capability”, Qualcomm et al.

- R4-2008477, “WF on feasibility of testing the transient periods”, CMCC

- R4-2010915, “Short Transient Period Testing”, Qualcomm Incorporated

- R4-2011859, “Email discussion summary for [96e] [119]_UE transient period”, Moderator(CMCC)

- R4-2016516, “On transient period UE capability”, Huawei, HiSilicon

- R4-1913932, “On-to-on transient period measurement in FR1”, Anritsu Corporation

- R4-1912461, “Test procedure for measuring the effect of transient period duration on EVM outside of transient for FR1”, Qualcomm Incorporated, Keysight

- R4-2014489, “Short Transient Period Testing”, Qualcomm Incorporated

# Annex- Proposed testing metric for the new capabilities

#### 6.4.2.1a	Error Vector Magnitude including symbols with transient period

In 6.4.2.1, EVM has been defined by excluding the symbols which have a transient period. In this section, measurement interval is defined for the symbols with a transient period to include these symbols in the RMS average EVM computation when the UE reports a transient period capability other than the default.

The test specifications are applicable to 15kHz and 30 kHz SCS, long subslot transmisisons with 14 symbols per slot and transient period specified in Figure 6.3.3.7-3.

In the case of PUSCH or PUCCH transmissions when the mean power, modulation or RB allocation across slot or subslot boundaries is expected to change the EVM result over the symbols where the transient occurs is calculated according to Table 6.4.2.1a-1

Table 6.4.2.1a-1: EVM definition for reported transient period

| Reported transient capability (us) | EVM definition | tpstart (µs) | SCS |
| --- | --- | --- | --- |
| 2 | EVM0=max⁡(EVMl_new,EVMh) EVMlast=max⁡(EVMl,EVMh_new) | -0.5 | 15kHz or 30kHz |
| 4 | EVM0=max⁡(EVMl_new,EVMh) EVMlast=max⁡(EVMl,EVMh_new) | -1 | 15kHz or 30kHz |
| 7 | EVM0=max⁡(EVMl_new,EVMh) EVMlast=max⁡(EVMl,EVMh_new) | -2 | 15kHz or 30kHz |
| NOTE 1: EVMl , EVMh, EVMl_new,and EVMh_neware defined in Annex F NOTE 2: EVM0 is the EVM for a symbol right after a transition; EVMlast is the EVM for a symbol right before a transition NOTE 3: tpstart denotes the start position of the transient window relative to the symbol boundary |  |  |  |

The RMS average of the basic EVM measurements over [70] subframes for the symbols where the transient occurs for the different modulation schemes shall not exceed the values  specified in Table 6.4.2.1a-2 for the parameters defined in Table 6.4.2.1a-3.  This requirement can be verified with 64 QAM and 256 QAM modulation.

Table 6.4.2.1a-2: Requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| 64 QAM | % | [15] |
| 256 QAM | % | [5] |

Table 6.4.2.1a-3: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | Table 6.3.1-1 |
| UE Output Power for 256 QAM | dBm | Table 6.3.1-1 + 10 dB |
| Operating conditions |  | Normal conditions |

# F.4	Modified signal under test

Implicit in the definition of EVM is an assumption that the receiver is able to compensate a number of transmitter impairments.

The DFT-s-OFDM modulated signals or PRACH signal under test is modified and, in the case of DFT-s-OFDM modulated signals, decoded according to:

Z'(t,f)=IDFTFFTz(v-Δt)⋅e-j2πΔfv.ej2πfΔta(t,f)⋅ejϕ(t,f)

where

z(v) is the time domain samples of the signal under test.

The CP-OFDM modulated signals or PUSCH demodulation reference signal or PUCCH data signal under test is equalised and, in the case of CP-OFDM modulated signals decoded according to:

Z'(t,f)=FFTz(v-Δt)⋅e-j2πΔfv.ej2πfΔta(t,f)⋅ejϕ(t,f)

where

z(v) is the time domain samples of the signal under test.

To minimize the error, the signal under test should be modified with respect to a set of parameters following the procedure explained below.

Notation:

Δt is the sample timing difference between the FFT processing window in relation to nominal timing of the ideal signal.

Δf is the RF frequency offset.

ϕ(t,f) is the phase response of the TX chain.

a(t,f) is the amplitude response of the TX chain.

In the following Δc represents the middle sample of the EVM window of length W (defined in the next clauses) or the last sample of the first window half if Wis even.

The EVM analyser shall

-	detect the start of each slot and estimate Δt and Δf,

-	determine Δc so that the EVM window of length W is centred

-	on the time interval determined by the measured cyclic prefix minus 16κ samples of the considered OFDM symbol for symbol l for subcarrier spacing configuration µ in a subframe, with l = 0 or l = 7*2^µ for normal CP, i.e. the first 16κ samples of the CP should not be taken into account for this step. In the determination of the number of excluded samples, a sampling rate of 1/Tc is assumed. If a different sampling rate is used, the number of excluded samples is scaled linearly.

-	on the measured cyclic prefix of the considered OFDM symbol symbol for all other symbols for normal CP and for symbol 0 to 11 for extended CP.

-	on the measured preamble cyclic prefix for the PRACH

To determine the other parameters a sample timing offset equal to Δc is corrected from the signal under test. The EVM analyser shall then

-	correct the RF frequency offset Δffor each time slot, and

-	apply an FFT of appropriate size. The chosen FFT size shall ensure that in the case of an ideal signal under test, there is no measured inter-subcarrier interference.

The carrier leakage shall be removed from the evaluated signal before calculating the EVM and the in-band emissions; however, the removed relative carrier leakage power also has to satisfy the applicable requirement.

At this stage the allocated RBs shall be separated from the non-allocated RBs. In the case of PUCCH and PUSCH EVM, the signal on the non-allocated RB(s), Yt,f, is used to evaluate the in-band emissions.

Moreover, the following procedure applies only to the signal on the allocated RB(s).

-	In the case of PUCCH and PUSCH, the UL EVM analyzer shall estimate the TX chain equalizer coefficients a(t,f) and ϕ(t,f) used by the ZF equalizer for all subcarriers by time averaging at each signal subcarrier of the amplitude and phase of the reference and data symbols. The time-averaging length is 1 slot. This process creates an average amplitude and phase for each signal subcarrier used by the ZF equalizer. The knowledge of data modulation symbols may be required in this step because the determination of symbols by demodulation is not reliable before signal equalization.

-	In the case of PRACH, the UL EVM analyzer shall estimate the TX chain coefficients a(t) and ϕ(t) used for phase and amplitude correction and are seleted so as to minimize the resulting EVM. The TX chain coefficients are not dependent on frequency, i.e. a(t,f)=a(t) and ϕ(t,f)=ϕ(t). The TX chain coefficient are chosen independently for each preamble transmission and for each Δt.

At this stage estimates of Δf, a(t,f), ϕ(t,f) and Δc are available. Δt is one of the extremities of the window W, i.e. Δtcan be Δc+α-W2 or Δc+W2, where α=0 if W is odd and α=1 if Wis even. The EVM analyser shall then

-	calculate EVMl with Δt set to Δc+α-W2,

-	calculate EVMh with Δt set to Δc+W2,

-	calculate EVMl_tp with Δt set totp2*Tc+1, where is 1/Tc the sampling rate

-	calculate EVMh_tp with Δt set toCP-tp2Tc-1, where 1/Tc is the sampling rate and the CP is the cyclic prefix of the symbol on which EVM is calculated(e.g. long CP for the first symbol of the slot) in seconds

A pictorial representation of the EVM windows is given in Figure F.4-1

Fig.F.4-1  EVM measurement window