3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2101626

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda item:	7.4.4

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM core requirements maintenance for IAB

Document for:	Discussion

- Introduction

The core requirements for IAB were completed in the RAN4#96e meeting [1], and issue related to the gap applicability was discussed in the last RAN4#97e with agreements captured in the WF [2]. We further provide our views on this issue in this paper.

- Discussion

In the last RAN4 meeting, the issue related to the gap applicability was discussed with the following agreements:

- FFS Whether to specify applicable MGs for local area IAB-MTs - Options: - Option 1: Remove gap aspects from requirements in 38.174 - Option 2: Define at least one gap pattern for local area IAB-MT - Other options not precluded

It could be observed from the core requirements for RLM and BFD/CBD that the scaling factor P is defined with the MGRP. However, the gap applicability has not been discussed before and it is not clear which MG pattern to use when calculating the delay requirements for RLM and BFD/CBD.

As there is no measurement requirements for IAB, it is not easy to define which gap pattern IAB shall be supported. As mentioned above, the only impact is on the scaling factor for RLM and BFD/CBD. Thus, it is proposed that remove the gap aspects from the requirements and clarify that the evaluation period would be longer when MG is configured. We also provide an accompanied CR to reflect the proposed changes.

Proposal 1: Remove the gap aspects from the requirements and clarify that the evaluation period could be longer when MG is configured.

- Conclusions

Proposal 1: Remove the gap aspects from the requirements and clarify that the evaluation period could be longer when MG is configured.

References

[1] R4-2012234 on RLM requirements for IAB-MT, ZTE Corporation

[2] R4-2017383 WF on test cases for IAB-MTs, ZTE Corporation