3GPP TSG-RAN WG4 Meeting #98e	R4-2101752

Electronic Meeting, Jan. 25-Feb. 5, 2021

Title:	Discussion on inter-band CA HPUE SAR

Source:	OPPO

Agenda item:	9.18.2

Document for:	Approval

- Introduction

WI [1] was approved, in which inter-band CA SAR issue is one of the topic. Initial discussions happen in RAN4#97e, however, no conclusion was reached.

This paper continue discuss this aspect.

- Discussion

In [2], it pointed out that for EN-DC combinations, E-UTRA is MCG and is prioritized so the duty cycle solution is reporting SCG’s duty cycle according to MCG’s duty cycle. And for inter-band 2UL CA, PCC/SCC configuration is UE specific rather than fixed prioritization. Therefore, a solution for inter-band UL CA is proposed in [2] is that each carrier report UL duty cycle capability according to the other carrier’s configuration.

Following the approach, UE reports a group of combined capabilities for Band X + Band Y, as below:

- UE reports (20%, Y1%), (40%, Y2%), (60%, Y3%), (80%, Y4%) - Here 20%, 40%, 60% and 80% are used as reference UL percentage in Band X, and Yn% is the reported value with Y1% >Y2% >Y3% >Y4% - When Band X is the PCC - The scheduled duty cycle for Band X is 30%, then it is between 20% and 40% - Then capability (40%, Y2%) is taken - Only when the scheduled duty cycle on Band Y is below Y2% then the PC2 is applied - When Band Y is the PCC - The scheduled duty cycle for Band Y is between Y1% and Y2% - Then capability (20%, Y1%) is taken. - Only when the scheduled duty cycle on Band X is below 20% then the PC2 is applied

From above example it can be seen that only four duty cycle capabilities need to be reported which is similar to FDD-TDD HPUE mechanism.

Observation 1:   Reporting of combined Band X +Band Y duty cycle capability is a possible way for inter-band UL CA HPUE SAR issues.

Proposal 1:        It is proposed to consider reporting a group of combined maxUplinkdutycycle capabilities for inter-band UL CA HPUE SAR issue.

- Conclusion

Observation 1:   Reporting of combined Band X +Band Y duty cycle capability is a possible way for inter-band UL CA HPUE SAR issues.

Proposal 1:        It is proposed to consider reporting a group of combined maxUplinkdutycycle capabilities for inter-band UL CA HPUE SAR issue.

Reference

[1] R4-2016851, WF on UE PC2 for NR inter-band CA and SUL configurations, China Telecom

[2] R4-2009796, Discussion on SAR solution for interband PC2 2UL CA, CATT