3GPP TSG-RAN WG4 Meeting #98-e	R4-2102220

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Source:	ZTE Corporation, CMCC

Title:	TP for TR38.xxx_Clarification on PC2 CA_n28A-n41A, CA_n28-n79A and CA_n40A-41A

Agenda Item:	9.19.2

Document for:	Approval

- Introduction

This contribution provides a text proposal to clarify the PC2 2DL/2UL CA_n28A-n41A, CA_n28-n79A and CA_n40A-41A in TR38.XXX[1].

According the analysis, there are no harmonic, intermodulation and/or cross band isolation issues for the above three PC2 NR CA combinations with dual uplink carrier comparing with the corresponding PC3 NR CA. Therefore, no additional MSD requirements are defined comparing with the corresponding PC3 NR CA.

- Reference

- TR38.xxx, High power User Equipment (UE) (power class 2) for NR inter-band Carrier Aggregation with 2 bands downlink and 2 bands uplink; (Release 17), v0.1.0

- Text Proposal

----- Start of TP -----

  - 5.2		CA_n28-n41

    - 5.2.1	Configurations

Table 5.2.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n41A | CA_n28A-n41A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

    - 5.2.2	Maximum output power

Table 5.2.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28A-n41A | CA power class | Carrier n28 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

    - 5.2.3	REFSENS requirements

According to the PC3 CA_n28A-n41A study, there are no harmonic, intermodulation and cross band isolation issues, therefore no MSD requirements are defined for PC3 CA_n28A-n41A. The same approach can be applied to PC2 CA_n28A-n41A

      - 5.2.3.1	Power class 2 case a

Comparing with PC3 CA_n28A-n41A, no additional MSD are expected for this PC2 CA_n28A-n41A with dual uplink carrier.

      - 5.2.3.2	Power class 2 case b

Comparing with PC3 CA_n28A-n41A, no additional MSD are expected for this PC2 CA_n28A-n41A with dual uplink carrier.

    - 5.2.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n28A-n41A requirements are applied for PC2 CA_n28A-n41A.

  - 5.3		CA_n28-n79

    - 5.3.1	Configurations

Table 5.3.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n28A-n79A | CA_n28A-n79A | n28 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

    - 5.3.2	Maximum output power

Table 5.3.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n28A-n79A | CA power class | Carrier n28 power class | Carrier n79 power class |
| --- | --- | --- | --- | --- |
| CA_n28A-n79A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

    - 5.3.3	REFSENS requirements

According to the PC3 CA_n28A-n79A study, there are no harmonic, intermodulation and cross band isolation issues, therefore no MSD requirements are defined for PC3 CA_n28A-n79A. The same approach can be applied to PC2 CA_n28A-n79A

      - 5.3.3.1	Power class 2 case a

Comparing with PC3 CA_n28A-n79A, no additional MSD are expected for this PC2 CA_n28A-n79A with dual uplink carrier.

      - 5.3.3.2	Power class 2 case b

Comparing with PC3 CA_n28A-n79A, no additional MSD are expected for this PC2 CA_n28A-n79A with dual uplink carrier.

    - 5.3.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n28A-n79A requirements are applied for PC2 CA_n28A-n79A.

  - 5.4		CA_n40-n41

    - 5.4.1	Configurations

Table 5.4.1-1: NR CA configurations and bandwidth combinations sets for supporting power class 2

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n40A-n41A | CA_n40A-n41A | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes |  |

    - 5.4.2	Maximum output power

Table 5.4.2-1 UE Power Class 2 for uplink inter-band CA (two bands)

| Uplink CA configuration | Power class 2 cases for CA_n40A-n41A | CA power class | Carrier n40 power class | Carrier n41 power class |
| --- | --- | --- | --- | --- |
| CA_n40A-n41A | Case a | 26dBm | 23dBm | 23dBm |
|  | Case b | 26dBm | 23dBm | 26dBm |
|  | Case c | 26dBm | 26dBm | 23dBm |
|  | Case d | 26dBm | 26dBm | 26dBm |

The tolerance +2/-3 dB is applied. Also when the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB.

    - 5.4.3	REFSENS requirements

According to the PC3 CA_n40A-n41A, band n40 and band n41 are sychronous operation. Therefore, no MSD requirements are defined for PC3 CA_n40A-n41A. The same approach can be applied to PC2 CA_n40A-n41A

      - 5.4.3.1	Power class 2 case a

Comparing with PC3 CA_n40A-n41A, no additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

      - 5.4.3.2	Power class 2 case b

Comparing with PC3 CA_n40A-n41A, no additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

      - 5.4.3.3	Power class 2 case c

Comparing with PC3 CA_n40A-n41A, no additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

      - 5.4.3.4	Power class 2 case d

Comparing with PC3 CA_n40A-n41A, no additional MSD are expected for this PC2 CA_n40A-n41A with dual uplink carrier.

    - 5.4.4		∆TIB and ∆RIB values

For the ∆TIB,c and ∆RIB,c values, same PC3 CA_n40A-n41A requirements are applied for PC2 CA_n40A-n41A.

----- End of TP -----