3GPP TSG-RAN Meeting #98-e	R4-2102300

# Online Meeting,  Jan 25 – Feb 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1708 | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revision of PRS-RSRP measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-01-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PRS-RSRP measurement requirements need to be corrected and completed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Apply the measurement period definition to both “overlapping” and “non-overlapping” cases. - Update the definition of the effective period for a positioning frequency layer containing multiple PRS periodicities. - Fix the definition of the starting point of the measurement period. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PRS-RSRP measurement requirements would be incomplete and/or incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Start of change >

#### 9.9.3.5	Measurement Period Requirements

When the physical layer receives NR-DL-AoD-ProvideAssistanceData message and NR-DL-AoD-RequestLocationInformation message from LMF via LPP [34], the UE shall be able to measure multiple (up to the UE capability specified in Clause 9.9.3.3) PRS-RSRP measurements, defined in TS 38.215 [4], from configured PRS resources for configured TRPs on configured PRS frequency layers, within TPRS-RSRP,total ms.

The measurement period for PRS-RSRP measurements, TPRS-RSRP, total, is defined as:

TPRS-RSRP, total=i=1LTPRS-RSRP,i+L-1*maxTeffect,i

where i is the index of positioning frequency layer,

TPRS-RSRP,i is the measurement period for PRS-RSRP measurements in frequency layer i as defined further in this clause,

L is total number of positioning frequency layers, and Teffect,i is the periodicity of PRS-RSRP measurement in frequency layer i as defined further in this clause.

TPRS-RSRP,i=CSSFi*NRxBeam,i*NPRS,islotN'LPRS,iN*Nsample-1*Teffect,i+Tlast

where

CSSFi is the carrier specific scaling factor for PRS-RSRP measurements specified in clause 9.1.5.2,NRxBeam,i is CSSFi is the carrier specific scaling factor for NR PRS-based measurements specified in clause 9.1.5.2 as CSSFwithin_gap,i,

NRxBeam,i is the scaling factor for Rx beam sweeping, and NRxBeam,i=1 if positioning frequency layer i is in FR1 and NRxBeam,i=8 if positioning frequency layer i is in FR2,

LPRS,i is the time duration specified in clause 5.1.6.5 of TS 38.214 [26, clause 5.1.6.5],

NPRS,islot is the maximum number of DL PRS resources of frequency layer i configured in a slot,

{N,T} is UE capability combination per band where N is a duration of DL PRS symbols in ms corresponding to durationOfPRS-ProcessingSysmbols in TS 37.355 [34] processed every T ms corresponding to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34] for a given maximum bandwidth supported by UE corresponding to supportedBandwidthPRS in TS 37.355 [34],

N’ is UE capability for number of DL PRS resources that it can process in a slot as indicated by maxNumOfDL-PRS-ResProcessedPerSlot  in clause 6.4.3 of TS 37.355 [34],

Nsample is the number of PRS-RSRP measurement samples and Nsample= 4,

Tlast is the measurement duration for the last PRS-RSRP sample, including the sampling time and processing time, Tlast = Ti + LPRS,i

Teffect,i is periodicity of PRS-RSRP measurement in frequency layer i:

Teffect,i= TiTavailable_PRS,i*Tavailable_PRS,i

where

Ti	corresponds to durationOfPRS-ProcessingSymbolsInEveryTms in TS 37.355 [34],

Tavailable_PRS,i= LCMTPRS,i,MGRPi, the least common multiple between TPRS,i and MGRPi

TPRS,i is the least common multiple of the periodicities of all the DL PRS resources in frequency layer i,

MGRPi is the measurement gap repetition period configured for measurement of positioning frequency layer i.

When PRS-RSRP measurements are configured for DL-AoD, the time TPRS-RSRP,iTPRS-RSRP,total starts from the first MG instance aligned with DL PRS resources in the assistance data after both the NR-DL-AoD-RequestLocationInformation message and NR-DL-AoD-ProvideAssistanceData message from LMF via LPP [34] are delivered to the physical layer of UE.

If handover occurs while PRS-RSRP measurements are being performed then the UE shall complete the ongoing PRS-RSRP measurements session. The UE shall also meet the PRS-RSRP measurement requirements in this clause and measurement accuracy requirements in clause 10.1.24. However in this case the PRS-RSRP measurement period TPRS-RSRP,total.HO shall be as follows:

TPRS-RSRP, total,HO=TPRS-RSRP, total+K*Teffect+THO   ms

where

K is the number of times handover occurs during TPRS-RSRP,total.HO;

Teffect is the largest Teffect,i among all positioning frequency layers;

THO is the time during which the PRS-RSRP measurement may not be possible due to handover; it can be up to Tinterrupt as defined in clause 6.1.

When the PRS-RSRP measurement is configured together with UE Rx-Tx time difference measurement, the UE behaviour at a serving cell change for the PRS-RSRP measurement is the same as the UE behaviour for the UE Rx-Tx time difference measurement specified in clause 9.9.4.5, and the PRS-RSRP measurement shall meet the accuracy requirements in clause 10.1.24.

When the PRS-RSRP measurement is configured together with RSTD measurement, the UE behaviour at a serving cell change for the PRS-RSRP measurement is the same as the UE behaviour for the RSTD measurement specified in clause 9.9.2.5, and the PRS-RSRP measurement shall meet the accuracy requirements in clause 10.1.24.

# < End of change >