3GPP TSG-RAN WG4 Meeting #98-e                                          R4-2102383

Electronic Meeting, 25th January– 5th Febuary, 2021

Source: 	Huawei, Hisilicon

Title: 	On Tx diversity requirements

Agenda Item:	7.19.2

Document for:	Approval

# Introduction

TxD requirements were further discussed in last RAN4 meeting, but the progress was not as good as expected [1]. There are three aspects for the TxD remaining issues:

- the TxD related requirements to be specified

- measurement of the TxD related requirements or UE with multiple transmitter antenna connectors

- applicability of the TxD related requirements

For the requirements part, some general principles were captured in the previously agreed WFs, we see the requirements are converging. It’s time to focus on the requirements CR.

As for the measurement part, some of them can be decided by RAN4, but some could be left to RAN5, and not all of them need to be addressed before the core requirements are completed, so we think it has less priority than the core requirements. And regarding the applicability of the requirements, it depends on how to identify the requirements are specific to 2Tx.

This contribution will further analyse above there aspects, and our proposals are provided.

# Discussion

## TxD related requirements

Enabling transparent Tx diversity (TxD) was agreed at least from Rel-16 was already agreed in RAN4. Essentially, there are no big difference for the 2Tx requirements between TxD and UL MIMO. Requirements for ULFPTx and PC1.5 were agreed in [2][3], in which TxD as an UE implementation are considered.

As a reference, the requirements for ULFPTx can be used for specifying the TxD requirements in a similar way. It is noted that not all requirements for ULFPTx are defined on both Tx antenna connectors. For TxD, corresponding requirements should be aligned with ULFPTx.

Observation 1: Requirements for ULFPTx can be used as a reference to define the TxD related requirements.

With the principles of reusing the similar requirement as ULFPTx in mind, we see progress for the TxD relevant requirements in the WF [1].

For the power and emissions related requirements, the criteria is that “requirements are defined as the sum of powers from both connectors”. Apart from that, MPR will be defined for dual Tx. As a precedent example, PC1.5 is a handy reference. Besides MPR, the power and emissions related requirements which require summing of the measurement at each antenna connector, EVM is the main leftover requirement. Two different definitions for EVM are proposed, and it will be further discussed in the following section.

Observation 2: Main TxD requirements are power/emissions related ones as well as MPR and EVM.

### Requirements need to be verified with multiple antennas

### MPR

For both TxD and UL MIMO, it was already agreed that unwanted emissions should be specified per UE level rather than per antenna connector. According to the proposed changes for SE and SEM, the requirements can alternatively be tightened by 3dB at each antenna connectors instead. Thus for the 2x23dBm 2Tx implementation, no matter TxD or UL MIMO, the PC2 MPR requirements shall be revisited accordingly.

Based on our evaluation [4], MPR for both DFT-s-OFDM and CP-OFDM are obviously increased especially for the edge RB allocation. The proposed MPR values for power class 2 UE supporting 2Tx transmission are listed in Table 1.

Table 1: Maximum power reduction (MPR) for power class 2 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 4 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 4 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 4 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 4 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 4 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 4 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

As analyzed in [5], since inner allocations are EVM or IBE limited rather than SE and SEM limited, it proposed that PC2 single port inner allocation MPR level can be reused for PC2 power class inner allocation MPR in Tx Diversity and UL MIMO modes, while MPR for outer and edge allocation for PC2 UL MIMO or Tx Diversity implemented with two 23dBm PA is FFS. At least for the inner RB allocations, we have the same observation based on our evaluation.

In our view, there is no need to change the PC3 MPR requirement for TxD or UL MIMO, and it is also noticed that the same PC3 MPR requirement is applicable for 2Tx as well.

Observation 3: According to ULFPTx, there is no difference of PC3 MPR requirement for 1Tx or 2Tx

### EVM

In the agreed WF [1], it was agreed that EVM definition for transparent TxD should be captured in the Annex, which means, the core requirements measured at each antenna connector are not changed, what matters for UE with multiple antenna connectors is how to deal with the measured data in the test procedure.

There are two different proposals for TxD EVM, one was agreed in RAN4#95-e, i.e.

Option 1: EVM=(P1*EVM12+ P2*EVM22)/(P1 + P2)

Which means that EVM should be measured at each antenna connector to get EVM1 and EVM2, then the total EVM for TxD is calculated using RSS method but considering weighting factor of power at each antenna connector.

The other one was discussed in last meeting, which includes two options, i.e.

    - Option 2a.

      - EVMport=100∙ 11HΣ'-111-12where Σ'=n' n'H .

    - Option 2b.

EVMport=minEVM1,EVM2

Where EVM1 and EVM2 are the EVM values for the first and second antenna connectors.

Our preference is option 1, since the TxD behavior may not be kept during the test, which actually reflects the real scenario of UE implementation. Once the output power is lowered down, it is not necessary to maintain 2Tx always, which is natural even from power saving perspective. On the other hand, unlike UL MIMO, the power distributed at two antenna connectors could be different. As an extreme example, the power could be transmitted at only one antenna connector. Thus the total EVM calculated with power weighting factor is proper to be defined for TxD. During the discussion, some companies also favored option 2b. To make progress, we are also open to this option.

### Summary

Table 2 below lists the summary of all affected requirement for transparent TxD and proposed changes in the specification. Besides MPR and EVM, all other requirements can be defined in a similar way as ULFPTx. For ACLR, some changes are needed according to the WF for TxD.

Table 2: Summary of affected requirement for transparent TxD and proposed changes

| Clause | Requirement | Proposed clarifications or changes |
| --- | --- | --- |
| 6.2.1 | UE maximum output power | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.2.2 | MPR | Align with ULFPTx, the PC3 MPR is used for both 1Tx and 2Tx. Similar to PC1.5, add new MPR for PC2 with 2Tx. |
| 6.2.4 | Configured transmitted power | Similar to ULFPTx, add Pcmax,c tolerance for 2Tx |
| 6.3.1 | Minimum output power | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.3.2 | Transmit OFF power | Align with ULFPTx, clarify the requirement is defined at each antenna connector. |
| 6.3.3 6.3.3.1 | Transmit ON/OFF time mask | Align with ULFPTx, clarify the requirement is defined at each antenna connector. |
| 6.3.4 6.3.4.1 | Power control | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.4 | Transmit signal quality | Align with ULFPTx, clarify the requirement is defined at each antenna connector. |
| 6.4.2.1 | EVM | According to the WF in R4-2011768, the measurement procedure is defined in Annex F. |
| 6.5.2 | Out of band emission | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.5.2.4 | ACLR | Make changes according to WF in R4-2008465. |
| 6.5.3 | Spurious emissions | Align with ULFPTx, clarify the requirement is defined as the sum of emissions at each transmit antenna connector. |
| 6.5.4 | Transmit intermodulation | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |

## TxD test related issues

One part of the TxD WF [1] is related to measurement of the requirements and some progress were made compared to RAN4#96e.

For “Declaration for default Tx connector”, it was agreed that:

- UE declares which connectors will be active per band under test. TE needs to detect ACK and NACK and any other expected response from UE from all declared TX antenna connectors.

  - The word “active” can be replaced by “used for TxD during one test procedure”. (Not necessarily to have transmission all the time.)

  - UE declaration needs to describe exact two antenna connectors under test.

For other remaining test related issues, we have the following consideration.

### UE behaviour under conformance testing

Regarding the UE behaviour under conformance testing, there options are listed.

- Option 1a: UE will keep the Tx diversity status unchanged in conformance testing.

- Option 1b: Test mode signalling is implemented to instruct UE to keep Tx div status unchanged

- Option 2: TE will detect and sum for every power step and change in condition from all connector

Firstly, we disagree that a specific test mode should be developed just for the TxD related test, which cause unnecessary burden for UE implementation. And we don’t think that Tx diversity status should be maintained all the time during the test. Once the output power lowered down to a certain level, there is no need to transmit at two antenna connectors, which would be a natural implementation choice for UE vendors. Our preference is option 2.

It is proposed not to keep TxD status unchanged all the time during the test and no test mode is needed.

### Power splitting behaviour

There are two options for power splitting behaviour:

- Option 1: Only allow equal power split between connectors

- Option 1a: Per instructed as test mode, UE should keep equal power split between connectors in all cases.

- Option 2: Allow any power split between connectors

For simplicity of measurement, our view is that the power should be split equally between connectors, however, the power does not have to be equally split in real application, which is up to UE implementation and we think such flexibility left to UE is necessary.

It is proposed to split the power equally between connectors for some requirements, e.g. MOP, SE, SEM during the test but no need to limit the UE behavior like that in real application. Detailed test cases can be determined by RAN5.

### Summary

Table 3 below lists the summary of test related issues and proposals for transparent TxD test.

Table 3: Summary of test related issues for TxD

| Items | Proposed measurement procedure or UE behavior |
| --- | --- |
| UE behavior under conformance testing | No need to keep TxD status unchanged all the time during the test and test mode is not necessary. |
| Power splitting behavior | Split the power equally between connectors during the test for some requirements but no need to limit the UE behavior like that in real application. |

## Applicability of the TxD related requirements

The left issue in the WF is applicability of the TxD related requirements.

ULFPTx is a new feature introduced in Rel-16. For a UE supporting ULFPTx, e.g. mode 1, essentially the UE is implemented by TxD, in that sense, the TxD can be explicitly indicated via ULFPTx in Rel-16. However, if a UE does not support Rel-16 ULFPTx, it’s not clear whether UE supports transparent TxD. Thus there are some options in last meeting on whether and how RAN4 introduce signalling for transparent TxD:

- Option 1: Introduce some sort of signaling by UE

  - Option 1a. Use ModifiedMPRbehavior bits to signal additional relaxations;

  - Option 1b: Introducing a new (capability) signalling for TxD

  - Option 1c: Introducing a new power class (e.g. PC2.5) for TxD

- Option 2: Based on UE vendor declaration.

- Option 3: Using existing signalling to indicate the 2Tx implementation capability.

As confirmed in the RAN1 LS [9], transparent TxD can be supported by both single SRS port and two SRS ports. No matter whether the UE supporting code-book based UL MIMO or ULFPTx, the maximum SRS ports are two ports, which can be indicated by IE maxNumberSRS-Ports-PerResource. However, since TxD is not specified in RAN1 spec, unlike UL MIMO, which can be configured to a certain mode based on UE reported capability, e.g. specific code book or ULFPTx transmission mode, there is no way to configure the UE to operate in TxD mode, even the UE can indicate that it has 2Tx capability via the maxNumberSRS-Ports capability. Therefore, a new capability to indicate the UE can support TxD is not helpful to solve the issue, because the network cannot configure the UE to operate under TxD mode. If the spec is enabled to configure the TxD mode, lots of spec impact is envisioned for both RAN1 and RAN2, which is not the purpose of the discussion on transparent of TxD.

As specified by RAN5, even for a UE supporting UL MIMO, the UE shall pass the test with txConfig absent, which means the UE transmits PUSCH on one antenna port. For a UE indicating with 2Tx, the possible UE implementation could be 23+23dBm or 26+23dBm, which can pass the MOP test via TxD for 23+23dBm implementation or 1Tx 26dBm. Of course, for the 26+23dBm UE, it can also use TxD to fulfil the MOP requirement, which is up to UE implementation. In other words, even a 2Tx UE has the capability of TxD, it cannot make sure that the UE always operates under TxD mode, which depends on UE specific implementation. Generally, the RF requirements are implementation agnostic, thus the requirements actually should be defined based on 2Tx UE capability rather than for TxD operation mode.

Observation 4: The so called transparent TxD requirements should be defined for UE supporting 2Tx rather than for UE operateing in TxD mode.

With this understanding, it is clear that the transparent TxD requirements can also be applicable to UE implementation without transparent TxD as long as the UE indicates 2Tx capability. Consequently, the requirements for 2Tx should be specified in a generic way rather than the requirements dedicated to TxD.

# Conclusion

Detailed analysis for all possibly affected requirements as well as the test related issues are provided in this contribution.

Observation 1: Requirements for ULFPTx can be used as a reference to define the TxD related requirements.

Observation 2: Main TxD requirements are power/emissions related ones as well as MPR and EVM.

Observation 3: According to ULFPTx, there is no difference of PC3 MPR requirement for 1Tx or 2Tx

Observation 4: The so called transparent TxD requirements should be defined for UE supporting 2Tx rather than for UE operateing in TxD mode.

Since the transparent TxD requirements would be the same for both Rel-15 and Rel-16, though the analysis of the contribution is under TEI16 agenda, after reviewing all the TxD related requirements, we can see that the affected requirements are almost aligned with the changes for eMIMO ULFPTx. The work load is not as large as expected.

Once the TxD requirements for Rel-16 is finished, RAN4 can consider the release independent issue and make a decision for Rel-15.

Proposal 1: It is proposed to focus on the affected requirements and corresponding spec changes list in the table below and close the TEI topic ASAP:

| Clause | Requirement | Proposed clarifications or changes |
| --- | --- | --- |
| 6.2.1 | UE maximum output power | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.2.2 | MPR | Align with ULFPTx, the PC3 MPR is used for both 1Tx and 2Tx. Similar to PC1.5, add new MPR for PC2 with 2Tx. |
| 6.2.4 | Configured transmitted power | Similar to ULFPTx, add Pcmax,c tolerance for 2Tx |
| 6.3.1 | Minimum output power | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.3.2 | Transmit OFF power | Align with ULFPTx, clarify the requirement is defined at each antenna connector. |
| 6.3.3 6.3.3.1 | Transmit ON/OFF time mask | Align with ULFPTx, clarify the requirement is defined at each antenna connector. |
| 6.3.4 6.3.4.1 | Power control | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.4 | Transmit signal quality | Align with ULFPTx, clarify the requirement is defined at each antenna connector. |
| 6.4.2.1 | EVM | According to the WF in R4-2011768, the measurement procedure is defined in Annex F. |
| 6.5.2 | Out of band emission | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |
| 6.5.2.4 | ACLR | Make changes according to WF in R4-2008465. |
| 6.5.3 | Spurious emissions | Align with ULFPTx, clarify the requirement is defined as the sum of emissions at each transmit antenna connector. |
| 6.5.4 | Transmit intermodulation | Align with ULFPTx, clarify the requirement is defined as the sum of output power at each transmit antenna connector. |

Proposal 2: It is proposed to make decision on the test related issues list in the table below:

| Items | Proposed measurement procedure or UE behavior |
| --- | --- |
| UE behavior under conformance testing | No need to keep TxD status unchanged all the time during the test and test mode is not necessary. |
| Power splitting behavior | Split the power equally between connectors during the test but no need to limit the UE behavior like that in real application. |

A companion CR is provided in [9].

# References

[1] R4-2016830, Way Forward on NR TxD & Power Class, vivo

[2] R4-2011762, CR to TS38.101-1 on introduction of Uplink Full Power Transmission, Samsung, Qualcomm

[3] R4-2011783, CR for 38.101-1: Introduction of Power Class 1.5, T-Mobile USA

[4] R4-2008213, On Rel-15 PC2 UL MIMO unwanted emissions and MPR requirement, Huawei, HiSilicon

[5] R4-2006668, RIMD impact on EVM and MPR for UL MIMO and Tx Diversity, Skyworks

[6] R4-2011519, Further Considerations on the EVM Definition for Antenna Ports Including Transparent Transmit Diversity, Lenovo, Motorola Mobility

[7] R4-2008465, WF on Enabling Transparent TxD in Rel-16, Qualcomm

[8] R1-2007245, Reply LS on feasibility of UL FPTx modes and transparent TxD for certain UE implementation, RAN1

[9] R4-2102384, CR for TS 38.101-1 Tx diversity requirements, Huawei, HiSilicon