3GPP TSG-RAN WG4 Meeting#98-e	R4- 2102406

Online, 25th Jan – 5th Feb, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on FR2 256QAM CQI Reporting Test Cases

Agenda item:	7.10.1.2

Document for:	Discussion

- Introduction

In the last meeting, WF [1] listed simulation assumptions for FR2 256QAM CQI reporting test cases. In this paper, we provide our simulation results and propose test SNR for defining requirements.

- Simulation Results

Based on simulation assumptions in WF [1], below are our simulation results for CQI reporting under TDLA30-35 ULA High, Rank1.

Table 1: Simulation results for CQI reporting under TDLA30-35 ULA High, Rank1, 50MHz CBW, 120kHz SCS

| SNR (dB) | Median CQI (256QAM) | Percentage not in Median CQI+/-1 | Percentage 256QAM CQI > 11 | Following 256QAM CQI BLER | Following 256QAM CQI Thpt | Following 64QAM CQI Thpt |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 3 | 28.50 | 0.00 | 0.08 | 15.9 | 15.4 |
| 4 | 4 | 22.29 | 0.00 | 0.08 | 18.4 | 19.1 |
| 5 | 4 | 22.41 | 0.00 | 0.10 | 19 | 18.6 |
| 6 | 5 | 38.25 | 0.00 | 0.16 | 20.9 | 21 |
| 7 | 6 | 44.70 | 0.00 | 0.16 | 23.5 | 23.6 |
| 8 | 6 | 41.78 | 0.00 | 0.08 | 30.1 | 30.4 |
| 14 | 9 | 32.03 | 4.63 | 0.07 | 49.5 | 49.6 |
| 15 | 9 | 29.35 | 9.01 | 0.07 | 52.4 | 52.3 |
| 16 | 10 | 35.08 | 14.74 | 0.07 | 55.8 | 54.8 |
| 17 | 10 | 33.62 | 22.17 | 0.08 | 58.7 | 57.2 |
| 18 | 10 | 36.91 | 29.84 | 0.08 | 61.5 | 58.7 |
| 19 | 11 | 34.59 | 39.22 | 0.07 | 65.3 | 60.2 |
| 20 | 11 | 34.84 | 49.94 | 0.07 | 68.1 | 61.3 |
| 21 | 12 | 39.46 | 60.78 | 0.08 | 70.2 | 62.5 |
| 22 | 12 | 36.05 | 70.77 | 0.08 | 72.8 | 62.9 |
| 23 | 13 | 33.37 | 78.44 | 0.08 | 75.3 | 63.3 |
| 24 | 13 | 30.09 | 85.63 | 0.09 | 77.3 | 63 |

Based on above results, we see that following CQI throughput with 256QAM CQI table has significant gain compared to following 64QAM CQI table starting at 19dB SNR. Adding 2dB for impairments, RAN4 should define the 256QAM CQI reporting requirements at least for 21dB SNR. Therefore, we propose the following.

Proposal 1: Define FR2 256QAM CQI reporting test for at least 21dB SNR to see the impact of 256QAM CQI table.

- Conclusions

This paper provides simulation result for FR2 256QAM CQI reporting test case. Following has been proposed.

Proposal 1: Define FR2 256QAM CQI reporting test for at least 21dB SNR to see the impact of 256QAM CQI table.

# References

[1] R4-2017536, “WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM”, 3GPP TSG RAN WG4 Meeting #97-e, Online, Nov 2 - Nov 13, 2020.