3GPP TSG RAN WG4 Meeting #98-e	R4-2102640

Electronic Meeting, Jan 25-Feb 05, 2021

Agenda Item:	7.4.5.1

Source: 	Ericsson

Title:	General principles for IAB RRM test cases

Document for:	Discussion

- Introduction

In the last RAN4 meeting a WF on the RRM tests for IAB was approved [1]. It outlines high level general principles for defining RRM test cases. A test case list was also presented but no consensus was reached as RAN4 focused on general aspects of testing.

In this contribution we discuss testing aspects and general princples related to IAB-MT RRM tests.

# Test configuration parameters in RRM tests

One general principle agreed in the WF was [1]:

- Take UE test cases in TS 38.133 Annex as baseline when defining test cases for IAB-MTs.

This means the IAB-MT test cases should reuse the parameters used in the UE RRM tests or as much as possible if the exact value is not possible.

We reiterate support for this general principle. However, since IAB-MT is a network node therefore the RRM tests should be applicable to any set of configurations supported by IAB. By configurations we refer to TDD configurations, PRACH configurations, SMTC configurations, SSB configuration, RLM-RS configuration, SRS configuration, channel bandwidth, SCS etc. In the test cases in the annex, test requirements can be derived for specific set of configurations used in UE tests. However, the actual test, can be performed any set of configurations which may be different than the one used in the text in the annex of TS 38.174. This can be decided by the manufacturer. This is general principle for BS conformance tests e.g. test can be done for any TDD configuration.

- Proposal 1: In IAB-MT RRM test requirements are derived using the corresponding configuration parameters as example.

- Proposal 2: The actual IAB-MT RRM test can be conducted by any set of configuration parameters and corresponding test requirements shall be based on the actual configuration parameters used in the test.

# Scenario for IAB-MT RRM test cases

Some of the UE RRM test cases are defined with multiple serving cells. For IAB-MT tests, we propose in all tests there is only one serving cell. However, there can be more than one cell in some tests to account for a target cell e.g. RRC re-establishment and RRC release with redirection.

Tests will obviously be conducted within the same FR i.e. all cells in FR1 or in FR2.

- Proposal 3: In IAB-MT RRM tests only one serving cell shall be considered.

- Summary

In this paper we have analyzed principles of developing RRM tests to verify the IAB-MT RRM requirements defined in TS 38.174. Following are the main proposals:

- Proposal 1: In IAB-MT RRM test requirements are derived using the corresponding configuration parameters as example.

- Proposal 2: The actual IAB-MT RRM test can be conducted by any set of configuration parameters and corresponding test requirements shall be based on the actual configuration parameters used in the test.

- Proposal 3: In IAB-MT RRM tests only one serving cell shall be considered.

- References

- R4-2017383	WF on test cases for IAB-MTs, ZTE