3GPP TSG-RAN WG4 Meeting #98-e                     	  R4-2102694

# Electronic Meeting, Jan. 25 - Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7052 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to applicability of E-UTRAN E-CID measurements requirements for NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15 |  |  |  |  |  | Date: |  |  | 15/01/2021 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Applicability for E-UTRAN E-CID measurement requirements is corrected, as current references - for PSCell and SCell carrier frequencies point to E-UTRAN FDD intra-frequency requirements - for non-serving E-UTRA carrier frequencies point to E-UTRAN TDD intra-frequency requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | In subclause 3.3 abbreviations for terms PCC and SCC are added, since not contained in TS 21.905. References for E-UTRAN E-CID measurement requirements are corrected in subclause 3.6.3 to refer to - subclause 8.19.5 (Intra-frequency E-UTRAN E-CID measurements in NE-DC operation) applicable to PSCell and SCell carrier frequencies - subclause 8.19.3 (E-UTRAN inter frequency measurements in NE-DC operation) applicable to non-serving E-UTRA carrier frequencies |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong references for NE-DC operation are kept in spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [26] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [26].

1x RTT	CDMA2000 1x Radio Transmission Technology

ABS	Almost Blank Subframe

ARQ	Automatic Repeat Request

AP	Access Point

AWGN	Additive White Gaussian Noise

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

CA	Carrier Aggregation

CC	Component Carrier

CCA	Clear Channel Assessment

CCCH SDU	Common Control Channel SDU

CE	Coverage Enhancement

CGI	Cell Global Identifier

CPICH	Common Pilot Channel

CPICH Ec/No	CPICH Received energy per chip divided by the power density in the band

CRS	Cell-specific Reference Signals

C-RNTI	Cell RNTI

CSI	Channel-State Information

CSI-RS	CSI Reference Signal

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DL	Downlink

DMTC	Discovery signal Measurement Timing Configuration

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DUT	Device Under Test

E-CID	Enhanced Cell-ID (positioning method)

ECGI	Evolved CGI

eDRX_IDLE	Extended IDLE-mode DRX

eDRX_CONN	Extended CONNECTED-mode DRX

eNB	E-UTRAN NodeB

EN-DC	E-UTRA-NR Dual Connectivity

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

FDD	Frequency Division Duplex

FS3	Frame Structure type 3

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigational Satellite System

GSM	Global System for Mobile communication

HARQ	Hybrid Automatic Repeat Request

HD-FDD	Half-Duplex FDD

HO	Handover

HRPD	High Rate Packet Data

IDC	In-Device Coexistence

IEEE	Institute of Electrical and Electronics Engineers

LBT	Listen before talk

LPP	LTE Positioning Protocol

LWA	LTE-WLAN Aggregation

MAC 	Medium Access Control

MCG	Master Cell Group

MeNB	Master eNB

MBMS	Multimedia Broadcast Multicast Service

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MBSFN ABS	MBSFN Almost Blank Subframe

MDT	Minimization of Drive Tests

MGRP	Measurement Gap Repetition Period

MIB	Master Information Block

MPDCCH	MTC Physical Downlink Control Channel

NE-DC	NR-E-UTRA Dual Connectivity

NG-RAN	NG  Radio Access Network

NGEN-DC	NG-RAN E-UTRA-NR Dual Connectivity

NR	New Radio

NSA	Non-standalone

NPBCH	Narrowband Physical Broadcast CHannel

NPDCCH	Narrowband Physical Downlink Control CHannel

NPDSCH	Narrowband Physical Downlink Shared CHannel

NPRACH	Narrowband Physical Random Access CHannel

NPUSCH	Narrowband Physical Uplink Shared CHannel

NPSS	Narrowband Primary Synchronization Signal

NRS	Narrowband Refernce Signal

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NSCH	Narrowband Synchronization Channel

NSSS 	Narrowband Secondary Synchronization Signal

OCNG	OFDMA Channel Noise Generator

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OTDOA	Observed Time Difference of Arrival

PBCH	Physical Broadcast Channel

PCC	Primary Component Carrier

P-CCPCH	Primary Common Control Physical Channel

PCell	Primary Cell

PCFICH	Physical Control Format Indicator CHannel

PDCCH	Physical Downlink Control CHannel

PDSCH	Physical Downlink Shared CHannel

PHICH	Physical Hybrid-ARQ Indicator CHannel

PLMN	Public Land Mobile Network

PMCH	Physical Multicast Channel

PRACH	Physical Random Access CHannel

ProSe	Proximity-based Services

PRS	Positioning Reference Signal

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control Channel

PSCell	Primary SCell

PSS	Primary Synchronization SignalPSSCH	Physical Sidelink Shared CHannel

psTAG	Primary Secondary Timing Advance Group

pTAG	Primary Timing Advance Group

PTW	Paging Time Window

PUCCH	Physical Uplink Control CHannel

PUSCH	Physical Uplink Shared Channel

RS-SINR	Reference Signal Signal to Noise and Interference RatioRSCP	Received Signal Code Power

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

QAM	Quadrature Amplitude Modulation

RACH	Random Access Channel

RAT	Radio Access Technology

RNC	Radio Network Controller

RNTI	Radio Network Temporary Identifier

RRC	Radio Resource Control

RRM	Radio Resource Management

RSS	Resynchronization Signal

SCC	Secondary Component Carrier

SCE	Small Cell Enhancement

SCH	Synchronization Channel

SCell	Secondary Cell

SCG	Secondary Cell GroupSDU	Service Data Unit

SCS	Subcarrier spacing

SeNB	Secondary eNB

SFN	System Frame Number

SI	System Information

SIB	System Information Block

SLSS	SideLink Synchronization Sequence

SON	Self Optimized Network

SPDCCH	Short Physical Downlink Control channel

SPUCCH	Short Physical Uplink Control channel

SRS	Sounding Reference Signal

SS-RSRP	Synchronization Signal based Reference Signal Received Power

SS-RSRQ	Synchronization Signal based Reference Signal Received Quality

SS-SINR	Synchronization Signal based Signal to Noise and Interference Ratio

SSB	Synchronization Signal Block

SSS	Secondary Synchronization Signal

SSTD	SFN and subframe time difference

sTAG	Secondary Timing Advance Group

TAG	Timing Advance Group

TDD	Time Division Duplex

TP	Transmission Point

TTI	Transmission Time Interval

UE	User Equipment

UL	Uplink

UMTS	Universal Mobile Telecommunication System

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

V2V	Vehicle to Vehicle

V2X	Vehicle to Everything

WLAN	Wireless Local Area Network

WB-RSRQ	Wide Bandwith RSRQ

End of Change 1

Start of Change 2

### 3.6.3	Applicability of requirements for NE-DC operation

Requirements for NE-DC operation are applicable for the UE which has been configured with the following number of E-UTRA CCs and NR CCs:

-	up to 5 E-UTRA DL CCs in total with 1 E-UTRA UL CC in SCG. The applicable number of NR CC for EN-DC in the SCG is specified in clause 3.6.2 of TS 38.133 [50].

In addition to the requirements explicitly defined for a UE configured with NE-DC the following requirements shall also apply for the UE configured with NE-DC:

-	Random access requirements in section 6.2 for random access procedures on PSCell and activated SCell(s),

-	UE transmit timing requirements in section 7.1 for psTAG using PSCell as a reference cell and sTAG(s),

-	UE timer accuracy requirements in section 7.2 for UE operation on PSCell and SCell(s),

-	Timing advance requirements in section 7.3 for 1ms TTI and 4 subframe HARQ processing,

-	Radio link monitoring requirements in section 7.6 for PSCell,

-	SCell activation and deactivation delay requirements for E-UTRA carrier aggregation in section 7.7 for 1ms TTI and 4 subframe HARQ processing except those for CA with frame structure # 3,

-	E-UTRAN intra-frequency measurement requirements in section 8.1.2.2 for PSCC and SCC, except requirements specified for UE configured with highSpeedEnhancedMeasFlag,

-	E-UTRAN inter-frequency measurement requirements in section 8.1.2.3 for non-serving E-UTRA carrier frequencies, except requirements specified for UE configured with highSpeedEnhancedMeasFlag,

-	E-UTRAN OTDOA intra-frequency RSTD measurements requirements in section 8.1.2.5 for PSCC and SCC, except those for UE category 1bis,

-	E-UTRAN OTDOA inter-frequency RSTD measurements requirements in section 8.1.2.6 for non-serving E-UTRA carrier frequencies, except those for UE category 1bis,

-	E-UTRAN E-CID measurements requirements in section 8.19.5 for PSCell and SCell carrier frequencies, and in section 8.19.3 for non-serving E-UTRA carrier frequencies,

-	Requirements in Section 9 for intra-frequency RSRP, RSRQ, RS-SINR, and RSTD measurements accuracy on PSCC and SCC,

-	Requirements in Section 9 for inter-frequency RSRP, RSRQ, RS-SINR, and RSTD measurements accuracy on non-serving E-UTRA carrier frequencies,

-	Power headroom requirements in Section 9 for PSCell and SCell(s).

End of Change 2

For Information Only

#### 8.1.2.7	E-UTRAN E-CID Measurements

##### 8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements

…

##### 8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements

….

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements

8.1.2.7.3.1	Introduction

The requirements in section 8.1.2.7.3 shall apply provided the UE has received ECID-RequestLocationInformation message from E-SMLC via LPP requesting the UE to report E-CID E-UTRAN FDD intra-frequency RSRP and RSRQ measurements [24].

The UE capable of SRS carrier based switching, when configured to perform SRS carrier based switching for SRS transmission and/or RACH transmission over one or more SCells without PUSCH, shall perform UE Rx-Tx time difference measurement and meet the requirements defined in Section 8.1.2.7.3 provided the following condition is met:

-	at least one downlink subframe and one uplink subframe per radio frame are available for doing UE Rx-Tx time difference measurement at the UE in the PCell.

8.1.2.7.3.2	Measurement Requirements

The requirements in section 8.1.2.2.1 and section 8.1.2.8.1 also apply for this section except the measurement reporting requirements. The measurement reporting requirements for E-CID RSRP and RSRQ are defined in section 8.1.2.7.3.3.

8.1.2.7.3.3	Measurement Reporting Delay

This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report.

Reported RSRP and RSRQ measurements contained in periodically triggered measurement reports shall meet the requirements in section 9.1.2 and 9.1.5 respectively.

##### 8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements

8.1.2.7.4.1	Introduction

The requirements in section 8.1.2.7.4 shall apply provided the UE has received ECID-RequestLocationInformation message from E-SMLC via LPP requesting the UE to report E-CID E-UTRAN TDD intra-frequency RSRP and RSRQ measurements [24].

The UE capable of SRS carrier based switching, when configured to perform SRS carrier based switching for SRS transmission and/or RACH transmission over one or more SCells without PUSCH, shall perform UE Rx-Tx time difference measurement and meet the requirements defined in Section 8.1.2.7.4 provided the following condition is met:

-	at least one downlink subframe and one uplink subframe per radio frame are available for doing UE Rx-Tx time difference measurement at the UE in the PCell.

8.1.2.7.4.2	Measurement Requirements

The requirements in section 8.1.2.2.2 and section 8.1.2.8.2 also apply for this section except the measurement reporting requirements. The measurement reporting requirements for E-CID RSRP and RSRQ are defined in section 8.1.2.7.4.3.

8.1.2.7.4.3	Measurement Reporting Delay

This requirement assumes that that the measurement report is not delayed by other LPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PUSCH with subframe or slot or subslot duration. This measurement reporting delay excludes any delay caused by no UL resources for UE to send the measurement report. When the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

Reported RSRP and RSRQ measurements contained in periodically triggered measurement reports shall meet the requirements in section 9.1.2 and 9.1.5 respectively.

…

## 8.19	Measurements for NR – E-UTRA Dual Connectivity

### 8.19.1	Introduction

This clause contains requirements for UE supporting dual connectivity with NR PCell and E-UTRA PSCell.

Requirements in this clause are applicable to UEs which have been configured with NE-DC. Requirements in this clause are applicable to both E-UTRA FDD and E-UTRA TDD PSCell in combination with an NR PCell.

The application of measurement gap sharing and calculation of carrier specific scaling factor are the same as defined in clause 8.17.1.1 and 8.17.1.2, except that

-	The term PCell and PSCell shall be deemed to be swapped, and

-	UE is not expected to be configured to identify and measure cells on any of: inter-RAT NR, UTRAN, and GSM carriers.

### 8.19.2	Intra-frequency Measurements

PSCC intra-frequency measurements shall meet all applicable requirements in clause 8.1.2.2. If SCG DRX is in use, then the PCell intra-frequency requirements for when DRX is in use in clause 8.1.2.2 shall apply and shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.1.

SCC intra-frequency measurements shall meet all applicable requirements in clause 8.3.3. If SCG DRX is in use, then the PCell intra-frequency requirements for when DRX is in use in clause 8.1.2.2 shall apply and shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.1.

### 8.19.3	Inter-frequency Measurements

The E-UTRAN inter-frequency measurement requirements defined in section 8.19.3 shall apply when the UE capable of NE-DC is operating in NE-DC mode. The requiremenents in section 8.17.3 are applicable for gap pattern id # 0, 1, 2, 3, 4, 6, 7, 8, 10 as specified in Table 8.1.2.1-1.

The same requirements defined in clause 8.17.3 shall apply, except that

-	The term EN-DC shall be deemed to be replaced with NE-DC, and

-	The MCG DRX shall be deemed to be replaced with SCG DRX.

### 8.19.4	Void

### 8.19.5	Intra-frequency E-CID Measurements

PSCC intra-frequency E-CID measurements shall meet E-UTRAN E-CID intra-frequency measurements requirements in clause 8.1.2.7. If SCG DRX is in use, then the PCell intra-frequency requirements for when DRX is in use in clause 8.1.2.7 shall apply and shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.1.

SCC intra-frequency E-CID measurements shall meet all applicable requirements in clause 8.3.3. If SCG DRX is in use, then the PCell intra-frequency requirements for when DRX is in use in clause 8.1.2.2 shall apply and shall depend on the SCG DRX cycle. Otherwise, the requirements for when DRX is not in use shall apply. The applicable measurement accuracy requirements are in clause 9.1.