3GPP TSG-RAN WG4 Meeting #98-e	R4-2102784

Electronic Meeting, 25 January – 5 February, 2021

Title: 	Link level simulation results for SRS-RSRP

Source: 	Huawei, HiSilicon

Agenda item:	7.7.2.3.3

Document for:	Discussion

Introduction

In RAN4#97, the RRM requirements for gNB positioning are discussed, and the outcomes are captured in [1]. It is agreed to derive the Es/Iot side condition based on system level simulation. Once the side condition is agreed, the accuracy numbers and the related SRS configurations can be derived from link level simulations. RAN4# has agreed the link level simulation assumption in [2].

In this paper we will provide our simulation results for gNB SRS-RSRP measurement performance.

Discussion

Simulation assumption

The simulation assumption is same as in [2].

Simulation results

In Table 1-3, we show the SRS-PSRP estimation accuracy for 3 different SNR level with different SCS, where SNR1=-6dB, SNR2=-13dB and SNR3=-17dB. The results are for absolute SRS-RSRP accuracy, which is derived as the max(5%, 95%) value on the CDF curve for delta-RSRP. “n/a” in the table means the resource is not measurable.

Table 1: Link level simulation results for SRS-RSRP estimation with 15kHz

| RB | Comb and Symb | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  | SNR1 | SNR2 | SNR3 | SNR1 | SNR2 | SNR3 | SNR1 | SNR2 | SNR3 | SNR1 | SNR2 | SNR3 |
| 24 | 2 + 2 | 1.7 | 5.0 | n/a | 1.9 | 12.4 | n/a | 1.6 | 8.1 | n/a | 1.5 | 13.8 | n/a |
|  | 4 + 4 | 1.7 | 5.6 | n/a | 1.8 | n/a | n/a | 1.9 | 9.1 | n/a | 1.6 | n/a | n/a |
|  | 8 + 8 | 1.7 | 4.8 | n/a | 2.0 | n/a | n/a | 1.8 | n/a | n/a | 2.0 | n/a | n/a |
|  | 8 + 12 | 1.4 | 3.9 | n/a | 1.5 | n/a | n/a | 1.5 | 10.1 | n/a | 1.6 | 21.1 | n/a |
| 52 | 2 + 2 | 1.2 | 3.5 | n/a | 1.2 | 4.4 | n/a | 1.1 | 4.8 | n/a | 1.1 | 4.1 | n/a |
|  | 4 + 4 | 1.2 | 3.3 | n/a | 1.1 | 6.3 | n/a | 1.1 | 4.6 | n/a | 1.2 | 4.4 | n/a |
|  | 8 + 8 | 1.2 | 3.6 | n/a | 1.3 | 5.6 | n/a | 1.2 | 4.7 | n/a | 1.3 | 4.6 | n/a |
|  | 8 + 12 | 1.0 | 3.0 | 9.9 | 1.1 | 3.5 | n/a | 0.9 | 3.2 | n/a | 1.0 | 3.2 | n/a |
| 104 | 2 + 2 | 0.9 | 2.7 | 6.3 | 0.9 | 3.1 | 11.7 | 0.8 | 3.0 | n/a | 0.8 | 2.7 | n/a |
|  | 4 + 4 | 0.8 | 2.8 | 5.0 | 0.8 | 2.9 | 12.8 | 0.8 | 3.0 | 9.9 | 0.8 | 2.5 | 13.9 |
|  | 8 + 8 | 0.9 | 2.7 | 8.2 | 0.8 | 3.4 | n/a | 0.7 | 2.9 | n/a | 0.9 | 3.3 | n/a |
|  | 8 + 12 | 0.7 | 2.3 | 4.8 | 0.6 | 2.4 | 7.4 | 0.6 | 2.3 | 9.9 | 0.8 | 2.3 | 7.8 |
| 264 | 2 + 2 | 0.6 | 1.8 | 3.5 | 0.5 | 1.8 | 4.1 | 0.5 | 1.6 | 3.8 | 0.5 | 1.7 | 4.2 |
|  | 4 + 4 | 0.6 | 1.8 | 3.6 | 0.5 | 2.1 | 4.4 | 0.5 | 1.6 | 3.7 | 0.5 | 1.6 | 4.0 |
|  | 8 + 8 | 0.6 | 1.8 | 3.6 | 0.5 | 2.1 | 5.6 | 0.5 | 1.7 | 4.8 | 0.6 | 1.8 | 4.0 |
|  | 8 + 12 | 0.5 | 1.6 | 3.1 | 0.4 | 1.5 | 4.7 | 0.4 | 1.4 | 3.3 | 0.5 | 1.5 | 3.8 |

Table 2: Link level simulation results for SRS-RSRP estimation with for 30kHz

| RB | Comb and Symb | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  | SNR1 | SNR2 | SNR3 | SNR1 | SNR2 | SNR3 | SNR1 | SNR2 | SNR3 | SNR1 | SNR2 | SNR3 |
| 48 | 2 + 2 | 1.2 | 3.6 | n/a | 1.2 | 3.8 | n/a | 1.2 | 4.2 | n/a | 1.2 | 4.5 | n/a |
|  | 4 + 4 | 1.2 | 3.6 | n/a | 1.2 | 4.9 | n/a | 1.1 | 4.7 | n/a | 1.2 | 5.6 | n/a |
|  | 8 + 8 | 1.3 | 3.7 | n/a | 1.2 | 6.2 | n/a | 1.2 | 6.9 | n/a | 1.5 | 9.1 | n/a |
|  | 8 + 12 | 1.0 | 3.1 | 9.4 | 0.9 | 4.2 | n/a | 0.9 | 4.2 | n/a | 1.4 | 4.5 | n/a |
| 132 | 2 + 2 | 0.8 | 2.5 | 4.5 | 0.8 | 2.9 | 8.9 | 0.7 | 2.2 | 6.8 | 0.7 | 2.5 | 11.7 |
|  | 4 + 4 | 0.8 | 2.4 | 5.2 | 0.8 | 2.9 | 10.7 | 0.7 | 2.2 | 8.8 | 0.8 | 2.5 | n/a |
|  | 8 + 8 | 0.8 | 2.4 | 6.3 | 0.7 | 2.9 | n/a | 0.8 | 2.6 | n/a | 1.2 | 3.0 | 15.5 |
|  | 8 + 12 | 0.7 | 2.0 | 4.0 | 0.6 | 2.3 | 7.7 | 0.6 | 1.9 | 6.4 | 1.0 | 2.5 | 7.3 |
| 272 | 2 + 2 | 0.6 | 1.8 | 3.7 | 0.5 | 1.7 | 3.6 | 0.5 | 1.6 | 4.2 | 0.5 | 1.7 | 3.9 |
|  | 4 + 4 | 0.5 | 1.9 | 3.6 | 0.5 | 1.7 | 4.6 | 0.5 | 1.6 | 3.5 | 0.6 | 1.8 | 4.2 |
|  | 8 + 8 | 0.5 | 1.9 | 3.6 | 0.5 | 1.7 | 4.2 | 0.5 | 1.6 | 4.0 | 1.0 | 2.1 | 4.4 |
|  | 8 + 12 | 0.5 | 1.5 | 3.0 | 0.4 | 1.4 | 3.2 | 0.5 | 1.3 | 3.0 | 0.9 | 1.9 | 3.6 |

Table 3: Link level simulation results for SRS-RSRP estimation with for 120kHz

| RB | Comb and Symb | Channel |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | AWGN |  |  | TDL-C |  |  |
|  |  | SNR1 | SNR2 | SNR3 | SNR1 | SNR2 | SNR3 |
| 32 | 2 + 2 | 1.5 | 4.1 | n/a | 1.3 | 4.7 | n/a |
|  | 4 + 4 | 1.4 | 4.1 | n/a | 1.4 | 6.3 | n/a |
|  | 8 + 8 | 1.5 | 4.4 | n/a | 1.5 | 12.0 | n/a |
|  | 8 + 12 | 1.3 | 3.5 | n/a | 1.5 | 5.1 | n/a |
| 64 | 2 + 2 | 1.1 | 3.2 | n/a | 1.0 | 3.4 | n/a |
|  | 4 + 4 | 1.1 | 3.3 | n/a | 1.0 | 3.8 | n/a |
|  | 8 + 8 | 1.1 | 3.2 | n/a | 1.2 | 4.0 | n/a |
|  | 8 + 12 | 0.9 | 2.9 | 7.8 | 1.1 | 3.3 | 9.5 |
| 132 | 2 + 2 | 0.8 | 2.5 | 5.2 | 0.7 | 2.2 | 7.4 |
|  | 4 + 4 | 0.8 | 2.4 | 5.4 | 0.7 | 2.4 | 7.6 |
|  | 8 + 8 | 0.8 | 2.5 | 5.3 | 1.0 | 2.4 | 8.9 |
|  | 8 + 12 | 0.6 | 2.1 | 4.3 | 0.8 | 2.4 | 6.3 |

From the tables, we can observe that

Observation 1: The performance is very dependent on SNR conditions.

For example, with same SRS configuration with small BW like 24RB BW cannot work in -13dB and -17dB SNR condition, but it works well at -6dB.

Observation 2: The performance difference between different comb and symbol size is small.

The reason is that for a given BW, the number of SRS REs for 2+2, 4+4 and 8+8 are similar, so the processing gain is similar. The performance with 8+12 is in general a bit better than the other cases.

Observation 3: The performance are quite dependent on the channel profile.

Similar as UE side, the best accuracy performance is always seen under AWGN, and fading channel will lead to degradations.

Observation 4: The accuracy improves in proportion with BW in RB and the impact of SCS is small.

This is aligned with the expectation and can be most obviously seen in AWGN.

Conclusions

In this paper we will provide our simulation results for gNB SRS-RSRP measurement performance.

Observation 1: The performance is very dependent on SNR conditions.

Observation 2: The performance difference between different comb and symbol size is small.

Observation 3: The performance are quite dependent on the channel profile.

Observation 4: The accuracy improves in proportion with BW in RB and the impact of SCS is small.

Reference

- RP-2017159, WF on gNB positioning measurement requirements, Ericsson

- R4-2012142, Link simulation assumptions for deriving positioning SRS configurations, Nokia