3GPP TSG-RAN WG4 #98-e	     	          												 	      R4-2103142

Electronic Meeting, January 25th – February 5th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0689 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing parent clause for NR-DC PCMAX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Sub-clauses 6.2B.4.1 and 6.2B.4.2 without parent clause 6.2B.4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add missing parent clause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specifications structure is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< Start of Changes >>>

### 6.2B.3	UE additional maximum output power reduction for NR-DC

For inter-band NR-DC with one uplink assigned per band, the requirements in clause 6.2.3 apply for each uplink component carrier.

### 6.2B.4	Configured output power for NR-DC

#### 6.2B.4.1	Configured transmitted power level for NR-DC

The UE is allowed to set its configured maximum output power PCMAX,f,c,MCG and PCMAX,f,c,SCG for the respective MCG and SCG and its total configured maximum output power for NR-DC operation  PTotalNR-DC=10log10(PTotalNR-DC) with PTotalNR-DC as specified in clause 7.6.2 of [8]. The UE is configured with an inter-CG power sharing mode by NR-DC-PC-mode.The requirements apply for one uplink serving cell configured per CG and for asynchronous and synchronous NR-DC if not otherwise stated.

Unless otherwise stated, the configured maximum output power PCMAX,f,c,MCG (q) in physical-channel q for carrier f of serving cell c shall be set within the bounds if contained in the MCG,

PCMAX_L,f,c,MCG (q) ≤  PCMAX,f,c,MCG (q) ≤  PCMAX_H,f,c,MCG (q)

and the corresponding PCMAX_L,f,c,SCG (q) for a serving cell contained in the SCG,

PCMAX_L,f,c,SCG (q) ≤  PCMAX,f,c,SCG (q) ≤  PCMAX_H,f,c,SCG (q)

where PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,SCG and PCMAX_H,f,c,SCG are the limits for a serving cell c as specified in clause 6.2.4 modified as follows:

PCMAX_L,f,c,MCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,MCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass – ΔPPowerClass}

for the MCG and

PCMAX_L,f,c,SCG = MIN{MIN(PEMAX,c , PEMAX,NR-DC, PNR) – ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc)}

PCMAX_H,f,c,SCG = MIN{PEMAX,c, PEMAX,NR-DC, PNR, PPowerClass – ΔPPowerClass}

for the SCG, where

-	PEMAX,NR-DC is the value given by the field p-UE-FR1 of the PhysicalCellGroupConfig IE for the MCG as defined in [7];

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in [7];

For a UE provided with NR-DC-PC-mode = Semi-static-mode1,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass} + 0.3 dB

with PPowerClass set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

If for synchronous NR-DC operation a UE is provided NR-DC-PC-mode = Semi-static-mode2, the PTotalNR-DC is determined as above and

-	if at least one symbol of slot i1 of the MCG/SCG is indicated as uplink or flexible to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided, overlaps with a symbol for any ongoing transmission overlapping with slot i2 of the SCG/MCG, the UE determines a maximum power for the transmission on the SCG/MCG overlapping with slot i2 using the configured maximum power PCMAX,f,c,SCG or PCMAX,f,c,MCG for the SCG or MSG, respectively,

-	otherwise (i.e. an ongoing transmission overlapping with slot i2 of the SCG/MCG overlaps with only semi-static downlink symbols within slot i1 of the MCG/SCG), the UE determines a maximum power for the transmission on MCG or the SCG overlapping with slot i2 using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability for dynamic power sharing between the MCG and the SCG and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass}

with PPowerClass set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG except

-	if UE transmission(s) in slot i1 of the MCG or in slot i2 of the SCG do not overlap in time with any UE transmission(s) on the SCG or the MCG, respectively, the UE determines a maximum transmission power in slot i1 of the MCG or in slot i2 of the SCG using the configured maximum power as specified in clause 6.2.4.

If a UE indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG as specified in clause 7.6.2 of [8], and is provided with NR-DC-PC-mode = Dynamic,

PTotalNR-DC = MIN{PEMAX, NR-DC, PPowerClass}

with PPowerClass set to power class 3 in case the UE indicates a higher power class in any CG. The UE determines the maximum transmission power for the MCG and the SCG using the respective configured maximum power PCMAX,f,c,MCG and PCMAX,f,c,SCG.

The measured total maximum output power PUMAX over both CGs measured over the transmission reference time duration is

PUMAX = 10 log10 (pUMAX,c,MCG + pUMAX,c,SCG),

where pUMAX,c,MSG and pUMAX,c,SCG denote the measured output power of serving cells c contained in the respective MSG and SCG expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When a subframe p on the MSG overlap with a physical-channel q on the SCG, then for PUMAX evaluation, the subframe p on the MCG is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframe and physical-channel durations are used on the carriers. The PPowerClass shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| Transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different CG carriers | MCG subframe | MIN(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX{PCMAX_NR-DC_H(p,q), PCMAX_NR-DC_H(p,q+1), … , PCMAX_NR-DC_H(p,q+n)}

where PCMAX_NR-DC_H entries are the applicable upper limits for each overlapping scheduling unit pairs (p,q), (p, q+1), up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG, while PCMAX_L is computed as follows:

PCMAX_L = MIN{PCMAX_NR-DC_L(p,q), PCMAX_NR-DC_L(p,q+1), … , PCMAX_NR-DC_L(p,q+n)}

where PCMAX_NR-DC_L entries are the applicable lower limits for each overlapping scheduling unit pairs (p,q), (p, q+1) up to (p, q+n) for each applicable Teval duration, where q+n is the last physical-channel on the SCG overlapping with subframe p on the MCG.

For a UE provided with NR-DC-PC-mode = Semi-static-mode1 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the values of the PNR for the respective MCG and SCG expressed in linear scale

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)]

with pCMAX_L,f,c,MCG, pCMAX_H,f,c,MCG, pCMAX_L,f,c,SCG, and pCMAX_H,f,c,SCG the values of the respective PCMAX_L,f,c,MCG, PCMAX_H,f,c,MCG, PCMAX_L,f,c,MCG, and PCMAX_H,f,c,SCG expressed in linear scale, while the measured configured maximum power PUMAX  for each CG shall meet the requirements as specified in clause 6.2.4 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG  as specified in this clause.

If for synchronized NR-DC a UE is provided with NR-DC-PC-mode = Semi-static-mode2 and configured with pNR,MCG + pNR,SCG ≤ PTotalNR-DC with pNR,MCG and pNR,SCG the linear-scale values of the PNR for the respective MCG and SCG

PCMAX_NR-DC_L(p,q) = 10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)]

PCMAX_NR-DC_H(p,q) = 10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)]

while the measured configured maximum power PUMAX for each CG shall meet the requirements specified in Table 6.2.4-2 but with bounds for PCMAX,f,c,MCG (p) and PCMAX,f,c,SCG  as specified in this clause except

-	if an ongoing transmission overlapping with physical channel q of the SCG or subframe p of the MCG overlaps with only semi-static downlink symbols within the respective subframe p of the MCG or physical channel q of the SCG as indicated to a UE by tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated, if provided,

then the measured configured maximum power PUMAX for the transmission subframe p on the MCG or physical channel q on the SCG shall meet the requirements as specified in clause 6.2.4 and with bounds for PCMAX,f,c,MCG (p) or PCMAX,f,c,SCG  as specified in clause 6.2.4.

For a UE provided with NR-DC-PC-mode = Dynamic,

PCMAX_NR-DC_L(p,q) = MIN{10 log10 [pCMAX_L,f,c,MCG (p) + pCMAX_L,f,c,SCG (q)], PTotalNR-DC}

PCMAX_NR-DC_H(p,q) = MIN{10 log10 [pCMAX_H,f,c,MCG (p) + pCMAX_H,f,c,SCG (q)], PTotalNR-DC}

while the measured configured maximum power PUMAX on the MCG shall meet the requirements as specified in clause 6.2.4-2 but with bounds for PCMAX,f,c,MCG (p) as specified in this clause, and the PUMAX on the SCG shall be within

PCMAX_L,f,c  –  MAX{TL,c, T(PCMAX_L,f,c)}  ≤  PUMAX,f,c  ≤  PCMAX_H,f,c  +  T(PCMAX_H,f,c).

where

PCMAX_L,f,c = MIN{PCMAX_L,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

PCMAX_H,f,c = MIN{PCMAX_H,f,c,SCG (p), 10 log10 (PTotalNR-DC – pNR,MSG)}

with limits as specified in Table 6.2.4-2 and pNR,MCG the value of the PNR for the MCG expressed in linear scale.

Table 6.2B.4.1.3-2: PCMAX tolerance for NR-DC

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs provided with NR-DC-PC-mode = Semi-static-mode1 or with NR-DC-PC-mode = Semi-static-mode2, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

#### 6.2B.4.2	ΔTIB,c for NR-DC

For inter-band NR-DC with one uplink carrier assigned per NR band, the ΔTIB,c for the corresponding inter-band CA configuration as specified in clause 6.2A.4.2 applies.

<<< End of Changes >>>