3GPP TSG-RAN WG4 Meeting # 98-e 												R4-2103342

Electronic Meeting, 25 Jan. – 5 Feb. 2021

Agenda item:			15.1

Source:	Moderator (Samsung)

Title:	Email discussion summary for [98e][156] NR_reply_LS_Part_3

Document for:	Information

Introduction

Briefly introduce background, the scope of this email discussion and provide some guidelines for email discussion if necessary.

RAN4 has received an LS from RAN1 [R1-2009807] on beam switching gaps for multi-TRP UL transmission, in which RAN1 agreements and working assumptions on multi-TRP transmission/repetition schemes for both PUCCH and PUSCH are shared, with questions to RAN4 as follows:

While the questions below are formulated based on different beams in the context of FR2, RAN1 is also interested in the answers for FR1 in the case of PUCCH/PUSCH repetitions follow different transmission power control. RAN1 seeks few answers from RAN4 to proceed further with the working assumptions based on existing RAN1 agreements, Question 1: What are the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? Question 2: In RAN4 perspective, are there additional considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? Question 3: For different beam mapping principles (i.e. cyclical and sequential mapping patterns), is there any additional complexity that RAN4 foresees when applying cyclical beam mapping vs sequential beam mapping? Question 4: In particular to multi-TRP intra-slot beam hopping (Scheme 2), can RAN1 assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs?

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

This email thread is used to discussion above questions and response the RAN1 LS, with the following plan to organize the discussion:

- 1st round: Discussion on issues based on companies’ contribution input.

- 2nd round: Achieve agreements on the reply LS to RAN1

Topic #1: Beam switching gaps for Multi-TRP UL transmission

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101017 | Apple | Observation #1: The transient time for SRS switching in FR1 could be reused with transmission switching with different power levels and from antenna ports. Observation #2: The transient time in FR2 could be 5us with transmission with different power levels. Observation #3: The time for TX beam switch ranges from few symbols up to 1 slot if the spatial filter to transmit the beam is known. Observation #4: The time for TX beam switch would further include the additional time for beam refinement on top of transient period if the spatial filter to transmit the beam is unknown. Observation #5: The beam switching delay might need to consider different UL TX timing for transmissions to multi-TRP. Question 1: What are the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? [RAN4] The transient period between two PUCCH/PUSCH TDMed repetitions should account for TX power change, TX beam change and possibly TX timing change. The TX timing change is not including in the transient period below. In FR1, the transient period needs to account for TX power change and ranges from 10us to 15us depending on whether the switch from one transmission to the next is from the same antenna port or different antenna ports. In FR2, the transient period shall account for TX power change, TX beam change. The transient due to TX power change is 5us in FR2. The TX beam change should also consider if the beams are known (Spatial filter for transmission is known and not outdated). In case the TX beam is unknown, the transient time needs to account for beam refinement time. In addition, the TX beam change also need to consider inter-panel beam switch. Accounting for inter-panel TDMed transmissions, the transient period could be few symbols up to 1 slot, assuming the beams are known. It is not practical to assume that UE would keep both panels active continuously and hence need to consider panel activation, de-activation time and the associated interruption on ongoing transmission on all CCs. The length of the interruption can be a few symbols up to 1 slot. Question 2: In RAN4 perspective, are there additional considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? [RAN4] In all cases, when the switching delay is longer than a threshold, e.g. CP length, a switching gap can be considered. In certain cases, when the switching delay is more than 1/3rd the symbol time in the operating SCS, a switching gap should be considered. Question 3: For different beam mapping principles (i.e. cyclical and sequential mapping patterns), is there any additional complexity that RAN4 foresees when applying cyclical beam mapping vs sequential beam mapping? [RAN4] Comparing the cyclic and sequential mapping, cyclic mapping has more frequent switches. Cyclic mapping would result in more power consumption at the UE in addition to more switching gaps. Cyclic mapping also increases the interruption to other ongoing transmissions with more frequent activation and de-activation of panels. Question 4: In particular to multi-TRP intra-slot beam hopping (Scheme 2), can RAN1 assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs? [RAN4] The current RAN4 requirements for transient times are applicable when RB hopping, or power change is applied. In case of RB hopping with different UL beams, we need to consider additional time due to power change and TX beam switching. In addition, RAN4 also would like to clarify the following with RAN1: Responses from RAN4 are based on the assumption that the same UL timing is used for transmission of both UL beams. Assuming that the UL beams are associated with different DL-RS from different TRP, there could be different DL timing correction for each. What is the TX timing assumption for different UL beam transmissions? |
| R4-2101157 | MediaTek | Observation 1: UL Tx beam switching is highly likely to be done together with UL transmit power change due to different path losses observed from 2 TRPs. Observation 2: The transition time of beam hopping is much smaller than the CP Proposal 1: RAN4 to reply a single value of transient period to RAN1, considering UL Tx beam, RB allocation and UL transmit power are changed at the same time. Proposal 2: The transition time for multi-TRP PUCCH/PUSCH transmission shall mainly consider the frequency hopping and power change. Proposal 3: The transition time for beam hopping, frequency hopping and power change can reuse the same transition time defined in TS38.101-1 and TS38.101-2, i.e., 15us and 5us for FR1 and FR2, respectively. Question 1: What are the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? Answer: UE UL beam switch between two PUCCH/PUSCH TDMed repetitions is expected to be conducted together with UL transmit power change. Therefore, 15us and 5us are required for FR1 and FR2, respectively. Question 2: In RAN4 perspective, are there additional considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? Answer: When the transient period is larger than CP, the switching gap is needed to avoid the performance degradation. Question 3: For different beam mapping principles (i.e. cyclical and sequential mapping patterns), is there any additional complexity that RAN4 foresees when applying cyclical beam mapping vs sequential beam mapping? Answer: RAN4 does not see any additional complexity. Question 4: In particular to multi-TRP intra-slot beam hopping (Scheme 2), can RAN1 assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs? Answer: Yes. |
| R4-2102389 R4-2102390 | Huawei, HiSilicon | Question 1: What are the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? Answer 1: For the case that different beams switched in the same panel, the switching could be completed in CP for 120kHz SCS for FR2 UE, thus transient period of 5us as defined in the RAN4 spec plays the main role between two PUCCH/PUSCH TDMed repetitions. For the case that different beams switched cross panels, RAN4 once discussed similar beam switching issue in Rel-15. The switching time is reported as UE capability, where 224 and 336 symbol length are used for cross panel switching. In this case, the trainset period would be determined by the beam switching time. Question 2: In RAN4 perspective, are there additional considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? Answer 2: As 5us transient period defined in RAN4 specification occupies larger part of a symbol for 120kHz SCS, a blanked symbol is considered in the time mask requirement for FR2 UE. However, whether a switching gap should be defined from performance or PHY design perspective, RAN4 thinks that it should be determined by RAN1. Question 3: For different beam mapping principles (i.e. cyclical and sequential mapping patterns), is there any additional complexity that RAN4 foresees when applying cyclical beam mapping vs sequential beam mapping? Answer 3: From the definition, beam switching for cyclical mapping could occur more frequently than that of sequential beam mapping. However, from implementation perspective, no big difference or additional complexity is foreseen for the UE by RAN4. Question 4: In particular to multi-TRP intra-slot beam hopping (Scheme 2), can RAN1 assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs? Answer 4: For RB hopping, transient period is defined as 5us for FR2 UE. Similar to Q1, beam hopping could be happened in the same panel or between two panels. Transient period due to RB hopping is the main factor for the same panel case, while for cross panels case, switching period is mainly determined by beam switching, which is 224 or 336 symbol length according to UE capability based on RAN4 agreement. Regarding to the question for FR1 in the case of PUCCH/PUSCH repetitions follow different transmission power control, since beam switching is less relevant to FR1 than that for FR2, transient period due to power change defined as 10us in RAN4 specification can be considered by RAN1. |
| R4-2102710 | Vivo | Observation 1: RAN1’s main interest is in possible impact of the beam related behaviour to transient period and gaps etc. Observation 2: There are many situations that would need to be considered in RAN1’s provided conditions. Observation 3: Between continuous ON-power transmissions, RAN4 have defined transient period allowed for the following two cases: power change or RB hopping is applied. Observation 4: For the minimum requirement number, generally 10us for FR1 and 5 us for FR2. Observation 5: Only in the case of the highest SCS and transient period is required on both sides of the symbol, a one symbol gap is defined in RAN4 requirements. (As in Figure 6.3.3.9-3 in TS38.101-1 and Figure 6.3.3.9-3 in TS38.101-2) Observation 6: The beam switching delay for PUCCH & PUSCH beam sweeping is not related to PDCCH/MAC_CE/RRC decoding, and it is not related to whether the status is known or unknown. Observation 7: Indicating change to the spatial relation info of the SRSs that provide beam sweeping information of PUSCH & PUCCH transmission is another issue, and the requirements are already discussed in R16 RRM enhancement. Question 1: What are the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? For two PUCCH/PUSCH TDMed repetitions (with different UL beams), according to the previous RAN1 background, they may “follow different transmission power control”. It means there may be a power change. In case there is a power change, the current transient period minimum requirements (10us for FR1 and 5us for FR2) should apply. In case no such power change, this transient period need not be considered. Although the transient period is defined in R15 considering beam sweeping within a panel, companies see it can also apply to the case of beam sweeping between panels. Therefore, the same requirements are assumed. Question 2: In RAN4 perspective, are there additional considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)? Currently RAN4 only consider a one symbol gap in the case of the highest SCS and transient period is required on both sides of the symbol. For other cases, blanked symbol is not defined. Question 3: For different beam mapping principles (i.e. cyclical and sequential mapping patterns), is there any additional complexity that RAN4 foresees when applying cyclical beam mapping vs sequential beam mapping? As in answer for question 2, RAN4 only consider a one symbol gap in the case of the highest SCS and transient period is required on both sides of the symbol. For other cases, blanked symbol is not defined. Therefore, there is no additional complexity. Question 4: In particular to multi-TRP intra-slot beam hopping (Scheme 2), can RAN1 assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs? As long as RB hopping and/or power changes applies, RAN4 requirement is the same. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1-1: transient period between two PUCCH/PUSCH TDMed repetitions

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel

- Proposals:

  - Option 1: 5us

  - Option 2: 5us if the beams are known; In case the TX beam is unknown, the transient time needs to account for beam refinement time

- Recommended WF

  - TBA.

Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels

- Proposals:

  - Option 1: 5us

  - Option 2: the transient period would be determined by the beam switching time which is reported as UE capability, where 224 and 336 symbol length are used for cross panel switching

  - Option 3: ranges from few symbols up to 1 slot if the spatial filter to transmit the beam is known. Additional beam refinement time need to be considered if the beam is unknown. Panel activation, de-activation time and the associated interruption on ongoing transmission on all CCs shall also be considered which can be a few symbols up to 1 slot.

- Recommended WF

  - TBA

Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE

- Proposals:

  - Option 1: reply RAN1 with a single value 10us

  - Option 2: reply RAN1 with a single value 15us

  - Option 3: ranges from 10us to 15us depending on whether the switch from one transmission to the next is from the same antenna port or different antenna ports

- Recommended WF

  - TBA

Sub-topic 1-2: blanked symbol(s) consideration for a switching gap

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)

- Proposals:

  - Option 1: In all cases, when the switching delay is longer than a threshold, e.g. CP length, a switching gap can be considered. In certain cases, when the switching delay is more than 1/3rd the symbol time in the operating SCS, a switching gap should be considered.

  - Option 2: When the transient period is larger than CP, the switching gap is needed to avoid the performance degradation

  - Option 3: Whether a switching gap should be defined from performance or PHY design perspective, RAN4 thinks that it should be determined by RAN1 though RAN4 defines minimum time mask requirement with a blanked symbol

  - Option 4: Currently RAN4 only consider a one symbol gap in the case of the highest SCS and transient period is required on both sides of the symbol. For other cases, blanked symbol is not defined.

- Recommended WF

  - TBA.

Sub-topic 1-3: different beam mapping patterns

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping?

- Proposals:

  - Option 1: RAN4 does not see any additional complexity.

  - Option 2: Cyclic mapping would result in more power consumption at the UE in addition to more switching gaps. Cyclic mapping also increases the interruption to other ongoing transmissions with more frequent activation and de-activation of panels.

  - Option 3: Power consumption could be slightly different, however, from implementation perspective, no big difference or additional complexity is foreseen for the UE by RAN4.

- Recommended WF

  - TBA.

Sub-topic 1-4: multi-TRP intra-slot beam hopping (Scheme 2)

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs

- Proposals:

  - Option 1: Yes

  - Option 2: The current RAN4 requirements for transient times are applicable when RB hopping, or power change is applied. In case of RB hopping with different UL beams, we need to consider additional time due to power change and TX beam switching.

  - Option 3: beam hopping could be happened in the same panel or between two panels. Transient period due to RB hopping is the main factor for the same panel case, while for cross panels case, switching period is mainly determined by beam switching, which is 224 or 336 symbol length according to UE capability based on RAN4 agreement.

- Recommended WF

  - TBA

Sub-topic 1-5: TX timing aspect

Sub-topic description

Open issues and candidate options before e-meeting:

Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions

- Background: Responses from RAN4 are based on the assumption that the same UL timing is used for transmission of both UL beams. Assuming that the UL beams are associated with different DL-RS from different TRP, there could be different DL timing correction for each.

- Proposals:

  - Option 1: ask RAN1 what the TX timing assumption is for different UL beam transmissions.

  - Option 2: tell RAN1 the same UL timing is assumed for different UL beam transmissions in current RAN4 requirements.

- Recommended WF

  - TBA.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| MediaTek | Issue 1-1-1: both options are fine to us. In our understanding, RAN1 does not care known or unknown Tx beam. Some clarification is needed on whether this additional know/unknown information can help RAN1 progress. Issue 1-1-2: support option 2. In our contribution, we do not cover the case with panel switch. If beam switching cross panel is considered, UE needs more time to warm up. Issue 1-1-3: support option3. The transient period is depending on same or different antenna ports. It is fine to us to provide RAN1 the detail. It is up to RAN1 to select which value for their scenarios. Issue 1-2-1: support option2. In our understanding, if the transient time is covered by CP, then it should be ok without switching gap. Issue 1-3-1: support option 2 and 3. We agree the power consumption may be different when cyclic mapping is applied, although it is not clear at this moment about how much the difference in power consumption is. Issue 1-4-1: For the transient time of RB hopping in the same panel, the existing transient time could be reused. For the different panel, the transient time shall follow the requirement of the beam hopping. Issue 1-5-1: According to the conclusion of uplink spatial relation switching in RRM section, UE is not provided with time of a timing tracking. As a consequence, timing accuracy requirement for uplink spatial relation switching is not defined. We believe it is completely up to UE implementation. |
| Qualcomm | Sub topic 1-1: Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel - Option 1: 5us for FR2 Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels - Option 1: 5us for FR2. In this scenario the UE knows which beams it is using to do switching. It also knows whether these beams are located on different panels. If switching across panels is required, then the UE would turn on both panels well before switching takes place so that it can avoid the panel turn on time and reduce the duration of the switching transient. If the panels are turned on before switching, as one would expect, there is no need to consider panel turn on time in the transient time. Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE - Option 3: ranges from 10us to 15us depending on whether the switch from one transmission to the next is from the same antenna port or different antenna ports Sub topic 1-2: Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) - Option 5: For short subslot currently RAN4 only consider a one symbol gap in the case of the highest SCS and transient period is required on both sides of the symbol. For other cases, blanked symbol is not defined. Sub topic 1-3: Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? - Option 1: RAN4 does not see any additional complexity. Though the power consumption may be higher with cyclic switching on account of there being more switching events. Sub topic 1-4: Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs - Option 1: Yes Sub topic 1-5: Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions - Option 2: Tell RAN1 the same UL timing is assumed for different UL beam transmissions in current RAN4 requirements. |
| Apple | Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel Option 2. Since RAN1 in their LS mentioned that beams may be known or unknown. If known beams are guaranteed, then we can go with option 1. Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels We support option 3. It might not be practical to assume that UE activates both panels all the time . Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE Option 3. RAN1 requested range of transient time. Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) Option 1 was our proposal in our paper based on the blanked symbol for SRS switching. To simplify we are fine with option 2 as well. Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? Option 2. Cyclic mapping would result in more power consumption at the UE in addition to more switching gaps. Cyclic mapping also increases the interruption to other ongoing transmissions with more frequent activation and de-activation of panels. Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs Option 2: The current RAN4 requirements for transient times are applicable when RB hopping, or power change is applied. In case of RB hopping with different UL beams, we need to consider additional time due to power change and TX beam switching. Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions Option 1: ask RAN1 what the TX timing assumption is for different UL beam transmissions. |
| vivo | Issue 1-1-1 Option 1. In our view known or unknown states is not related to the issue that RAN1 is discussing. Issue 1-1-2 Option 1. Agree with Qualcomm’s understanding. Issue 1-1-3 Option 3. Issue 1-2-1 Option 4 is our proposal and new option 5 from Qualcomm is also ok Option 1/2 can also be considered as supplemental information but wording need to be further checked, e.g. “is needed” may be consider to be “may be needed” Issue 1-3-1 Option 1. Power consumption is highly related to UE implementation and is not related to RAN4 requirements on this issue. In addition, this is not related to complexity which was asked. Issue 1-4-1 Slightly prefer option 1. According to R16 requirements, we do not think additional gap for UL beam switching is needed. Anyway we are open to discuss. But RAN4 should firstly clarify the intention from RAN1 and make sure that known/unknown states are not related to the issue. Issue 1-5-1 Option 2. |
| Samsung | Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel Prefer option 1. In case beam is unknown, beam refined time is needed for the beam switch corresponding to the same TRP, but what RAN1 interested is the beam switching gap between beams corresponding different TRPs. If consensus is not achieved, we support vivo’s comment “RAN4 should firstly clarify the intention from RAN1 and make sure that known/unknown states are not related to the issue” Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels We think this issue is depending on if the panels are activated. If panels are activated already, option 1 makes sense; if the panel switched to is not activated, then option 2 is reasonable. Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE Option 3. Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) Prefer option 2. Option 4 and Qualcomm proposed Option 5 describes current RAN4 requirement which is based on the same UL beam. In order to cover different UL beam cases, option 2 is more general answer. Option 4 and 5 could be for RAN1’s information if consensus can be achieved. Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? Prefer Option 2. It is meaningful to tell RAN1 the addition complexity what RAN4 can foresee. Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs This issue is related with Issue 1-1-2. That depends on if we need to consider panel activation and deactivation. We are fine with option 3 and can be further revised to option 3a: Option 3a: if beam hopping happens in the same panel or between two activated panels, transient period due to RB hopping is the main factor; if the panel hopped to is not activated, transient period is mainly determined by beam switching, which is 224 or 336 symbol length according to UE capability. Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions We have no strong view but at least the information in option 2 shall be shared with RAN1. |
| Huawei, HiSilicon | Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel Option 1: 5us for FR2. Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels Option 2. The values of 224 and 336 symbol length are previous agreement in Rel-15. Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE Option 3 is ok for us. Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) Option 3. RAN4 can feedback to RAN1 the transient period based on implementation perspective. Whether to consider a switching gap can be decided by RAN1 from performance perspective. Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? Option 3. Power consumption is one aspect to be considered. But that may not be directly related to the question by RAN1 on complexity issue. Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs Option 3. The transient period depends on whether the switching happens in the same panel or different panels. Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions Tx timing is derived from DL timing and the DL timing difference for multi-TRP is assumed in a CP based on RAN1 agreement. We don't think that RAN4 needs to consider this issue in the reply LS. |
| Nokia | Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel Option 1 Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels Option 1: if we understand QC’s comment correctly, their explanation makes sense. We understand comments on power consumption. However, UE does not have to always make both panels activated. The UE just makes both panels activated before UL beam switching as long as it needs. Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE Option 1: In Rel16, we have had a discussion that there UEs to deal with down to 2us transient period for FR1, though we understand that this is an optional feature. Still it would be expected that current 10us must have sufficient margin. Thus, we prefer to take 10us even if physical antenna ports are switched. Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) None of the provided options. RAN4 should focus on finding out required time as requirements. If the required period for UL beams switching is treated as guard period like SRS antenna switching, it depends on RAN1 decision. If no guard period is introduced, then, according to relation between required transient period and symbol length per SCS, RAN4 may define blanked symbol. Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? Option 1: This does not impact on RAN4 UE RF requirements, though it would change occasions of transient period and it would be related to power consumption. Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs Option 1: Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions None of the provided options. It is better to check if there have been discussion on this in RRM room first. Then, we can discuss what information we are going to share with RAN1. |
| Ericsson | Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel Option 1 Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels Option 1, supportive of Qualcomm’s understanding. Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE Option 3 Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) At this early stage it is hard to evaluate, RAN4 has not started any work on this related W.I Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? Option 1: Any effect on pwr consumption at the UE is not a complexity in itself. |
| Qualcomm | Sub topic 1-1: Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel - Option 1: 5us for FR2 (if beams are known) - If beams are unknown the transient time needs to account for beam selection time. Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels - Option 1: 5us for FR2. In this scenario the UE knows which beams it is using to do switching. It also knows whether these beams are located on different panels. If switching across panels is required, then the UE would turn on both panels well before switching takes place so that it can avoid the panel turn on time and reduce the duration of the switching transient. If the panels are turned on before switching, as one would expect, there is no need to consider panel turn on time in the transient time. - If inter-panel switching is required and a panel must be woken up, then this will add additional delay to the transient time and if the beam to switch to is not known it will add more time for beam selection Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE - Option 3: ranges from 10us to 15us depending on whether the switch from one transmission to the next is from the same antenna port or different antenna ports Sub topic 1-2: Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) - Option 5: For short subslot currently RAN4 only consider a one symbol gap in the case of the highest SCS and transient period is required on both sides of the symbol. For other cases, blanked symbol is not defined. Sub topic 1-3: Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? - Option 1: RAN4 does not see any additional complexity if beams are known. Though the power consumption may be higher with cyclic switching on account of there being more switching events. - If switching across panels or if the beams are unknown there will be additional delays as described in Issue 1-1-2 Sub topic 1-4: Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs - Option 1: Yes (as long as beams are known and the panels are on) - If beams are not known and the panels have to be turn on there will be additional delays Sub topic 1-5: Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions - Option 2: tell RAN1 the same UL timing is assumed for different UL beam transmissions in current RAN4 requirements. |

CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Issue 1-1-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel - Proposals: - Option 1: 5us - Option 2: 5us if the beams are known; In case the TX beam is unknown, the transient time needs to account for beam refinement time - 1st round discussion summary - 5 companies support Option 1 (vivo, Samsung, Huawei, Nokia, Ericsson) - 2 companies support Option 2 (Apple, Qualcomm) - 1 company supports both (MediaTek) Tentative agreements: - 5us transient period for different UL beams switched in the same panel if the beams are known Candidate options: Recommendations for 2nd round: - Suggest to agree Option 1 based on majority view - If necessary, further discuss RAN4 understanding on beam known/unknown status and its relationship with this issue |
|  | Issue 1-1-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels - Proposals: - Option 1: 5us - Option 2: the transient period would be determined by the beam switching time which is reported as UE capability, where 224 and 336 symbol length are used for cross panel switching - Option 3: ranges from few symbols up to 1 slot if the spatial filter to transmit the beam is known. Additional beam refinement time need to be considered if the beam is unknown. Panel activation, de-activation time and the associated interruption on ongoing transmission on all CCs shall also be considered which can be a few symbols up to 1 slot. - 1st round discussion summary: - Companies show different views but mainly focus on Option 1 and Option 2. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: - Suggest to further discuss between Option 1 and Option 2 - Further discuss whether it is necessary to distinguish panel activation status |
|  | Issue 1-1-3: the ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions for FR1 UE - Proposals: - Option 1: reply RAN1 with a single value 10us - Option 2: reply RAN1 with a single value 15us - Option 3: ranges from 10us to 15us depending on whether the switch from one transmission to the next is from the same antenna port or different antenna ports - 1st round discussion summary: - All companies support Option 3 except one company supports Option 1. Tentative agreements: - Option 3 Candidate options: Recommendations for 2nd round: - Suggest to agree Option 3 |
| Sub-topic#1-2 | Issue 1-2-1: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams) - Proposals: - Option 1: In all cases, when the switching delay is longer than a threshold, e.g. CP length, a switching gap can be considered. In certain cases, when the switching delay is more than 1/3rd the symbol time in the operating SCS, a switching gap should be considered. - Option 2: When the transient period is larger than CP, the switching gap is needed to avoid the performance degradation - Option 3: Whether a switching gap should be defined from performance or PHY design perspective, RAN4 thinks that it should be determined by RAN1 though RAN4 defines minimum time mask requirement with a blanked symbol - Option 4: Currently RAN4 only consider a one symbol gap in the case of the highest SCS and transient period is required on both sides of the symbol. For other cases, blanked symbol is not defined. - Option 5(new): For short subslot currently RAN4 only consider a one symbol gap in the case of the highest SCS and transient period is required on both sides of the symbol. For other cases, blanked symbol is not defined. - Option 6(new): RAN4 should focus on finding out required time as requirements. If the required period for UL beams switching is treated as guard period like SRS antenna switching, it depends on RAN1 decision. If no guard period is introduced, then, according to relation between required transient period and symbol length per SCS, RAN4 may define blanked symbol. - Option 7(new): At this early stage it is hard to evaluate, RAN4 has not started any work on this related W.I - 1st round discussion summary: - No obvious majority view. 3 companies support Option 2. No more than two companies support other options. Tentative agreements: N/A Candidate options: - Solution-1: When the transient period is larger than CP, the switching gap is needed to avoid the performance degradation (based on Option 1/2) - Solution-2: It depends on RAN1 decision whether applying blanked symbol(s) (based on Option 3/6) - Solution-3: RAN4 has no evaluation yet. Share with RAN1 current RAN4 requirement for information. (based on Option 4/5/7) Recommendations for 2nd round: - Further discuss in 2nd round based on above Solution-1/2/3 or combinations of them. |
| Sub-topic#1-3 | Issue 1-3-1: whether RAN4 foresees any additional complexity when applying cyclical beam mapping vs sequential beam mapping? - Proposals: - Option 1: RAN4 does not see any additional complexity. - Option 2: Cyclic mapping would result in more power consumption at the UE in addition to more switching gaps. Cyclic mapping also increases the interruption to other ongoing transmissions with more frequent activation and de-activation of panels. - Option 3: Power consumption could be slightly different, however, from implementation perspective, no big difference or additional complexity is foreseen for the UE by RAN4. - 1st round discussion summary: - 4 companies support Option 1 (Qualcomm, vivo, Nokia, Ericsson) - 3 companies support Option 2 (MediaTek, Apple, Samsung) - 2 companies support Option 3 (MediaTek, Huawei) - Companies foresee there is power consumption difference but some companies don’t think it is related with RAN4 complexity. Tentative agreements: - RAN4 foresees more power consumption due to more frequent beam switching events when applying cyclical beam mapping vs sequential beam mapping, however, RAN4 does not see any additional complexity from RAN4 UE RF requirement perspective. Candidate options: Recommendations for 2nd round: - Further discuss above tentative agreements. |
| Sub-topic#1-4 | Issue 1-4-1: whether RAN1 can assume the same requirement as RB hopping with respect to transient period in current RAN4 requirements, if the two hops have different UL beams in addition to different RBs - Proposals: - Option 1: Yes - Option 2: The current RAN4 requirements for transient times are applicable when RB hopping, or power change is applied. In case of RB hopping with different UL beams, we need to consider additional time due to power change and TX beam switching. - Option 3: beam hopping could be happened in the same panel or between two panels. Transient period due to RB hopping is the main factor for the same panel case, while for cross panels case, switching period is mainly determined by beam switching, which is 224 or 336 symbol length according to UE capability based on RAN4 agreement. - 1st round discussion summary: - Companies provide different views similar as subtopic#1-1 due to different understanding of beam known/unknown issue and panel activation issue Tentative agreements: N/A Candidate options: Recommendations for 2nd round: - Focus on beam known/unknown issue and panel activation issue discussion in subtopic#1-1 |
| Sub-topic#1-5 | Issue 1-5-1: whether different TX timing is considered for different UL beam transmissions - Proposals: - Option 1: ask RAN1 what the TX timing assumption is for different UL beam transmissions. - Option 2: tell RAN1 the same UL timing is assumed for different UL beam transmissions in current RAN4 requirements. - Option 3(new): It is better to check if there have been discussion on this in RRM room first. Then, we can discuss what information we are going to share with RAN1 - Option 4(new): RAN4 does not need to consider this issue in the reply LS - 1st round discussion summary: - Three companies support Option 2 while other companies have different view. Tentative agreements: N/A Candidate options: Recommendations for 2nd round: - Further discuss if RAN4 share TX timing related information with RAN1 based on Option 2 and Option 3. If consensus could not be achieved, suggest not to cover this issue in the reply LS since this issue is not mentioned in RAN1 LS. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| R4-2103290 | Reply LS on Beam switching gaps for Multi-TRP UL transmission | Samsung |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round

Issue 2-1: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched in the same panel

- Proposals:

  - Option 1: 5us

  - Option 2: 5us if the beams are known; In case the TX beam is unknown, the transient time needs to account for beam refinement time

- Recommendation for 2nd round

  - Suggest to agree Option 1 based on majority view

  - If necessary, further discuss RAN4 understanding on beam known/unknown status and its relationship with this issue

| Company | Comments |
| --- | --- |
| Nokia | If the recommendation means that a reply LS simply says 5us and does not include knonw/unknonw things, we are ok with the recommendation. |
| Samsung | Option 1. Beam known/unknown status affects beam switching time for the same TRP. It is not applicable for beam switching between TRPs which is RAN1 concerned scenario. |
| Qualcomm | Option 1 if the beams are known and the TRP panels are powered up for FR2. Option 2 if beams are unknown. |
| Apple | Option 2. RAN1 LS to RAN4 mentioned beams may be known or unknown. Hence, we support 5us if beams are known. Also, RAN1 requested for range of transient time to be considered, hence we could provide transient time for known and unknown. |
| Huawei | Option 1. We are also fine to have further discussion on the impact of unknown/known beam status to align the understanding in RAN4. |
| MediaTek | We are fine to both options. To progress, we can go with option2 which mentions that 5us is only applicable in known beam case. It is up to RAN1 on how to proceed based on RAN4’s feedback. |
| vivo | Option 1 is simple and it is enough. Known and unknown states are defined in RAN4 but not in RAN1. It is not preferred to only explicitly write known or unknown state in the LS, which may cause confusion in RAN1. Suggest to only say 5us which provides enough information. |

Issue 2-2: ranges of the transient period(s) between two PUCCH/PUSCH TDMed repetitions with different UL beams switched cross panels

- Proposals:

  - Option 1: 5us

  - Option 2: the transient period would be determined by the beam switching time which is reported as UE capability, where 224 and 336 symbol length are used for cross panel switching

- Recommendation for 2nd round

  - Further discuss whether it is necessary to distinguish panel activation status

| Company | Comments |
| --- | --- |
| Nokia | Option 1. This does not mean a UE’s both panels are always on. The UE turns on the non-active panel just before the beam switching. |
| Samsung | It is not clear if RAN1 assume UE has already turned on the non-active panel before the beam switching starts. If both panels are not always on, it is better to distinguish the transient period depends on panel active/non-active status, i.e., if panel is active, go with Option 1; if panel is non-active, go with Option 2. If companies have strong concern on the proposal to distinguish by panel activation/non-activation status, we are also fine with Option 2 at this stage rather than Option 1. |
| Qualcomm | Option 1 where the panels are on well ahead of switching. However, if panels are not switched on in time then additional time is required for power up. |
| Apple | 5us for known if panel are already activated, otherwise panel activation time need to be considered. |
| Huawei | Option 2. Even in RAN1 there is no agreement on the panel activation/non-activation status. Also when we have discussion in Rel-15 for the beam switching time, there is no agreement that the switching time, i.e. 224, 336 are defined for non-active case. Our preference is to send the existing agreement to RAN1, and once there is clear conclusion on the panel status in RAN1, we can further respond based on their further request. |
| MediaTek | If no simple conclusion can be achieved, perhaps we can provide 2 values to RAN1 for already-activated panel and inactive panel. It is still up to RAN1 to decide how to take these values in their future work. |
| vivo | The use case discussed in RAN1 should be the case where both panels are activated. Therefore, we support option 1. Anyway suggestions from Qualcomm and Apple are also acceptable. |

Issue 2-3: whether there are additional RAN4 considerations that RAN1 shall account for a switching gap (blanked symbol(s)) between two PUCCH/PUSCH TDMed repetitions (with different UL beams)

- Proposals:

  - Solution-1: When the transient period is larger than CP, the switching gap is needed to avoid the performance degradation

  - Solution-2: It depends on RAN1 decision whether applying blanked symbol(s)

  - Solution-3: RAN4 has no evaluation yet. Share with RAN1 current RAN4 requirement for information.

- Recommendation for 2nd round

  - Further discuss in 2nd round based on above Solution-1/2/3 or combinations of them

  - Focus on the “additional” RAN4 consideration

| Company | Comments |
| --- | --- |
| Nokia | Solution-2: RAN4 just focuses on making a consensus on the required time for beam switching at least at this moment. |
| Samsung | We see no conflicts among these 3 solutions. Given RAN1 is asking for “additional” RAN4 consideration, we agree with Nokia that Solution-2 should be the essential answer for this issue. So we support Solution-2, but also fine to capture Solution-1 and Solution-3 for information to RAN1. |
| Qualcomm | Currently, we see only solution 3 as being defined in RAN4. Once the switching scenario is clearer (i.e. whether the beams are known or not and TRPs are on ahead of time or not) then RAN4 can evaluate whether switching gaps are needed and how long they should be for other scenarios currently not defined in RAN4. |
| Apple | Based on the moderator’s recommendation to keep in mind the “additional” RAN4 consideration, we support Solution 3. As RAN4 could have a better understanding once RAN1’s design is complete and RAN4 can evaluate better. |
| Huawei | Solution-2. We think that it is up to RAN1’s decision based on the info provided by RAN4. Sharing the current RAN4 requirement is ok, but we don’t think that further evaluation is needed in RAN4, unless there are clear demand from RAN1. |
| MediaTek | We think RAN4 can try to reply RAN1 with a hybrid from Solution-1 and 2, e.g., When the transient period is larger than CP, performance degradation is expected. But it depends on RAN1 decision whether applying blanked symbol(s). |
| vivo | It would be good if some additional information about this transient period can be provided to RAN1. Therefore we support solution-1 or solution-3. |

Issue 2-4: whether different TX timing is considered for different UL beam transmissions

- Proposals:

  - Option 1: ask RAN1 what the TX timing assumption is for different UL beam transmissions.

  - Option 2: tell RAN1 the same UL timing is assumed for different UL beam transmissions in current RAN4 requirements.

  - Option 3: It is better to check if there have been discussion on this in RRM room first. Then, we can discuss what information we are going to share with RAN1

  - Option 4: RAN4 does not need to consider this issue in the reply LS

- Recommendation for 2nd round

  - Further discuss if RAN4 share TX timing related information with RAN1 based on Option 2 and Option 3. If consensus could not be achieved, suggest not to cover this issue in the reply LS since this issue is not mentioned in RAN1 LS.

| Company | Comments |
| --- | --- |
| Nokia | We are fine with either of the recommendation or option 2. |
| Samsung | We support Option 2, it is basis for the answers to other questions from RAN1 and it is beneficial for setting up common understanding between work groups and avoiding misunderstanding. Anyway, we are also fine with the moderator recommendation. |
| Qualcomm | We support option 2. We are also agreeable with the moderator recommendation that if consensus cannot be achieved on this issue that the LS reply not be delayed since it was not a question asked of RAN4 in the RAN1 LS. |
| Apple | We support option 1 and option 2. We don’t think they are mutually exclusive. RAN4’s reply could be based on same UL timing assumption and also ask RAN1 on further clarification if they assume same or different UL timing. Regarding option 2, we don’t have any requirements in RAN4 for different UL beam transmissions with different timing assumptions. |
| Huawei | Option 4. Since this issue is not asked by RAN1 by their original LS, it may cause ambiguity by this issue. In our understanding, RAN1 already had some agreements related to UL timing. As this issue is more relevant to RRM, we are also open to have further discussion in RRM session. But before any conclusion reached, we don’t think the issue needs to be mentioned in the reply LS. |
| MediaTek | We disagree with Option 2. Support Option 4 because the intention to inform RAN1 about this issue is not clear to us. |
| vivo | Fine with option 2/4. Also fine with moderator recommendation. |

Summary on 2nd round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

During 2nd round discussion, different views are still shown though more convergence is emerging, especially on the beam spatial filter known/unknown issue, cross panel beam switch issue (e.g. panel active/non-active issue) and TX timing issue. On the other hand, companies show the willingness of further analysis on the open issues. So in the reply LS, companies have compromised to reply RAN1 LS based on the most solid consensus.

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103290 | Reply LS on Beam switching gaps for Multi-TRP UL transmission is recommended to be approved |