3GPP RAN WG4 Meeting #98-e		R4-2103567

Electronic meeting, 25th Jan – 5th Feb 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on EN-DC NR SpCell in FR1 and 2 NR SCells in FR2 for Dormant SCell switch via DCI 2_6 within and after 3 OFDM symbols |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Perf |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, SCell dormancy is introduced, and its corresponding core requirements need to be verified in EN-DC NR FR1 SpCell and 2 NR FR2 dormant SCells scenario when the dormant BWPs are switched by DCI 2_6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a test case of EN-DC DCI 2_6 based Domant BWP Switch on Multiple NR FR2 SCells |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The dormant BWP switching delay and interruption requirements upon the BWP switching and during CSI/RRM measurements on dormant SCell might not be fully guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5.x.y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.5.5.x.y	EN-DC DCI 2_6 based Domant BWP Switch on Multiple NR FR2 SCells

##### A.5.5.x.y.1	Test Purpose and Environment

The purpose of this test is to verify the NR SCell dormant BWP switch delay requirement defined in clause 8.6.2A.1, interruption requirements due to the NR SCell dormant BWP switch defined in clause 8.2.1.2.15.1 for NR victim cells and in clasue 7.32.2.14.1 of TS36.133 for E-UTRA victim cell, respectively, and interruption requirements due to CSI and RRM measurements on the NR dormant SCells defined in clasues 8.2.1.2.15.2 and 8.2.1.2.15.3 for NR victim cells and in clause 7.32.2.14.2 of TS36.133 for E-UTRA victim cell, respectively. Supported test configurations are shown in Table Table A.5.5.x.y.1-1.

The general test parameters are given in Table A.5.5.x.y.1-2, and NR cell specific test parameters are given in Table A.5.5.x.y.1-3 and Table A.5.5.x.y.1-4 below. And the E-UTRAN cell specific test parameters can refer to Table A.3.7.2.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1), one NR FR1 PSCell (Cell 2), and three NR FR2 SCells (Cell 3-5) as given in Table A.5.5.x.y.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell and SCell are specified in Table A.5.5.x.y.1-3 and Table A.5.5.x.y.1-4 below.

PDCCHs indicating new transmissions shall be sent continuously on PCell (Cell 1), PSCell (Cell 2), and SCell (Cell 5) to ensure that the UE will have ACK/NACK sending except the time before T1 and during T3. PDCCHs indicating new transmissions shall be sent continuously on SCells (Cell 3,4) to ensure that the UE would have ACK/NACK sending except for the time duration when BWP is switching on the cells and the time duration of when active BWP of the cell is dormant.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), Cell 2 (PSCell) on radio channel 2 (PSCC), and Cell 3-5 (SCells) on radio channels 3-5 (SCCs), respectively.

-	UE is configured with 2 different UE-specific downlink BWPs for Cell 3 and Cell 4, BWP-1 and BWP-2. BWP-1 and BWP-2 always include bandwidth of the initial DL BWP and SSB. Here, BWP-2 on Cell 3 and Cell 4 is configured as dormant BWP.

-	UE is configured with 1 UE-specific downlink BWP the same as initial BWP for Cell 3 and Cell 4.

-	UE is indicated in firstActiveDownlinkBWP-Id that the active DL BWP is BWP-1 in Cell 3 and Cell 4.

-	UE is configured with DRX.

-	UE is configured to monitor PDCCH for DCI format 2_6 from Cell 2 at ps-Offset before the start of onDuration. ps-Offset is selected to correspond to the dormancy switching time specified in clause 8.6.2A.

All cells have constant signal levels throughout the test.

The test consists of 3 successive time periods, with durations of T1, T2, T3, and T4, respectively.

During T1,

Time period T1 starts when a DCI format 2_6 command for Cell 3 and Cell 4 DL BWP switch to BWP-2, sent from the test equipment to the UE, is received at the UE side in PSCell’s slot # denoted i. The UE shall switch its bandwidth part from BWP-1 to BWP-2.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (i+ TMultipleBWPswitchDelay+X) as defined in clause 8.6.2A.2. The UE shall be continuously scheduled on the cell starting from the beginning of the DL slot right after slot (i+ TMultipleBWPswitchDelay+X).

The UE shall be able to receive PDSCH at the beginning of the DL slot right after SCell(Cell 5)’s DL slot (i+ TMultipleBWPswitchDelay+X) as defined in clause 8.6.2A.2. The UE shall be continuously scheduled on the cell starting from the beginning of the DL slot right after slot (i+ TMultipleBWPswitchDelay+X).

PCell(Cell 1) interruption due to dormant BWP switch on PSCell shall occur within the dormant BWP switch delay.

SCell(Cell 5) interruption due to dormant BWP switch on SCell(Cell 5) shall occur within the dormant BWP switch delay.

During T2,

Time period T2 starts when dormant BWP switch latency requirement test is completed. The test equipement shall schedule PDSCH every slot.

The UE shall be able to report ACK/NACK corresponding to the scheduled PDSCH to PSCell except for the allowed times as defined in clasues 8.2.1.2.15.2 and 8.2.1.2.15.3.

The UE shall be able to report ACK/NACK corresponding to the scheduled PDSCH to PCell except for the allowed times as defined in clause 7.32.2.14.2 of TS36.133.

During T3,

Time period T3 starts when interruption due to SSB based RRM measurement and CSI measurement requirements test is completed. Test equipment shall not transmit PDCCH, hence, the UE doesn’t monitor PDCCH except DCI format 2_6 based PDCCH.

During T4,

Time period T4 starts when a DCI format 2_6 command for Cell 3 and Cell 4 DL BWP switch to BWP-1, sent from the test equipment to the UE, is received at the UE side in PSCell’s slot # denoted j. The UE shall switch its bandwidth part from BWP-2 to BWP-1.

The UE shall be able to receive PDSCH at the beginning of the DL slot right after PSCell’s DL slot (j+ TMultipleBWPswitchDelay+X) as defined in clause 8.6.2A.2. The UE shall be continuously scheduled on the cell starting from the beginning of the DL slot right after slot (j+ TMultipleBWPswitchDelay+X).

The UE shall be able to receive PDSCH at the beginning of the DL slot right after all SCell’s (Cell 3,4,5) DL slot (j+ TMultipleBWPswitchDelay+X) as defined in clause 8.6.2A.2. The UE shall be continuously scheduled on the cells starting from the beginning of the DL slot right after slot (j+ TMultipleBWPswitchDelay+X).

PCell(Cell 1) interruption due to dormant BWP switch on PSCell shall occur within the dormant BWP switch delay.

SCell(Cell 5) interruption due to dormant BWP switch on SCell(Cell 5) shall occur within the dormant BWP switch delay.

Table A.5.5.x.y.1-1: Supported test configurations for EN-DC DCI 2_6 based Domant BWP Switch on Multiple NR FR2 SCells

| Config | Cell 1 | Cell 2 | Cell 3, Cell 4, Cell 5 | DCI 2_6 of Cell 2 |
| --- | --- | --- | --- | --- |
| 1 | LTE FDD | 15kHz SSB SCS, FDD | 120kHz SSB SCS, TDD | within 3 OFDM symbols |
| 2 | LTE FDD | 15kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | within 3 OFDM symbols |
| 3 | LTE FDD | 30kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | within 3 OFDM symbols |
| 4 | LTE TDD | 15kHz SSB SCS, FDD | 120kHz SSB SCS, TDD | within 3 OFDM symbols |
| 5 | LTE TDD | 15kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | within 3 OFDM symbols |
| 6 | LTE TDD | 30kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | within 3 OFDM symbols |
| 7 | LTE FDD | 15kHz SSB SCS, FDD | 120kHz SSB SCS, TDD | after 3 OFDM symbols |
| 8 | LTE FDD | 15kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | after 3 OFDM symbols |
| 9 | LTE FDD | 30kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | after 3 OFDM symbols |
| 10 | LTE TDD | 15kHz SSB SCS, FDD | 120kHz SSB SCS, TDD | after 3 OFDM symbols |
| 11 | LTE TDD | 15kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | after 3 OFDM symbols |
| 12 | LTE TDD | 30kHz SSB SCS, TDD | 120kHz SSB SCS, TDD | after 3 OFDM symbols |
| Note 1: 10 MHz bandwidth for Cell 2 with 15kHz SSB SCS. Note 2: 40 MHz bandwidth for Cell 2 with 30kHz SSB SCS. Note 3: 100 MHz bandwidth for Cell 3,4,5. Note 4: The UE is only required to be tested in one of the supported test configurations. |  |  |  |  |

Table A.5.5.x.y.1-2: General test parameters for EN-DC DCI 2_6 based Domant BWP Switch on Multiple NR FR2 SCells

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRAN carrier frequenciy is used. |
| NR RF Channel Number |  | 2,3,4,5 | Four NR radio channels are used for this test. RF channel number 2 is in FR 1 and RF channel numbers 3,4,5 are in a band where intra-band FR2 CA is allowed. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Active PSCell |  | Cell 2 | Primary SCG cell on NR RF channel number 2. |
| Configured activated SCell |  | Cell 3,4,5 | Configured activated secondary cell on NR RF channel numbers 3,4,5. |
| CP length |  | Normal |  |
| DRX |  | DRX.3 | As specified in clause A.3.3 |
| ps-Offset |  | Depending on UE capability | Monitoring of DCI 2_6 ahead of start of drx-onDurationTimer. Value of ps-Offset shall correspond to SCell dormancy switching time for switching of two SCells, as specified in clause 8.6.2A. Actual value depends on reported UE capabilities. |
| ps-WakeUp |  | true | Wake up for onDuration in case DCI format 2_6 is not detected. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell2 timing offset to cell1 | s | 3 |  |
| Cell3,4,5 timing offset to cell1 | s | 3 |  |
| Timing offset among cell3,4,5 | s | 0 |  |
| T1 | s | [0.2] | During this time cell 3,4 switch to dormancy from non-dormancy. |
| T2 | s | [10] | During this time cell 3,4 are dormant. |
| T3 | S | [0.1] | During this time PDCCH is not transmitted from all cells. |
| T4 | s | [0.2] | During this time cell 3,4 switch to non-dormancy from dormancy. |

Table A.5.5.x.y.1-3: Cell specific test parameters for EN-DC DCI 2_6 based Domant BWP Switch on Multiple NR FR2 SCells

| Parameter |  |  | Unit |  | Cell 2 |  | Cell 3,4 |  | Cell 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency range |  |  |  |  | FR1 |  | FR2 |  | FR2 |
| Duplex mode |  |  |  |  | FDD |  | TDD |  | TDD |
| TDD configuration | Config 1,4,7,10 |  |  |  | NA |  | TDDConf.3.1 |  | TDDConf.3.1 |
|  | Config 2,5, 8,11 |  |  |  | TDDConf.1.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |
|  | Config 3,6,9,12 |  |  |  | TDDConf.2.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |
| BWchannel | Config 1,2,4,5,7,8,10,11 |  | MHz |  | 10: NRB,c = 52 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |
|  | Config 3,6,9,12 |  | MHz |  | 40: NRB,c = 106 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |
| SSB Configuration | Config 1,2,4,5,7,8,10,11 |  |  |  | SSB.1 FR1 |  | SSB.1 FR2 |  | SSB.1 FR2 |
|  | Config 3,6,9,12 |  |  |  | SSB.2 FR1 |  | SSB.1 FR2 |  | SSB.1 FR2 |
| Downlink initial BWP Configuration |  |  |  |  | DLBWP.0.2 |  | DLBWP.0.2 |  | DLBWP.0.2 |
| Active (non-dormant) DL BWP-1 Configuration |  |  |  |  | NA |  | DLBWP.1.1 |  | NA |
| Active (dormant) DL BWP-2 Configuration |  |  |  |  | NA |  | DLBWP.1.1 |  | NS |
| Uplink initial BWP Configuration |  |  |  |  | ULBWP.0.2 |  | ULBWP.0.2 |  | ULBWP.0.2 |
| Active Uplink BWP-1 Configuration |  |  |  |  | NA |  | ULBWP.1.1 |  | NA |
| Active Uplink BWP-2 Configuration |  |  |  |  | NA |  | ULBWP.1.1 |  | NA |
| SMTC Configuration |  |  |  |  | SMTC.1 |  | SMTC.1 |  | SMTC.1 |
| TRS configuration | Config 1,4,7,10 |  |  |  | TRS.1.1 FDD |  | TRS.2.1 TDD |  | TRS.2.1 TDD |
|  | Config 2,5, 8,11 |  |  |  | TRS.1.1 TDD |  | TRS.2.1 TDD |  | TRS.2.1 TDD |
|  | Config 3,6,9,12 |  |  |  | TRS.1.2 TDD |  | TRS.2.1 TDD |  | TRS.2.1 TDD |
| TCI state |  |  |  |  | TCI.State.0 |  | TCI.State.0 |  | TCI.State.0 |
| PDSCH Reference measurement channel | Config 1,4,7,10 |  |  |  | SR.1.1 FDD |  | SR.3.1 TDD |  | SR.3.1 TDD |
|  | Config 2,5, 8,11 |  |  |  | SR.1.1 TDD |  | SR.3.1 TDD |  | SR.3.1 TDD |
|  | Config 3,6,9,12 |  |  |  | SR.2.1 TDD |  | SR.3.1 TDD |  | SR.3.1 TDD |
| RMSI CORESET Parameters | Config 1,4,7,10 |  |  |  | CR.1.1 FDD |  | CR.3.1 TDD |  | CR.3.1 TDD |
|  | Config 2,5, 8,11 |  |  |  | CR.1.1 TDD |  | CR.3.1 TDD |  | CR.3.1 TDD |
|  | Config 3,6,9,12 |  |  |  | CR.2.1 TDD |  | CR.3.1 TDD |  | CR.3.1 TDD |
| Dedicated CORESET Parameters for scheduling PDCCH | Config 1,4,7,10 |  |  |  | CCR.1.1 FDDNote1 |  | CCR.3.1 TDD |  | CCR.3.1 TDD |
|  | Config 2,5, 8,11 |  |  |  | CCR.1.1 TDDNote1 |  | CCR.3.1 TDD |  | CCR.3.1 TDD |
|  | Config 3,6,9,12 |  |  |  | CCR.2.1 TDDNote1 |  | CCR.3.1 TDD |  | CCR.3.1 TDD |
| Dedicated CORESET Parameters for DCI 2_6 | Config 1,4,7,10 |  |  |  | CCR.1.1 FDDNote1 |  | NA |  | NA |
|  | Config 2,5, 8,11 |  |  |  | CCR.1.1 TDDNote1 |  | NA |  | NA |
|  | Config 3,6,9,12 |  |  |  | CCR.2.1 TDDNote1 |  | NA |  | NA |
| CSI-RS configuration |  |  |  |  | NA |  | CSI-RS.3.1 TDD |  | NA |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB |  | 0 |  | 0 |  | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 2 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 2 |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  |  |  |  | N/A Link only, see clause A.3.7A |  | AWGN |  |
| Note 1: monitoringSymbolsWithinSlot of DCI 2_6 seaerch space is set to ‘00110000000000’ for Configuratoin 7,8,9,10,11,12 Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |  |

Table A.5.5.x.y.1-4: OTA related test parameters for EN-DC DCI 2_6 based Domant BWP Switch on Multiple NR FR2 SCells

| ParameterNote 6 | Unit | Cell 2 | Cell 3,4 | Cell 5 |
| --- | --- | --- | --- | --- |
| Angle of arrival configuration |  | N/A Link only, see clause A.3.7A | Setup 1 defined in clause A.3.15.1 |  |
| Assumption for UE beams Note 7 |  |  | Fine | Fine |
| Note1 | dBm/15kHzNote4 |  | [-111.7112] | [-111.7112] |
| Note1 | dBm/SCSNote3 |  | [-102.7103] | [-102.7103] |
|  | dB |  | [7] | [7] |
| SS-RSRPNote2 | dBm/SCS Note4 |  | [-95.785] | [-95.785] |
|  | dB |  | [718] | [718] |
| IoNote2 | dBm/95.04 MHz Note4 |  | [-65.956] | [-65.956] |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: All parameters apply for configuration 1 and 2 Note 7: Information about types of UE beam is given in B.2.1.3 and does not limit UE implementation or test system implementation. |  |  |  |  |

##### A.5.5.x.y.2	Test Requirements

During T1, the UE shall start to send the ACK for PSCell in the DL slot right after PSCell’s DL slot (i+ TMultipleBWPswitchDelay+X) as defined in clause 8.6.2A.2.

During T2, the UE shall transmit at least [98.5]% of ACK/NACK on NR PCell.

During T4, the UE shall start to send the ACK for PSCell in the DL slot right after PSCell’s DL slot (j+ TMultipleBWPswitchDelay+X) as defined in clause 8.6.2A.2.

The rate of correct events observed during repeated tests shall be at least 90%.

During T1, the start of the interruption of PCell and SCell (Cell 5) during dormant BWP switch on SCells (Cell 3,4) shall not happen outside the dormant BWP switch delay.

During T1, the start of the interruption of PCell and SCells (Cell 3,4,5) during dormant BWP switch on SCells (Cell 3,4) shall not happen outside the dormant BWP switch delay.

<End of Change 1>