3GPP TSG-RAN4 Meeting #98-e		R4-2103666

# Electronic Meeting, Jan 25-Feb 05, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1740 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RRM core requirements for band n262 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_47GHz_Band-Core |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To introduce band n262 for UE power class 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Band n262 (47 GHz) is introduced in a separate band group for UE power class 3. Note: RRM requirements for n262 for other UE power classes will be defined after corresponding UE RF requiremenrs are agreed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE supporting band n262 may not fulfil RRM requirements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGES----------------------------

### 3.5.3	NR operating bands in FR2

NR frequency bands grouping for FR2 is specified in Table 3.5.3-1.

Table 3.5.3-1: NR frequency band groups for FR2

| Group | Band group notation | Operating bands |
| --- | --- | --- |
| A | NR_TDD_FR2_A | n2571, n2581, n2611 |
| B | NR_TDD_FR2_B | n2574, n2584, n2614 |
| C | NR_TDD_FR2_C |  |
| D | NR_TDD_FR2_D |  |
| E | NR_TDD_FR2_E |  |
| F | NR_TDD_FR2_F | n2604 |
| G | NR_TDD_FR2_G | n2601 |
| H | NR_TDD_FR2_H |  |
| I | NR_TDD_FR2_I |  |
| J | NR_TDD_FR2_J |  |
| K | NR_TDD_FR2_K |  |
| L | NR_TDD_FR2_L | n2572, n2582, n2612 |
| M | NR_TDD_FR2_M |  |
| N | NR_TDD_FR2_N |  |
| O | NR_TDD_FR2_O |  |
| P | NR_TDD_FR2_P |  |
| Q | NR_TDD_FR2_Q |  |
| R | NR_TDD_FR2_R |  |
| S | NR_TDD_FR2_S |  |
| T | NR_TDD_FR2_T | n2573, n2583, n2613 |
| U | NR_TDD_FR2_U |  |
| V | NR_TDD_FR2_V |  |
| W | NR_TDD_FR2_W |  |
| X | NR_TDD_FR2_X |  |
| Y | NR_TDD_FR2_Y | n2603 |
| Z | NR_TDD_FR2_Y |  |
| AA | NR_TDD_FR2_AA | n2593 |
| AB | NR_TDD_FR2_AB |  |
| AC | NR_TDD_FR2_AC |  |
| AD | NR_TDD_FR2_AD |  |
| AE | NR_TDD_FR2_AE | n2623 |
| NOTE 1: UE power class 1. NOTE 2: UE power class 2. NOTE 3: UE power class 3. NOTE 4: UE power class 4. |  |  |

----------------------END OF CHANGES----------------------------