3GPP TSG-RAN WG4 Meeting # 98-e 															 R4-2103950

Electronic Meeting, January 25th – February 5th, 2021

Agenda item:			11.11.1

Source:	Moderator (Qualcomm)

Title:	Email discussion summary for [98][312] NR_Repeater_General

Document for:	Information

# Introduction

A new WI on the definition of RF repeaters was approved in RP-202927. This e-mail discussion will focus on the general issues regarding the work for developing the requirements for these devices.

The following topics are covered in this e-mail discussion

- 1st round:

  - work plan

  - Applicable bands

  - Requirements and specification handling

  - Handling of TDD repeaters

  - Bandwidth Configuration

  - Others

- 2nd round:　Discussion to continue on most of the topics to confirm agreements

# Topic #1: Work Plan

This section discusses the work plan proposed by the rapporteur.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102834 | Qualcomm | RAN4#98-e (This meeting) Agreement on work plan targeting RAN4 RF conclusion by RAN#95 and RAN4 RRM by RAN#96 Discussion of parameters to be specified Discussion on bands to be included Discussion on regulatory limits Discussion on which TS document(s) are needed Discussion on EMC RAN4#98-bis-e Discussion on parameters to be specified Agreement on bands to be included Discussion on regulatory limits Agreement on which TS document(s) are needed RRM topics TBD RAN4#99 Normative values CR into draft TSs Normative value CR for regulatory limits Draft TSs provided to RAN#93 for information RRM topics TBD RAN4#100 bis Normative values CR into draft TSs RRM topics TBD RAN4#100 bis CRs into draft technical standard(s) RRM topics TBD RAN4#101 CRs into draft technical standard(s) TSs to be provided in RAN#94 for approval RRM topics TBD RAN4#102 CRs into draft technical standard(s) RRM topics TBD RAN4#102 bis RRM topics TBD RAN4#103 RRM topics TBD |
| R4-2102108 | Ericsson | Observation 1: Some attention may need to be paid to the TDD conducted test definition and setup Observation 1: The OTA test setup requires at least two spatially separated test gear transmission/reception points around the DUT. Observation 2: Both the signal generator and DUT transmit in the test setup. Additional consideration is needed for emissions measurement. The conformance discussion will start at a later stage in the WI. The purpose of this contribution is to highlight that conformance testing for repeaters is not a trivial re-use of BS and UE test principles and that a sufficient number of meetings should be planned for the conformance phase. |

## Open issues summary

The work plan should be discussed and agreed in this meeting such that the work can be organized accordingly.

### Sub-topic 1-1

Need for RRM requirements

The proposed work plan contains also work on RRM requirements as the WID TU allocation, this is like a miss from the TU allocation spreadsheet.

Issue 1-1: RRM Work

- Is there any need for any RRM related work?

  - Option 1: No

  - Option 2: Yes

- Recommended WF

  - No

### Sub-topic 1-2

Overall work plan

Whether the work plan as proposed by the rapporteur, without the RRM part that is discussed separately, is agreeable or changes have to made. Especially, it should be discussed how to handle the conformance part.

Issue 1-2: Overall workplan

- Proposals

  - Option 1: Yes, agreeable as is

  - Option 2: No, changes are needed

- Recommended WF

  - Option 2

The conformance part needs to be reflected in the work plan, companies should provide inputs on how the work should be organized.

### Sub-topic 1-3

WID Revision

If the answer to Sub-topic 1-1 is no, should the WID be revised in upcoming plenary to eliminate baseband TUs?

Issue 1-3: WID Revision

- Proposals

  - Option 1: WID should be revised to remove baseband core TUs

  - Option 2: No, baseband core TUs should be kept

- Recommended WF

  - Option 1

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-2: Overall workplan The revised WID should include a plan for the conformance specifications; as we point out in our contribution this is not copy/paste and needs sufficient time. |
| Huawei | Issue 1-1: As this is RF only repeater there is no need for demodulation hence no need for RRM – recommended WF ok Issue 1-2: Few comments on workplan – channel BW’s are mentioned as RF parameter, existing repeaters do not really use the concept in the same way using “passband”, this should be discussed further. It seems a conformance requirement will be needed this should be planned. As existing repeater specs are not that big do we really need to split? For BS we currently split the conformance but all core are in 1 spec and this will be much smaller document. Issue 1-3: IS this not the same as 1-1? There should be no need for demod or BB. |
| ZTE | Issue 1-1: RRM and Dmod is not needed as this RF repeater. Issue 1-2: Few comments on workplan – As mentioned by Ericsson ad HW, for repeater OTA testing, we need more time to study it. Issue 1-3: option 1 to remove the baseband part. |
| CMCC | Sub-topic 1-1: RRM may be needed for some requirements, e.g. delay, UL timing. Sub-topic 1-2: option 2, conformance test should be added in the work plan. |
| CATT | Issue 1-1: RRM Work Agree with the recommended WF. Current WI assumes RF repeater which doesn’t have RRM function in our understanding. |  |
| Nokia, Nokia Shanghai Bell | Issue 1-1: RRM Work We support option 1. There should not be RRM works for this WI, however, in case RRM impact is identified later on, then this may be reconsidered. Issue 1-2: Overall workplan We agree that conformance part is missing in WID. Thus, it should be reflected in updated WID appropriate to include conducted and OTA tests. |  |
| MTK | Issue 1-1: RRM Work FFS It is not 100% clear whether RRM part is needed or not. Suggest to keep it for now. Issue 1-2: Overall workplan Option 2 (changes are needed) Some general aspects need to be discussed as well, but current WP does not mention these. Issue 1-3: WID Revision Same comment as Issue 1-1. Prefer to keep it for now. |  |
| Samsung | Issue 1-1: RRM Work According to the agreed WID, RRM and demo are not included in objective. As a RAN4 dedicated WI, only specification impact on RF as core part and EMC as perf part is addressed in WI. Issue 1-2: Overall workplan We agree with moderator summary and other companies’ comment that conformance testing aspect as perf part should be addressed in work plan as well. |
| QCOM | Issue 1-1 No RRM needed Issue 1-2 Changes are required to address conformance test Issue 1-3 Revise WID remove baseband core TUs |
| CommScope | Issue 1-1: RRM Work No RRM work is needed. Issue 1-2: Overall workplan Agree the revised WID should include a plan for the conformance specifications Issue 1-3: WID Revision The is no need for a baseband part. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Most companies agree that there is no need for any RRM work, however, there are a few opinions that some work might be needed. Tentative agreement: No need for RRM work Further discuss in the 2nd round if this can be agree, companies that commented that RRM work might be needed are invited to provide more arguments on what exactly would be needed |
| Sub-topic#1-2 | There seems to be consensus that current work plan needs to be revised along the lines of: - Include conformance testing and provide enough time because the OTA testing discussion will be somewhat complicated(new test setup needed) - Remove RRM work pending the discussion in Sub-topic#1-1 - Included general aspect like what is discussed under this thread and that are likely to continue for a few meetings given that there are some disagreements Moderator recommends to assign a tdoc for a work plan and continue the discussion in the 2nd round with the goal to approve it. |
| Sub-topic#1-3 | As for sub-topic#1-1, most companies agree that WID should be revised to remove the baseband TUs. The conclusion on this depends also on the 2nd round of sub-topic#1-1, discussion should continue in the 2nd round: Further discuss the needs for baseband TUs in the 2nd round |

---------------GTW note on Feb.1 ---------------

Agreements：

RRM is out of scope based on current WID.

The TU request will be updated to align with work plan and WID.

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| R4-2103880 | Work Plan for RF Repeaters | Qualcomm |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Discussion in the 2nd round to continue based on the document provided by Qualcomm

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103880 | Agreeable |

# Topic #2: Applicable Bands

This section discuss which bands will be applicable for repeaters.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100832 | CMCC | Proposal 3: NR repeater is suggested to be designed for all operating bands specified in TS 38.104 including both FR1 and FR2. |
| R4-2101041 | NTT Docomo | Observation 2: NR repeater is expected to be widely used for covering areas and filling the gap of areas in NR bands including corresponding LTE FDD and TDD bands. Proposal 1: RAN4 considers all of NR bands defined in TS 38.104 as NR repeater’s operating bands. |
| R4-2102580 | Huawei | Proposal 1: NR Repeater specification shall consider all the duplex modes considered in the NR work so far, i.e. FDD, TDD, SDL, SUL. |
| R4-2100635 | CommScope | 2.3 The NR repeater specification should include requirements for all NR operating bands (cf. TS 36.106 clause 5.5). The list of bands shall be prioritized to ensure that the most commonly used bands are included in the initial release of the specification. |

## Open issues summary

It should be discussed which bands should be applicable for the repeaters and whether there would be any priorities. Based on the input from several companies, there is a strong desire to have all the defined bands applicable.

### Sub-topic 2-1

Supported bands

Issue 2-1: Which bands should be applicable to Repeaters and in the scope of the current WI

- Proposals

  - Option 1: All bands defined so far

  - Option 2: Only some of the bands, criteria to choose/prioritise to be discussed

- Recommended WF

  - Option 1

Based on the inputs, Option 1 is recommended. Given the large operator interest, it would be very difficult to include only some bands in the scope or set some priorities. The proposal in R4-2102580 would be automatically covered within Option 1.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | In principle all bands is OK; specific bands could be ruled out if they have some specific issue. Do we assume that there is no such thing as a multi-band repeater ? Do we assume that the passband of a repeater is contiguous ? If the answer to either of those is yes then we need to consider multi-band/multi-carrier requirements… |
| Huawei | Issue 2-1: The requirements don’t really differ to much with bands, co-location should of course include all bands (as its a protection req). It doesn’t seem too much effort to include all bands. Non-consecutive channels are included in the existing repeater definition of passband, but multi-band is not. |
| ZTE | Fine with all bands, since repeater RF requirement should be band agnostic in most cases. Open to further discuss the multiband repeater. |
| CMCC | Considering NR repeater will be widely deployed, it would be better to design all the operating bands for NR repeater without any priority. |
| CATT | Agree that taking option 1 as a beginning is ok. |  |
| Docomo | Issue 2-1: We agree with recommended WF. |  |
| Nokia, Nokia Shanghai Bell | Issue 2-1: We support option 2. It should be concluded first which frequency ranges and duplex modes are part of the work. The current WID only states to “consider” FR1 (FDD and TDD) and FR2. It would be beneficial to aim to understand the complexity and implications of supporting all these cases first. When complexity is better understood for each case then it can be seen whether it is reasonable and feasible to develop requirements for all of them. |  |
| MTK | Suggest to start from all bands. Future adjustment can be done if we encounter any problem. |  |
| QCOM | Issue 2-1 all bands should be the baseline. |  |
| CommScope | All NR bands should be included; however, as noted the most commonly used bands should be prioritized. |  |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | The large majority of companies support the proposed WF to include all the bands, including 2 operators. Tentative agreement: Include all bands in the scope For the 2nd round, check if the above agreement can be confirmed. Discuss whether there is a need to prioritize some commonly used bands or not |

------------------------------GTW note on Feb.1 ----------------------

E///: We are ok to include all bands in the scope. Single band repeater or multi-band supporting?

Pivotal Commware: We agree with E///.  We support multi-band repeater. Prioritization on multi-band required.

Commonscope: We will support all the bands; we are interested on multi-band supporting repeater.

Verizon: If considering single band, which frequency range should be selected, FR1 or FR2?

CMCC: All the rel-16 operating bands should be included, for Rel-17 bands, we can consider later.

Ericsson: We are talking to introduce multi-band specific requirements similar as BS introduced. (Assuming with single PA to support multi-bands).

Nokia: In the WID, FDD and TDD bands, better to clarify.

Huawei: we should support on SUL/SDL band as well.

Nokia: FR1 FDD, TDD and FR2 TDD. SUL is not in the scope, it should be addressed in RAN-P.

QC: For single –band, multi-band supporting, we can further discuss.

CMCC: For duplex mode, we have SDL band in LTE specification. I think no company plan to exclude SUL/SDL bands, we should include them.

China Telecomm: We also support to include all the bands. For the bands, we think we can consider rel-16 bands, for bandwidth we should further discuss.

Agreements:

Include all bands in the scope:

-Take the existing Rel-16 bands in the specification as basis

- Further discuss the multi-bands supporting issue

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Round 2 – Issue 2-1: Confirm that all bands can be included in the scope.

| Company | Comments |
| --- | --- |
| AT&T | We support the all bands can be included in the scope of the WI. |
| Commscope | We agree with the recommended WF. |
| QCOM | We should be specific on which bands. There was some discussion on rel16, some more on SUL, and SDL, and then some comment on bands under development. We should get a list and stick with those. Bands under development should be excluded. |

Round 2 – Issue 2-2: Should there be any prioritization of bands?

| Company | Comments |
| --- | --- |
| AT&T | We don’t see a need to prioritize any bands at this stage. |
| FirstNet | We should include band n14 for public safety/first responder needs |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Based on the GTW discussion and the 2nd round comments, the following is agreeable:

Include all bands in the scope:

-Take the existing Rel-16 bands in the specification as basis

- Further discuss the multi-bands supporting issue

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #3: Requirements and Specification Handling

The scope of the work in terms of conducted and OTA requirements and how to handle the specification has to be clarified. Several papers provided proposals, one of the topics that is not clear is whether radiated requirements for FR1 will be handled or not.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2100375 | CATT | Proposal 1: No need to define NR repeater types, the requirement can be defined according to FR1 (conducted) and FR2 (radiated) frequency ranges. |
| R4-2100635 | CommScope | The NR repeater specification shall include requirements for repeaters with both conducted and radiated interfaces (external and internal antennas). |
| R4-2100832 | CMCC | Proposal 1: Two specifications are the baseline for NR repeater RF requirements, one for RF core requirements and the other for conformance parts, both including the conducted and radiated requirements just as how the IAB specifications have been categorized. |
| R4-2101963 | ZTE | Proposal 1: non-AAS based repeater should be prioritized for FR1 FDD NR based repeater; Proposal 2:use the requirements of TS 36.106 spec as starting point for FR1 FDD NR based repeater; Proposal 3: non-AAS based repeater should be prioritized for FR1 TDD NR based repeater |

## Open issues summary

Several contributions are discussing which specifications should be defined(conducted or radiated), how they should be handled(single spec vs. splitting in multiple specs). The main discussion point seems to be whether FR1 radiated requirements are needed or not, this will be treated in a separate sub-topic.

### Sub-topic 3-1

Conducted and Radiated Requirements

Issue 3-1: Conducted and Radiated Requirements

- Proposals

  - Option 1: Define both conducted requirements(for FR1) and radiated requirements (FR2)

  - Option 2: other proposals

- Recommended WF

  - Option 1

Considering how all NR RF requirements are defined, there is a clear need for both conducted(FR1) and radiated requirements (FR2)

### Sub-topic 3-2

Radiated Requirements for FR1

Whether there is a need for FR1 radiated requirements is not yet clear, this issue needs discussion

Issue 3-2: Need for Radiated Requirements in FR1

- Proposals

  - Option 1: Yes, radiated requirements are needed for FR1

  - Option 2: No, only conducted requirements are enough in the current WI

- Recommended WF

  - Option 1

AAS type of devices are not ruled out, deployment should be possible in the higher FR1 bands.

### Sub-topic 3-3

Specification handling

One of the issues raised is the spec handling, whether specifications should be split into conducted and radiated or not. Considering the WI just started, the moderator is proposing to focus on the RF core requirements for now and discuss the handling of the conformance specifications closer to the start of the performance part.

Issue 3-3: Handling of RF Core Specifications

- Proposals

  - Option 1: Single specification covering both conducted and radiated

  - Option 2: Separate specifications for conducted and radiated

- Recommended WF

  - Option 1

The RF core specifications defined so far for network nodes are using a single specification covering both.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1: Conducted and Radiated Requirements Agree with option 1 FR1 conducted and FR2 OTA. OTA is necessary for FR2; for FR1 there is no real need for OTA for repeaters. Issue 3-2: Need for Radiated Requirements in FR1 Option 2. A cost-effective RF repeater would not include a large AAS array which would not accommodate connectors in FR1. It is worth to consider that developing OTA requirements means developing conformance tests and MUs, and these tests and MUs are not the same as the OTA test/MU for a BS as explained in our contribution. It would be better to focus the effort in analyzing the test scenario on FR2 where OTA is unavoidable. |
| Huawei | Issue 3-1: Option 1 is ok Issue 3-2: The WI states the repeater does not perform adaptive beam forming towards the UE so effectively has fixed antennas, hence its unlikely an AAS would be useful. Without this its clearly easier to specify a conducted interface so for the moment we can concentrate on conducted for FR1 but such product defining issues should perhaps be specified in the WID |
| ZTE | Issue 3-1: Conducted and Radiated Requirements Agree with option 1 Issue 3-2: Need for Radiated Requirements in FR1 Agree with Option 2 Since no adaptive beamforming is needed for FR1 and FR2, then not sure whether we still need to have AAS based architecture. Of course, if operators have strong interest on that and can also manage the cost for repeater instead of IAB, then it should be also fine for us. |
| CMCC | Sub topic 3-1: we support option 1, at least defining conducted requirements for FR1 and radiated requirements for FR2 Sub topic 3-2: we support option 1, radiated requirements are needed for FR1. In some cases, FR1 NR repeater could only be tested by radiated requirements not conducted requirements. For example, donor BS is 1-O type and the requirements are tested only by radiated requirements. The repeater amplifies and forwards the received signal which could still only be tested by radiated requirement rather than conducted requirements. Sub topic 3-3: we suggest option1, only one specification covering both conducted and radiated requirements, the same as how the RF core requirements have been designed for other network nodes. |
| CATT | Issue 3-1: Conducted and Radiated Requirements Agree with the recommended WF. Issue 3-2: Need for Radiated Requirements in FR1 Option 2, repeater antenna design is different with BS, FR1 radiated requirements are not reasonable. Issue 2-1: Handling of RF Core Specifications Ok with the recommended WF. |  |
| Docomo | Issue 3-1: Conducted and Radiated Requirements OK with the recommended WF. Issue 3-3: Handling of RF Core Specifications OK with the recommended WF. |  |
| Nokia, Nokia Shanghai Bell | Issue 3-1: Conducted and Radiated Requirements As commented for issue 2-1 above: first we need to understand complexity and use cases. Issue 3-2: Need for Radiated Requirements in FR1 It might be beneficial to prioritize non-AAS repeaters for FR1 FDD and TDD and use it as starting point. Issue 3-3: Handling of RF Core Specifications We support option 1 to introduce only single core specification for conducted and radiated parts, as it was already done for both legacy BS NR and IAB core spec. |  |
| MTK | Issue 3-1: Conducted and Radiated Requirements Support Option 1 Issue 3-3: Handling of RF Core Specifications Support Option 1 |  |
| Samsung | Issue 3-2: Need for Radiated Requirements in FR1 The statement in WI scope on “It is assumed that the repeater does not perform adaptive beamforming towards the UE.” should be kept in mind during discussion. Based on this assumption it seems AAS type Repeater for FR1 should not be considered. |  |
| QCOM | Issue 3-1 We need at least conducted for FR1 and radiated for FR2 Issue 3-2 We need radiated requirements for FR1 AAS type. We should discuss whether there are presently any plans for deployment of AAS in FR1. This might help set priority. Issue 3-3 Single spec would be easier to develop and manage than separate. |  |
| CommScope | Issue 3-1: Conducted and Radiated Requirements There should be an Option 2 with conducted and radiated for both FR1 and FR2. FR2 repeaters could be directly connected to gNBs. Issue 3-2: Need for Radiated Requirements in FR1 Agree with Option 1 Issue 3-3: Handling of RF Core Specifications Agree with Option 1 (single spec for radiated and conducted) |  |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | Tentative agreements:Agree to have both conducted requirements(for FR1) and radiated requirements (FR2) Candidate options: Recommendations for 2nd round: Confirm the agreement |
| Sub-topic#3-2 | There is no consensus whether radiated requirements in FR1 should be in scope or not, this discussion should continue in the 2nd round. Candidate options: Same options as in the 1st round Recommendations for 2nd round: Continue the discussion, companies that are supportive of radiated requirements in FR1 should bring further arguments on how such repeaters can be implemented and use case |

| Sub-topic#3-2 | There is no consensus whether radiated requirements in FR1 should be in scope or not, this discussion should continue in the 2nd round. Candidate options: Same options as in the 1st round Recommendations for 2nd round: Continue the discussion, companies that are supportive of radiated requirements in FR1 should bring further arguments on how such repeaters can be implemented and use case |
| --- | --- |

| Sub-topic#3-3 | Tentative agreements:Introduce a single specification for RF core Recommendations for 2nd round: Confirm the agreement |
| --- | --- |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Round 2 – Issue 3-1: Confirm agreement to have both conducted requirements(for FR1) and radiated requirements (FR2)

| Ericsson | OK for us |
| --- | --- |
| Docomo | OK for us. |
| CMCC | OK for us |
| Nokia, Nokia Shanghai Bell | OK for us. |
| AT&T | Ok for us. |
| CommScope | OK for us, but to clarify, conducted or radiated shouldn’t be limited to FR1 or FR2. |
| QCOM | confirmed |

Round 2 – Issue 3-2: Need for radiated requirements in FR1

companies should bring further arguments on how such repeaters can be implemented and use case

| Ericsson | We do not see a clear motivation, since there will not be beamforming from the repeater, repeaters are meant to be low cost and there would not be a control mechanism for a large active array. Introducing OTA requirements implies the need to solve the OTA testing and derive MU/TT etc. for FR1 as well as FR2. So it would be preferable to focus on conducted for FR1, OTA for FR2. |
| --- | --- |
| Samsung | If between repeater to UE no adaptive beamforming assumed it’s questionable why for repeater to BS the active antenna system will be applied for this “simplest and most cost-effective” node. |
| CMCC | We need further check whether radiated requirements would be used in the future network deployment. It may be needed to reflect the spatial characteristics especially when the donor gNB is 1-O type. The efforts for FR1 radiated requirement is less and only scaling factor is needed. |
| Nokia, Nokia Shanghai Bell | We have similar view to Ericsson comment. We prefer to focus on conducted requirements for FR1. |
| AT&T | We are OK to take non-AAS repeaters for FR1 as the starting point but should not fully exclude the possibility of AAS repeaters for higher FR1 bands especially and this topic should be further studied and leave open the possibility of radiated requirements for FR1. |
| CommScope | The standard should not dictate whether a repeater uses integrated antennas/AAS or external antennas. Thus, radiated requirements shall be available for products which are launched with internal antennas, i.e. not having a conducted interface. For instance, an FR1 repeater may still have an internal antenna even if it doesn’t perform beamforming. Additionally, a FR2 repeater could have a direction connection to the base station. |

Round 2 – Issue 3-3: Confirm agreement: Introduce a single specification for RF core

| Docomo | OK for us. BS RF core specification has both of conducted and radiated requirements. |
| --- | --- |
| CMCC | OK for us |
| Nokia, Nokia Shanghai Bell | OK for us. |
| CommScope | OK for us |
| QCOM | Agreed |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Round 2 - Issue 3-1:

It is agreeable to have both conducted requirements and radiated requirements

Round 2 – Issue 3-2

There is no consensus whether radiated requirements in FR1 should be introduced or not, this topis should be discussed further

Round 2- Issue 3-3

It is agreeable to have a single specification for RF core

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: Handling of TDD Repeaters

TDD repeaters present several problems such as whether they have to be synchronized to the network, whether or not they have to be aware of the UL/DL configuration and how to handle dynamic TDD. These issues are discussed in this section.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102018 | Nokia | Observation 2: Discussion is needed on how to handle synchronization in case TDD repeaters are decided to be worked on. Proposal 1: Use cases and deployment scenarios need to be discussed and agreed to provide the starting point for requirement discussions. |
| R4-2100375 | CATT | Proposal 2: Transmitter off power needs to be defined for TDD repeaters, the same requirements as BS can be defined. Observation 1: It’s not easy to define transient period requirement for TDD repeater. Observation 2: TDD synchronization related requirements need more discussion. |
| R4-2100832 | CMCC | Proposal 2: at first, we should focus on the RF architecture discussion and clarify whether/how to distinguish different DL/UL terminologies at least for repeaters supporting TDD operating bands. |
| R4-2101156 | Mediatek Inc | Observation 1: NR repeater has no idea about the UE-specific information (e.g., SFI or DCI) that may overwrite the static UL/DL configuration in SIB. Proposal 1: RAN4 to discuss how to enable NR repeaters to get the SFI and scheduling DCI information for dynamic TDD deployments. Observation 2: How the repeater determines the starting time for UL transmission is not clear. Observation 3: How the repeater determines the starting time and duration for UL signal listening is not clear. Proposal 2: RAN4 to discuss how to the repeater determines the starting time for UL transmission as well as the starting time and duration for UL signal listening. |
| R4-2101963 | ZTE | Observation 1: for TDD NR based repeater, without explicit DL-UL pattern information or with static DL-UL pattern only at repeater, the deployment would be limited to certain scenarios; Proposal 4: for TDD NR based repeater, group delay introduced by repeater’s filter should be studied and clarified as this would impact DL-UL gap period and NCS configuration of PRACH channel. |
| R4-2102829 | Qualcomm | A TDD repeater can benefit from being able to switch direction matching the uplink and downlink slot structure used by the gNodeB. A repeater may be able to determine the subframe boundaries if the UL/DL pattern is fixed, however the uplink/downlink pattern can change with dynamic TDD. The fixed UL/DL repeater configuration would preclude network changes. An option is to include signalling from the gNodeB to the repeater, informing the repeater of the configuration. This allows flexibility in the network. It also allows higher gain TDD repeaters, additional coverage extension, and better end-to-end performance. Proposal 3: Companies should discussion whether signalling would be beneficial for the TDD repeater. |

## Open issues summary

Many papers are discussing the behaviour of repeaters in TDD bands, the need for synchronization and UL/DL configuration awareness. Other topics brought up are support for dynamic TDD and definition of requirements related to TDD such as Rx-Tx switching time, whether the repeater should be aware of the exact timing when UL starts.

### Sub-topic 4-1

Synchronization for TDD

It should be discussed whether the repeater has to be synchronized to the network and know the exact symbol timing.

Issue 4-1: Synchronization for TDD

- Proposals

  - Option 1: Repeater has to synchronize to the gNB timing

  - Option 2: Synchronization is not needed

- Recommended WF

  - Option 1

In order for the repeater to function correctly in a TDD network, it needs to synchronize to the gNB timing

### Sub-topic 4-2

UL/DL Configuration Awareness

Some companies raised the problem that besides synchronization, the repeater also has to be aware of the UL/DL configuration.dd

Issue 4-2: UL/DL Configuration Awareness

- Proposals

  - Option 1: Repeater needs to be aware of the UL/DL split

  - Option 2: Repeater does not need to be aware of the UL/DL split

- Recommended WF

  - Option 1

### Sub-topic 4-3

UL/DL Configuration Signaling

If the repeater should be aware of the UL/DL configuration, how will it acquire this information?

Issue 4-3: UL/DL Configuration Signaling

- Proposals

  - Option 1: Repeater can acquire the UL/DL Configuration by reading the cell broadcast information(e.g. SIB)

  - Option 2: Dedicated signaling would be needed to inform the repeater about the UL/DL configuration

- Recommended WF

  - TBA

As multiple options are possible, this point will require some discussion, companies are invited to provide their views or proposals.

### Sub-topic 4-4

Support for dynamic TDD

Support for dynamic TDD was brought up in multiple papers. While the static or semi-static UL/DL configuration can be acquired from the broadcast information, this will not enable support for dynamic TDD

Issue 4-4: Support for dynamic TDD

- Proposals

  - Option 1: Dynamic TDD should be supported

  - Option 2: There is no need to support dynamic TDD

- Recommended WF

  - Option 1

To enable deployment flexibility and forward compatibility, dynamic TDD should be supported

### Sub-topic 4-5

Requirements for TDD – Repeater Group Delay

Some companies brought up the need to introduce a requirement for group delay introduced by the repeater

Issue 4-5: Repeater Group Delay Requirement

- Proposals

  - Option 1: Group delay requirement is needed

  - Option 2: Group delay requirement is not needed

- Recommended WF

  - Option 1

Since the delay introduced by the repeater will impact the guard period needed for UL-DL switching, such requirement is needed

### Sub-topic 4-6

UL Timing

The issue whether the repeater should be aware of the exact UL timing (when to start amplifying signals in UL) was brought up and should be discussed.

Issue 4-6: UL Timing Knowledge

- Proposals

  - Option 1: Repeater needs to be aware of UL timing

  - Option 2: Repeater does not need to be aware of the exact UL timing

  - Option 3: Repeater can derive the UL timing autonomously

- Recommended WF

  - TBA

Companies are encouraged to provide their views. If Option 1 is chosen then proposals on how to derive the timing would be needed. If Option 2 or 3 is chosen then the behavior of the repeater should be explained.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 4-3: UL/DL Configuration Signaling Another option is for the repeater to be made aware of the DL/UL configuration by O&M configuration as opposed to signaling. Issue 4-4: Applying dynamic TDD assumes some potentially complex functionality in the repeater and also likely RAN1/2 changes. Also, in general dynamic TDD causes degradation to neighbor networks and so the scenarios in which it can be used are quite specialized (indoor, shielded from other networks). In the first step it may be better to focus on general repeaters. Issue 4-5: Repeater Group Delay Requirement To some extent the delay from the repeater to the UE can be managed by adjusting the UE timing advance, which will indirectly impact the repeater timing. However, there may be some issues handling UEs at different distances from the repeater and with overlapping DL RX / UL TX (if the GP is not large enough to absorb the group delay) that need investigation. Issue 4-6: UL Timing Knowledge As with 4-5; the network can control TA at the UE, which can indirectly control the timing of the repeater transmission. Problems will arise if the group delay and propagation delay become large enough that the guard periods between DL and UL cannot accommodate the additional delays. An analysis of delay budget may be needed. |
| Huawei | Issue 4-1: UL/DL switching is clearly required for TDD otherwise the system wouldn’t work as it cant transmit and receive at the same time in eh same direction on the same frequency. The current TDD UTRA spec includes UL/DL switching of gain sbut does not explicitly specify how the timing is derived (only that its specified wrt the BS). As such it can be considered left up to implementation. This seems a reasonable approach Issue 4-2: As above it clearly needs to be able to switch and hence needs to be aware, ideally it can do this from signals it receives from the BS or is the proposal the BS transmits new information to the repeater? If so that seems to be outside the WID Issue 4-3: This is a RAN4 WI to define the RF parameters, it seems a configurable repeater with specific info from the BS is outside that scope. Issue 4-4: If dynamic TDD can be supported within the scope we should investigate a way but the scope should not creep without readdressing the WID. The co-existence for dynamic TDD may also need o be addressed when using repeaters as the potential fr interference between cells is greater and will not have been examined in existing repeater analysis. Issue 4-5: Group delay is not specified for existing repeaters which implies it was either not needed or possible not thought about. Certainly we should study if it is necessary but its to early to definitively say yes or no. Issue 4-6: Again, existing TDD repeaters have a timing requirement but do not explicitly specify how it is derived. Clearly there should one or more conceivable ways of extracting the timing from the on air signals but this does not need to be explicitly specified. |
| ZTE | Issue 4-1: Synchronization for TDD Support the option 1 as NR is different from UTRA TDD and E-UTRA TDD where only cell specific timing is defined, therefore sync between donor and repeater could be left up to implementation. Issue 4-2: UL/DL Configuration Awareness Support the option 1. Issue 4-3: UL/DL Configuration Signaling This need some RAN plenary level discussion as this will impact other group. Issue 4-4: Support for dynamic TDD This need some RAN plenary level discussion as this will impact other group. Issue 4-5: Repeater Group Delay Requirement At least we need to have some common understanding on its impacts, then we could further discuss its necessity to define the requirements. Issue 4-6: UL Timing Knowledge It might be difficult to know per UE UL timing, if configured appropriately on DL-UL timing and then UL signal could be well captured we think. |
| CMCC | Sub topic 4-1: repeater should synchronize to the gNB timing. Sub topic 4-2: option 1, repeater needs to align its time reference with the known UL/DL split and determine to switch to UL or DL. Sub topic 4-3: we think option 1 is the baseline and option 2 is not excluded at current stage. If NR repeater is transparent to gNB and UE, one extra baseband processor would be required to decode the control information without any specification impact to other groups. Of cause the cost of this baseband processor should be managed. Besides, pre-defined configuration is also suggested as one cost-effective method without decoding any signaling. Of cause, some monitoring methods without any signaling could also identify the DL/UL by detecting signal strength and duration e.g. envelop detector. However the accuracy of this simple detection couldn’t be guaranteed considering the feasible deployment scenarios. It is excluded for NR repeater. Sub topic 4-4: we are OK with option 1, at least current stage, dynamic TDD should be included in the scope. Some co-existence issue may need to be analyzed considering the cross-link interference introduced by the dynamic TDD repeater. Sub topic 4-5: group delay requirement caused by repeater is not needed because group delay introduced by filter is dozens of nanoseconds, much less than the CP. Sub topic 4-6: repeater should know the exact UL timing. If we assume the repeater know the exact DL and UL configuration and is synchronized with gNB, then repeater would switch from DL to UL to amplify the UL signals during the DL-UL GP, before the beginning time of the first UL slot. Time advance between repeater and gNB could help repeater to determine the time advance before the beginning of UL slot to amplify the UL signal. For sub topic 4-3, 4-4, 4-6, at current stage, we suggest to maintain these features. Some analysis are necessary to study the impact to RAN4 specification and then we could decide whether to include them in the final scope. We should find the trade-off between the feasible function and cost-effective. Form our point of view, one baseband processor could help to enable more feasible functions if we could manage the implementation complexity. |
| CATT | Issue 4-1: Synchronization for TDD Option 1 direction is correct but the issue 4-5 is also related to this one. So we think the repeater-BS synchronization is not the same as BS-BS and IAB-parent. Issue 4-2: UL/DL Configuration Awareness and Issue 4-3: UL/DL Configuration Signaling Our current understanding is that it can be left to implementation. Issue 4-4: Support for dynamic TDD Dynamic may be a little difficult for some implementations, not sure if it’s defined as mandatory. Issue 4-5: Repeater Group Delay Requirement We don’t have clear view yet, but think it may be left for the deployment consideration. And traditional RF repeater’s Group delay should be very small. Issue 4-6: UL Timing Knowledge This is related to 4-2 and 4-3, we also think it can be left to implementation. |  |
| Docomo | Issue 4-3: UL/DL Configuration Signaling Option 2 may be out of the WI scope. If RAN4 does not adapt Dynamic TDD, then Option 1 will be preferable. Issue 4-4: Support for dynamic TDD As mentioned by Ericsson, it may be better to focus on simple repeater. Issue 4-5: Repeater Group Delay Requirement OK with recommended WF. |  |
| Nokia, Nokia Shanghai Bell | Issue 4-1: Synchronization for TDD We think that further discussion is needed on how to handle synchronization in case TDD repeaters are decided to be worked on. In case TDD repeater is not properly synchronized, there is a potential for harmful interference in the network. As the work item is contained to cover only RAN4, it is not possible to specify synchronization in PHY-layer. Therefore, a question remains whether synchronization requirements need to be developed in RAN4, or whether this is left purely for implementation. When this is discussed, it is important consider the possible negative impacts caused by synchronization errors. Issue 4-2: UL/DL Configuration Awareness Issue 4-3: UL/DL Configuration Signaling and Issue 4-4: Support for dynamic TDD: Discussion configuration awareness, signalling and support of dynamic TDD pattern changes starts to look like we are returning back to the smart repeaters which were scoped out of the WI. Therefore, RAN4 should aim at the simplest possible solutions. As the WID is contained within RAN4, additional signalling solutions do not appear feasible. It should be evaluated how accurately the repeater can synchronize with the fixed TDD pattern, and whether this level of accuracy makes TDD operation attractive e.g. from interference point of view. At least for now dynamic TDD does not appear easily feasible for simple analog repeater. Issue 4-5: Repeater Group Delay Requirement Does group delay here mean the potential processing delay of the repeater? It seems reasonable that there is a limit for the delay to avoid L1-changes. If this is achieved, then gNB will just configure TA accordingly for the UEs served via the repeater. Issue 4-6: UL Timing Knowledge It seems signalling exact UL timing is not possible as the work is purely contained within RAN4. Before agreeing to specify requirements, it would be good to understand the interference potential from misaligned UL timing. The impact of difference in UL timing can possibly be mitigated if only the repeater signal is being received by the gNB, though imposing such restriction may not be feasible. |  |
| MTK | Issue 4-1: Synchronization for TDD Support Option 1. To be more specific, the repeater is synchronized to gNB DL timing. We also need to discuss whether to have a requirement for timing synchronization Issue 4-2: UL/DL Configuration Awareness Support Option 1. Issue 4-3: UL/DL Configuration Signaling Support both Options. At least the repeater can acquire the UL\DL config from SIB. But if the network wants to later overwrite the SIB information to a UE through SFI or DCI, then we need Option 2 to make it work. Issue 4-4: Support for dynamic TDD Support Option 1. Since the repeater is typically used in the area of coverage hole, we believe the issue of co-existence with neighbouring cells is not that critical. Issue 4-5: Repeater Group Delay Requirement Support Option 1 (needed). Probably we may also need different requirements for FR1 and FR2. Issue 4-6: UL Timing Knowledge Support Option 1. Timing advance is anyway needed for UL forwarding. Otherwise, the repeater may forward only partial slots to gNB. How repeater knows the starting time of UL forwarding needs some more study. Same time, since different UEs also have different timing advance value, the repeater also need to know when to start listen to UE’s UL signals. |  |
| Samsung | It should be clarified that this WI focuses on RF impact purely within RAN4 to define cost efficiency and simple solution on repeater to improve the coverage. The issues raised under this topic with implication on other working group(s) which are clearly out of scope for WI objective should be left for non-standardized solution. The discussion should stick to WID as agreed in RAN-P meeting. |  |
| QCOM | Issue 4-1 Option 1 - It should sync. Sync is part of need for UL/DL split issue below. Issue 4-2 Option 1 - Repeater needs to be aware of UL/DL for stability, higher gain, and extened range. Issue 4-3 Option 2 - Some signalling can be used allowing some additional benefit such as configuring the repeater op frequency range under network control. Issue 4-4 Option 1 - Dynamic TDD should be supported. This would allow operators to deploy dynamic TDD in as system that includes repeaters. Issue 4-5 At this time some analysis is needed to determine whether or not we need group delay requirement. Issue 4-6: Option 3: UL at the repeater will start immediately after DL ends. |  |
| CommScope | Issue 4-1: Synchronization for TDD Support option 2, often a repeater must synchronize with the gNB to operate properly; however, if there is adequate TX/RX isolation then switching is not needed. Issue 4-2: UL/DL Configuration Awareness Support option 2. A switching repeater must be aware of the DL/UL period and duty cycle. This can be manually configured, detected off the air, or perhaps communicated via a side channel. How this information is obtained is outside the scope of this WI. However, again a non-switching repeater does not need to know the UL/DL configuration. Issue 4-3: UL/DL Configuration Signalling At a minimum option 1 can and should be supported. Option 2 would be a good extension, and could enable dynamic TDD, but option 1 is sufficient for static TDD. How this information is obtained is outside the scope of this WI. Issue 4-4: Support for dynamic TDD Option 2 should be the minimum requirement because it can be implemented without impact on other groups. Support for dynamic TDD (option 1) will cause extra effort and coordination with other groups, and would likely delay the completion of this WI. Issue 4-5: Repeater Group Delay Requirement We support Option 2. There is no need to specify a maximum group delay limit in the NR repeater specification. gNBs can be configured to accommodate repeaters with different delays, and the maximum allowable group delay will differ from installation to installation. Issue 4-6: UL Timing Knowledge We support Option 3, the repeater can derive UL timing autonomously from the SIB messages for the static TDD case (along with the DL timing) for use in a switching repeater (it should be noted that a non-switching repeater with adequate TX/RX isolation does not need to know the UL configuration). The DL Slots/symbols, UL slots/symbols, and TDD periodicity can be decoded or configured. Using the SSB as a reference point, the repeater can determine the start/stop time of the DL, UL and GP. Dynamic TDD would likely require other means to anticipate the TDD pattern, but this is a secondary priority as previously mentioned. Also, the repeater only really needs to know the DL pattern, as everything else can be regarded as UL. |  |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | Tentative agreements: Repeater has to synchronize to the gNB timing Candidate options: Recommendations for 2nd round:Capture the agreement in a WF |
| Sub-topic#4-2 | Tentative agreements: Repeater should be aware of the UL/DL split Candidate options: Recommendations for 2nd round: Capture the agreement in a WF |
| Sub-topic#4-3 | Companies agreed in the previous sub-topic that a TDD repeater should be aware of the UL/DL split, however, there is no consensus on how this can be achieved. Discussion on possible solutions should continue in the 2nd round: Candidate options: Multiple options were proposed by different companies, should be further discussed: - Repeater acquires UL/DL configuration from the cell broadcast - Dedicated signaling to inform repeater is introduced - Left to implementation - Configuration by O&M Recommendations for 2nd round: continue the discussion in the WF based on the proposed options |
| Sub-topic#4-4 | No consensus reached on this topic, this requires further study Recommendations for 2nd round: Continue the discussion, companies are invited to provide more arguments/proposals on the need and how to support dynamic TDD |
| Sub-topic#4-5 | There is no consensus whether the requirement is needed or not, some companies commented that further study is needed to understand the impact on the system. Recommendations for 2nd round: Continue the discussion, trying to identify what should be studied/analyzed to conclude on this |
| Sub-topic#4-6 | There is no consensus on this topic, discussion should continue to try to capture possible options, pros/cons and analysis on how to choose in the future. Companies seem to agree that repeater needs to know when to switch between UL and DL(implicitly some knowledge of UL timing is needed). Candidate options: - Repeater can derive the UL timing autonomously - Some mechanism needs to be standardized Recommendations for 2nd round: capture options and what should be further studied in the WF |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| R4-2103881 | WF on TDD Repeaters | CMCC |

--------------------GTW note Feb.1 -------------------

Discussion on GTW session of Feb.2rd

Pivotal Commware: We need to highlight the signaling is out of scope based on WID. The implementation solution to sync with gNB would be one possible solution.

CS: We think signaling not needed, the sync-with gNB can be implementation basis.

E///: The signaling is out of scope since this is pure RAN4 WI. We can rely on existing sync signaling and the repeater can be configured. When derive requirements, we should have a clear assumption.

Huawei: We should still stick the scope in RAN4 without any new signaling. We can leave it to implementation.

MTK: For loose requirements, we can rely on SSB decoding, if we want to have tighter requirements, additional signaling can be considered. We need to have DL/UL transmission separately with boundary.

CMCC：The repeater should sync with gNB DL. The remaining question whether dedicated RAN4 requirements needed or not? We think existing signaling can be used.

ZTE: Similar view as E///.

Samsung: RRM and baseband is out of scope for this WI, anything out of exiting WID should be further addressed in RAN-P.

CATT: Repeater sync to gNB different the sync approach among gNB-gNB; repeater just fwd. the signals from UE and BS. Repeater just need to turn on correct channel in corresponding time slots.

QC: we would like to discuss step by step, and further discuss how the sync achieved.

Nokia: I agree with CATT, it’s better to have common understanding on sync meaning for repeater.

Pivotal Commware: Sync with gNB should we define in implicit or explicit way?  The signaling with gNB which is out of WID scope.

Agreements:

RAN4 specify RF requirements based on the assumptions of TDD repeater has to synchronize to the gNB for TDD DL-UL pattern

- FFS whether dedicated requirements need to be specified for the sync with gNB

- FFS the exact methods to achieve sync with gNB in repeater side, implementation based on solution not excluded;

- RAN4 should respect the WID scope, to focus the discussion which aligned with the WID

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Discussion in the 2nd round should be based on the WF.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2103881 | Agreeable. The agreements from GTW are also captured in the WF |

# Topic #5: Bandwidth Configuration

NR offers a lot of flexibility for the channel bandwidth configuration including configuring dedicated channel BWs to different UEs. Also, the channel  bandwidth broadcast by the gNB is not necessarily the actual channel bandwidth used by the gNB.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2101156 | Mediatek Inc | Observation 4: It is not clear whether an NR Repeater does not have the knowledge aboutis able to acquire the real CBW signaled through dedicated RRC message to UE. Proposal 3: RAN4 to discuss how an NR repeater gets the information to the actual CBW used by the gNB. |
| R4-2102829 | Qualcomm | One option the passband(s) is/are preconfigured during installation and is/are not a dynamically changeable. Another option is repeater passbands may be configured under network control. Proposal 2: Interested companies should discuss the method for passband configuration. |

## Open issues summary

The repeater should be configured to operate with a certain channel bandwidth, this could be pre-configured(manual configuration) or under network control

### Sub-topic 5-1

Channel Bandwidth Configuration

Issue 5-1: Channel Bandwidth Configuration

- Proposals

  - Option 1: Repeater channel bandwidth is pre-configured

  - Option 2: Repeater channel bandwidth is derived from the network

- Recommended WF

  - TBA

Companies are invited to provide their opinions and if Option 2 is preferred, what would be the mechanism used.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 5-1: Sub topic 5-2: …. Others: |
| Huawei | Issue 5-1: Within the scope of the current WID it is difficult to see how the repeater can be configured by the network. Based on existing repeaters the passband seems to be pre-configured. |
| ZTE | Issue 5-1: Channel Bandwidth Configuration It should be pre-configured or customer based design. |
| CMCC | Sub topic 5-1: the channel bandwidth for NR repeater may be the same as donor gNB. To simplify implementation, option 1 is preferred. |
| CATT | Issue 5-1: Channel Bandwidth Configuration Our understanding is that repeater is a network similar node, not UE, so BW can be configured. |
| Docomo | Issue 5-1: Channel Bandwidth Configuration Based on existing repeaters the channel bandwidth is pre-configured. (Option 1) |
| MTK | Issue 5-1: Channel Bandwidth Configuration Support both Options. Pre-configured method should work as the baseline. Allowing over-the-air configuration from network can also be consider. |

| Samsung | As pointed by Huawei on work plan, the terminology should be clarified as passband rather than channel bandwidth. And we also agree this should be pre-configured. |
| --- | --- |

| QCOM | Issue 5-1 Some signaling would allow operator flexibility. We support Option 2 Network can broadcast separately the bandwidth to be used by repeaters |
| --- | --- |
| CommScope | Issue 5-1: Channel Bandwidth Configuration The CBW can be user configured during commissioning. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#5-1 | Companies agree that the baseline solution is to have the passband pre-configured. Whether a mechanism to configure the passband is needed can be further discussed if a need is shown. Tentative agreement: Baseline is that passband is pre-configured. FFS whether configurable passband should be supported Recommendations for 2nd round: Confirm the above agreement |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Confirm the above agreement: Baseline is that passband is pre-configured. FFS whether configurable passband should be supported

| Samsung | Agree with the baseline. According to current WID, mechanism such as signalling for donor BS to configure operation condition for repeater is not in scope. Hence not quite understand the discussion intention on further study part. Furthermore, capability signalling exchange seems not specified between repeater and donor BS as for LTE FDD repeater declaration is applied for operating band and frequency range (pass band) and bandwidth. |
| --- | --- |
| AT&T | Ok to confirm the agreement. |
| CommScope | Agreement confirmed. |
| QCOM | We agree |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Based on the comments, the following is agreeable:

Baseline is that passband is pre-configured. FFS whether configurable passband should be supported

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #6: Other Topics

Several other issues were brought up by different companies. The observations and proposals are summarized in Section 6.1.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2102018 | Nokia | Observation 3: Beamforming and antenna array assumptions need to be discussed for both the backhaul between parent BS and NR repeater as well as for the access link between repeater and the UE. Observation 5: Possible output power limitations need further clarifications Observation 6: Understanding on use cases and deployment scenarios is needed before it can be evaluated how much LTE repeater and IAB RF specifications can be leveraged. Proposal 1: Use cases and deployment scenarios need to be discussed and agreed to provide the starting point for requirement discussions. |
| R4-2101156 | Mediatek | Observation 5: The repeater may be only in the coverage of a specific DL Tx beam in FR2, but it is not clear whether the repeater has no idea is able to identify when the DL traffic will be transmitted along with that the Tx beam direction. Proposal 4: RAN4 to study how to inform repeater on which slot to forward or not to forward in FR2. |
| R4-2101963 | ZTE | Proposal 5: RF architecture for FR2 NR based repeater should also be discussed firstly. |
| R4-2102829 | Qualcomm | Proposal 1: RAN4 should discuss network control of repeater gain. Proposal 5: Donor side repeater antennas can use the same technologies and beam steering methods as a UE. Proposal 6: Service side repeater antennas may be fixed or autonomously adjustable. |

## Open issues summary

Some other issues than the ones discussed in the previous sections were also brought up. These are summarized below for further discussion

### Sub-topic 6-1

Deployment scenarios

R4-2102018 brought up the issue on discussing use cases and deployment scenarios, however, this issue was not brought up in any other paper. It should be clarified what exactly is not clear and whether there is any impact expected on the requirements to be defined

Issue 6-1: Deployment Scenarios

- Proposals

  - Option 1: There is no need for further discussion, the scenarios are clear.

  - Option 2: Further discussion is needed on which scenarios to be supported

- Recommended WF

  - TBA

Companies are invited to provide their views and potential impact to the future work and specification

### Sub-topic 6-2

RF Architecture and beam steering in FR2

Multiple papers brought up the need to discuss the RF architecture, especially for FR2. One issues is also whether the repeater could support beam steering or not

Issue 6-2: RF Architecture

- Proposals

  - Option 1: Repeater will support active antennas with some beam steering

  - Option 2: Repeater will have an antenna array with fixed gain and direction

  - Option 3: other RF architecture

- Recommended WF

  - TBA

Companies are invited to provide their comments on this issue and possible impact to the future work and RF requirements

### Sub-topic 6-3

Beam and slot awareness

In R4-2101156 the problem whether the repeater should be aware of which Gnb Tx beam it should forward and how to be made aware of the slot in which this beam is transmitted

Issue 6-3: Beam and slot awareness

- Proposals

  - Option 1: Does the repeater need to know which beam to forward and the slot in which it should forward it?

  - Option 2: Repeater does not need to be aware

  - Option 3: others

- Recommended WF

  - TBA

Companies are invited to provide input on this issue and possible solutions or explanations of the repeater behavior in FR2

### Sub-topic 6-4

Repetear Gain Control

R4-2102829 proposes to discuss the possibility for the network to control the gain of the repeater

Issue 6-4: TBA

- Proposals

  - Option 1: Network should be able to control the gain of the repeater

  - Option 2: no need for the gain to be controlled by the network

  - Option 3: Others

- Recommended WF

  - TBA

Companies are invited to provide their input on this proposals

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 6-1: Deployment Scenarios It is useful to clarify whether wide area, medium range or local area scenarios are being targeted (or all of these). If dynamic TDD is considered, it should be clarified what is the deployment scenario since in many scenarios dynamic TDD causes cross-operator interference. Issue 6-2: RF Architecture The WI assumes that there is not active beamforming between the repeater and the UE. The reason for this is to target developing “simple” repeater specifications before moving on to “smart” repeaters. This implies option 2 for the repeater – UE link. Issue 6-3: Beam and slot awareness Since the aim of the WI is to derive RF requirements for “simple” repeaters, this should be considered later as a part of smart repeaters to keep to the current WI scope. |
| Huawei | Issue 6-1: The use cases need to be clear, particularly for: TDD, scenarios where BS is beam sweeping, UE has beam steering, etc Issue 6-2: The WI specifically does not include active beam forming, some form of installation based static beamforming could be within scope however could complicate matters. As antenna isolation is extremely important for repeater operation currently the antenna selection and deployment is done on installation to ensure the required isolation. As each installation may have very different limitations this can only be done on sight and there are a number of ways it can be achieved. If the repeater antennas could change its nature then this could cause the system to oscillate and would be very difficult to test without knowing the exact installation scenario. Without some good reason to include such functionality we should probably avoid it. Issue 6-3: Again this would require specific information to be communicated to the repeater, this is outside the scope so option 2 is the target Issue 6-4: 2 issues here: 1) once again its outside the scope of the WI, 2) the repeater gain is optimised for the isolation which can be achieved. Clearly you could reduce this gain without issues but you could not increase it without risk of oscillation. |
| ZTE | Issue 6-1: Deployment Scenarios No strong opinions on that, in general, it should be supported for all scenarios we think. Issue 6-2: RF Architecture Support the option 2. Issue 6-3: Beam and slot awareness Support the option 2 which is aligned with WID. |
| CMCC | Sub topic 6-1: Option 1, no need to further discuss the scenarios. High-speed train is one typical deployment scenario to extend the coverage considering the ~30Db penetration loss of the high-speed train. From our point of view, all MA/LR/LA are the target scenario for future flexible deployment. Sub topic 6-2: we prefer option 2 that repeater will support active antennas with some beam steering for both FR1 and FR2 In high-speed train scenario, donor antennas would be deployed on the high-speed train to connect the donor BS. In this case the repeater is moving relative to the fixed deployed BS, fixed-directional or omnidirectional antenna would reduce the coverage range compared to feasible beam steering capability. As for how to adjust beam steering, it is up to repeater implementation. Sub topic 6-3: option 1. Repeater need to know which beam to forward and the slot in which it should forward it to reduce power consumption for FR2. Sub topic 6-4: option 2, gain is controlled by repeater itself. |
| CATT | Issue 6-1: Deployment Scenarios We think it may not be needed for FR1 because LTE study can be refered. FR2 repeater has not discussed before, but if the feature support is based on declaration, we need to understand what’s the expected discussion output for this issue. Issue 6-2: RF Architecture If the feature can be based on declaration, we may not need that discussion. Issue 6-3: Beam and slot awareness and 6-4 Need to discuss if they’re in the WI scope. |  |
| Docomo | Issue 6-1: Deployment Scenarios If RAN4 adapts new assumption such as repeater class (similar with BS class), then it is needs to be discussed each assumptions. Issue 6-2: RF Architecture Align with the scope of the WID, Option 2 is preferable. Issue 6-3: Beam and slot awareness Align with the scope of the WID, Option 2 is preferable. Issue 6-4: TBA Align with the scope of the WID, Option 2 is preferable. In order to control repeater’s gain by network, some additional information is needed. |  |
| Nokia, Nokia Shanghai Bell | Issue 6-1: Deployment Scenarios Option 2: Normally 3GPP RF requirements are based on a set of agreed deployment scenarios and use cases. In case of repeaters understanding the deployment scenarios will help to evaluate whether current NR and/or IAB RF requirements are possible to be re-used and whether there are need for multiple different classes of repeaters. Issue 6-2: RF Architecture Beam steering was ruled out by the WI. However, likely both omni and fixed antenna array options are possible. Discussion on the use cases and deployment scenarios would help. For example, if FR2 repeater has an antenna array, it can only serve rather narrow geographical area, potentially limiting the usefulness of the repeater. Issue 6-3: Beam and slot awareness In our understanding repeater will forward what it receives, without understanding the contents of the signal. Therefore, beam awareness does not appear feasible for simple repeater. Issue 6-4: Repeater gain control The need for gain control depends on use cases. If the channel conditions are expected to be vary greatly, then gain control could be useful. However, as the WI is contained within RAN4, there seems to be no way to specify such gain control. |  |
| MTK | Issue 6-2: RF Architecture Support Option 2 Issue 6-3: Beam and slot awareness Support Option 1. If the repeater knows which slot/beam to forward, it can reduce its power consumption as well as reduce unnecessary interference to other DL\UL link even in the same cell. Issue 6-4: FFS. We need to study more on how repeater determines its output power in order to know whether we need the control from network or not. |  |
| Samsung | Issue 6-2: RF Architecture According to WID objective, the standardization discussion can based on option 2. Issue 6-3: Beam and slot awareness According to WID objective, similar to RRM and demo, the mechanism of beam and slot awareness is out of scope. Issue 6-4: TBA According to WID objective, the network control is out of scope. |  |
| QCOM | Issue 6-1 We believe the use cases are clear, repeaters would act the same as a reflector. There is no need to do any study on the requirements. An RF repeater will simplify amplify what it receives on one port. Issue 6-2 This needs some discussions, baseline should be Option 2 as in the WID. Issue 6-3 Beam and slot awareness should be further discussed. Issue 6-4 Gain control can provide some flexibility to operators to adjust coverage of the repeaters. How this might be implemented should be discussed/ |  |
| CommScope | Issue 6-1: Deployment Scenarios It is useful to clarify deployment scenarios, but it is not necessary to define scenarios in the specification. Issue 6-2: RF Architecture Agree with option 1 to allow beam steering for FR2. Issue 6-3: Beam and slot awareness Agree with option 2, option 1 is outside scope of WI. Issue 6-4: TBA Agree with option 2, there is no need for gain control by the network. |  |

### CRs/TPs comments collection

Major close to finalize Wis and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going Wis, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#6-1 | Multiple companies commented that there is no need for further discussion on this topic while others have raised some questions whether different classes like for base station are needed and the extent that IAB discussion can be referenced. Discussion should continue based on some of the questions raised that are listed below. Whether dynamic TDD will be supported or not is still under discussion so it is too early to discuss interference scenarios for dynamic TDD. Candidate options: Recommendations for 2nd round: Continue the discussion in the second round on the following issues: - Should all MA/LR/LA be in scope or not? - Will repeater deployment scenarios be different than NR BS deployment scenarios or IAB deployment scenarios? - Any other clarifications or any other aspects need to be considered? |
| Sub-topic#6-2 | Most companies agree to Option 2. Some companies commented that some form of active antennas might be possible but they can be left to implementation Tentative agreements: Fixed gain and antenna pattern(direction) is the baseline. Continue the discussion on whether there would be any requirement impact if some form of active antenna is supported. Recommendations for 2nd round: Confirm above agreement(baseline) and continue the discussion on possible impact of active antennas on the RF requirement |
| Sub-topic#6-3 | Majority of companies commented that option 2(repeater does not need to be aware) should be agreed based on the current WID description. Companies Tentative agreements: Option 2 – repeater does not need to be aware Recommendations for 2nd round: Confirm above agreement. |
| Sub-topic#6-4 | Some companies commented that network controlled gain is not in the scope of the current WI. Tentative agreements: Network controlled gain is not in scope of the WI Recommendations for 2nd round: confirm above agreement |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Round 2 – Issue 6-1-1: Should all MA/LR/LA be in scope or not?

| Ericsson | Our view is that for FR1, the deployment scenarios should be differentiated. Maximum declarable power levels are determined for each scenario to enable proper co-existence. Also sensitivity may be relevant. |
| --- | --- |
| Docomo | It is OK to scope division classes if we need to separate classes for more appropriate requirements. In our NW, it is LTE FDD repeater but there are two use cases. The first is a low power repeater to cover indoor customers’ homes. The other is a large output power repeater to cover areas that are difficult to cover by Base Station, such as mountainous areas. When classifying, WA does not have an upper limit. |
| CATT | I tend to agree with the view from Docomo that maybe two sets of output power requirements can be defined for FR1 which is similar with LTE FDD repeater, there may be no need to define classes like BS. Repeater is different with BS that it can’t generate signals thus the output power after the large gain PA may not be as large like WA BS because the received power is small. So whether the repeater coverage can be as large as WA BS may need some discussion. However, the maximum output power for one set can be no upper limit to have some flexibility. For FR2, I don’t have clear view currently. The view that the output power may not be that large as WA BS is my guess. |
| CMCC | At least for DL, the deployment scenarios should be distinguished for appropriate requirements, e.g. power and emission requirements. It is suggested to reuse the BS-like approach to define the classes |
| Huawei | Its clear there are at least 2 deployment scenarios, but as the deployment of the repeater is probably not the same as the BS (where BS class is described by means of distance between BS and UE as well as additional parameters in TR’s , height above builds etc), using the same class names as BS may lead to confusion. |
| Nokia, Nokia Shanghai Bell | It seems that we will have different use cases, thus more classes are needed to be define. It will help to evaluate whether current NR and/or IAB RF requirements are possible to be re-used and whether there are need for multiple different classes of repeaters. |
| AT&T | Yes, different classes of repeaters may be needed for different use cases and deployment scenarios. |
| CommScope | LA/MR/WA classification is not needed. A two-tiered classification (low power, high power), as defined for LTE repeaters, is sufficient. |

| Samsung | We have no strong opinion. But “LA” in the question should be corrected as MR (Medium Range). |
| --- | --- |

Round 2 – Issue 6-1-2: Will repeater deployment scenarios be different than NR BS deployment scenarios or IAB deployment scenarios?

| Ericsson | The deployment scenarios are quite general so should be applicable in our understanding |
| --- | --- |
| CATT | Similar views with 6-1-1, the WA scenario may need some discussion. |
| CMCC | At least for DL, the same as BS deployment scenarios |
| Huawei | Same response as 6-1-1 |
| Nokia, Nokia Shanghai Bell | NR BS deployment scenarios should be consider as starting point. |
| AT&T | Possibly. Especially for FR1 TDD and FR2 bands, without “smart” repeater enhancements, deployment of repeaters may be more limited to targeted isolated coverage holes compared to more typical BS/IAB deployment scenarios which can more flexibility optimize the beamforming and coverage. Outdoor-to-indoor is another important use case for repeaters which may also be beneficial in reducing potential for adjacent band/network interference. |
| CommScope | Repeater deployment scenarios can be defined by two cases: a low power scenario for small area (primarily indoor) coverage, and a high-power scenario, for large area (primarily outdoor) coverage. |
| QCOM | This should be further discussed |

Round 2 – Issue 6-1-3: Any other clarifications or any other aspects need to be considered related to deployment scenarios?

| Ericsson | One thing that may need to be considered is how directional antennas are assumed to be and the extent to which this impacts co-existence properties. |
| --- | --- |
| CATT | For FR2, what’s the repeater’s beamforming behaviour? Fixed gain and fixed direction, fixed beamwidth, etc? Maybe the conclusion of 6-2-1 can answer my question. |
| CMCC | We should distinguish the DL and UL, only for DL the separate class is needed. |
| Nokia, Nokia Shanghai Bell | High power UL transmissions and fixed beam DL transmission may impact co-existence especially when there is no control over transmission power like there is for IAB. |

Round 2 – Issue 6-2-1: Fixed gain and antenna pattern(direction) is the baseline. Continue the discussion on whether there would be any requirement impact if some form of active antenna is supported.

| Ericsson | This may impact co-existence properties. Note that the WID assumes no active beamforming between repeater and UE. |
| --- | --- |
| Samsung | Agree with baseline. But the further discussion part may bring ambiguity that active antenna for repeater is in the WI scope which is not the case as commented in 1st round. |
| CMCC | The co-existence related requirements especially when repeaters are densely deployed. |
| Nokia, Nokia Shanghai Bell | As commented before beam steering was ruled out by the WI. We agree with baseline. |
| AT&T | At least this should be the baseline for FR1 FDD. More discussion may be needed for FR1 TDD/FR2. |
| CommScope | We agree with the baseline, although should allow beamsteering in RF2. |
| QCOM | We agree with continued discussion |

Round 2 – Issue 6-3-1: repeater does not need to be aware of which beam/slot to forward. This is outside the scope of current WI

| CMCC | We don’t have strong views. For FR2, we think repeater may need to be aware of which beam/slot to forward to reduce power consuming. However, if majority companies suggest this is outside the scope, we are OK. |
| --- | --- |
| AT&T | While signalling support for this aspect may not be included in the current WI, given this may pose challenges/restrictions on network operation in case of FR1 TDD/FR2 bands, the impact of this assumption should be further studied and understood when defining requirements. |

Round 2 – Issue 6-4-1: Network controlled gain is not in scope of the WI

| AT&T | At this stage we prefer for this point to remain FFS. |
| --- | --- |
| CommScope | Agree network controlled gain is out of scope. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Round 2 – Issue 6-1-1:

Companies agree that some differentiation of requirements based on some categorization(e.g. classes) is needed, however, the details are not clear yet. Most companies commented that 2 classes would be enough for FR1.

How many repeater classes are needed requires further study

Round 2 – Issue 6-1-2:

Most companies agree that the NR BS deployment scenarios can be taken as baseline, however, there is no consensus yet. There is also a view that less classes compared to NR BS are needed. Considering also the comments on Issue 6-1-1, this issue requires further discussion.

Round 2- Issue 6-1-3

Companies commented that further study is required on the beam patterns/gains for the repeaters and whether this will impact co-existence. Companies should provide more data in the upcoming meeting.

Round 2- Issue 6-2-1

Companies agree that fixed antenna gain pattern and gain is the baseline, hence, should be assumed.

Fixed antenna gain and pattern is assumed

Round 2 – Issue 6-3-1

Companies commented that current work item does not cover any sort of signaling support to make the repeater aware.

Repeater be made aware by the network about which beam/slot to forward is outside the scope

Round 2 – Issue 6-4-1

One company would like to further discuss the possibility of having a network controlled gain. AS this is not in the scope of the WI, companies can further contribute if there are ways to achieve this without any signaling. The following should be agreeable:

Network controlled gain is out of scope

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |