3GPP TSG-RAN WG4 Meeting#98-e	R4- 2103962

Online, 25th Jan – 5th Feb, 2021

Source: 	Qualcomm Incorporated

Title: 	Views on CSI Reporting test cases for eMIMO

Agenda item:	7.9.4.3

Document for:	Discussion

# Introduction

In WF [1], there were some open issues related to test metric and antenna correlation. In this paper, we discuss our views on those pending issues.

# Enhanced Type II Codebook

In WF [1], there were 3 options for the test point. As link adaptation algorithms try to keep the UE PDSCH BLER closer to 10%, we prefer to use the baseline of 90% test point. Therefore, we propose the following.

Proposal 1: Use 90% test point for defining Enhanced Type II PMI reporting test cases.

In WF [1], test metric is defined as the throughput ratio of following PMI and Random Type I PMI. We have below simulation results based on that test metric.

| Case | 90% Test Point |  |  |  | 70% Test Point |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Follow eType2 |  | Follow Type I |  | Follow eType2 |  | Follow Type I |  |
|  | SNR (dB) | Thpt Ratio | SNR (dB) | Thpt Ratio | SNR (dB) | Thpt Ratio | SNR (dB) | Thpt Ratio |
| FDD, 16x2, XP Medium | 8.18 | 6.16 | 13.86 | 2.19 | 6.65 | 10.27 | 11.04 | 2.34 |
| FDD, 16x4, XP Medium | 6.06 | 3.69 | 9.7 | 2.12 | 4.34 | 5.01 | 7.26 | 2.27 |
| TDD, 16x2, XP Medium | 9.34 | 4.18 | 14.37 | 2.05 | 7.59 | 6.08 | 12.9 | 1.84 |
| TDD, 16x4, XP Medium | 6.96 | 3.10 | 10.23 | 2.01 | 5.38 | 3.49 | 8.81 | 1.80 |
| FDD, 16x2, XP Custom Low | 8.22 | 4.01 | 12.14 | 2.16 | 6.38 | 5.75 | 9.43 | 2.41 |
| FDD, 16x4, XP Custom Low | 6.34 | 2.62 | 8.59 | 2.02 | 4.81 | 2.74 | 6.82 | 1.91 |
| TDD, 16x2, XP Custom Low | 9.59 | 2.89 | 13.40 | 1.93 | 7.72 | 3.45 | 11.17 | 1.81 |
| TDD, 16x4, XP Custom Low | 7.82 | 2.16 | 9.69 | 1.84 | 6.37 | 2.00 | 7.97 | 1.65 |

Based on above results, there is a significant difference in throughput ratios when following eType2 vs Type I codebooks. Therefore, we observe and propose the following.

Observation 1: There is a significant difference in throughput ratios when following Enhanced Type II vs Type I codebooks.

Proposal 2: Use the test metric of throughput ratio between following eType2 and random Type I for defining Enhanced Type II PMI reporting tests and no need to check whether UE reported codebook is not only within Type I codebook set.

As there is larger difference between throughput ratios for Type1 and eType2 under XP Medium, we propose the following.

Proposal 3: Use XP Medium correlation for defining Enhanced Type II PMI reporting tests.

# Conclusions

This paper discusses remaining open issues for enhanced Type II PMI reporting. Following has been observed and proposed.

Proposal 1: Use 90% test point for defining Enhanced Type II PMI reporting test cases.

Observation 1: There is a significant difference in throughput ratios when following Enhanced Type II vs Type I codebooks.

Proposal 2: Use the test metric of throughput ratio between following eType2 and random Type I for defining Enhanced Type II PMI reporting tests and no need to check whether UE reported codebook is not only within Type I codebook set.

Proposal 3: Use XP Medium correlation for defining Enhanced Type II PMI reporting tests.

# References

[1] R4-2017678, “Way Forward on PMI Reporting Requirement for NR eMIMO”, 3GPP TSG RAN WG4 Meeting #97-e, Online, Nov 2 - Nov 13, 2020.