3GPP TSG RAN WG4 Meeting #98-bis-e	R4-2106616

Electronic Meeting, April 12 – April 20, 2021

Agenda item:			5.7.2.2.1

Source:	vivo

Title:	Updated simulation results for CSI-RSRP measurement accuracy

Document for:	Information

- Introduction

In the RAN4#98-e meeting, the simulation results for CSI-RSRP accuracy [1] are provided based on updated simulation assumptions in [2]. The further updated simulation assumptions for CSI-RS based L3 measurement were agreed in [3]. The main purpose of the update is to evaluate CSI-RS based measurement measurement accuracy under the condition of negative timing offset and also the performance difference between timing offset of 0.9CP and CP.

In this contribution, we provide updated simulation results for CSI-RSRP measurement accuracy based on further updated simulation assumptions.

# Discussion

Simulation results for CSI-RSRP measurement accuracy are provided in Tables 1, 2, 3, 4 for different configurations in both FR1 and FR2 according to simulation assumptions in [3]. The accuracy is based on measurements of 5 samples. CDF curves are further provided in Annex for information.

Table 1. simulation results of CSI-RSRP accuracy under {4GHz, SINR = -6dB, 48PRB, D=3, 15KHz SCS}

|  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -CP |  |  | -0.9CP |  |  | -0.5CP |  |  | 0 |  |  |
| AWGN | -1.12 | 0.06 | 0.82 | -1.14 | -0.07 | 0.98 | -1.12 | 0.29 | 1.12 | -0.75 | 0.19 | 0.93 |
| EPA5 | -1.82 | -0.31 | 0.64 | -1.86 | -0.11 | 0.99 | -0.99 | 0.15 | 0.95 | -1.23 | 0.02 | 1.02 |
| ETU30 | -1.51 | -0.36 | 1.02 | -1.13 | -0.21 | 0.93 | -0.72 | 0.03 | 1.24 | -0.94 | -0.10 | 0.94 |
| ETU70 | -1.78 | -0.25 | 0.67 | -1.72 | -0.21 | 0.72 | -1.24 | -0.01 | 0.92 | -1.22 | 0.17 | 1.42 |
|  | CP |  |  | 0.9CP |  |  | 0.5CP |  |  |  |  |  |
| AWGN | -1.14 | -0.28 | 0.62 | -1.13 | -0.17 | 0.67 | -0.97 | 0.11 | 0.85 |  |  |  |
| EPA5 | -1.96 | -0.38 | 0.65 | -1.43 | -0.31 | 0.85 | -1.12 | 0.22 | 1.00 |  |  |  |
| ETU30 | -1.55 | -0.48 | 0.38 | -1.38 | -0.38 | 0.56 | -1.34 | 0.01 | 0.94 |  |  |  |
| ETU70 | -1.38 | -0.47 | 0.96 | -1.41 | -0.26 | 0.73 | -1.32 | 0.14 | 1.26 |  |  |  |

Table 2. simulation results of CSI-RSRP accuracy under {4GHz, SINR = -6dB, 48PRB, D=3, 30KHz SCS}

|  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -CP |  |  | -0.9CP |  |  | -0.5CP |  |  | 0 |  |  |
| AWGN | -1.21 | 0.12 | 1.00 | -1.10 | -0.05 | 0.82 | -0.76 | 0.43 | 1.04 | -0.79 | 0.29 | 1.04 |
| EPA5 | -1.30 | -0.32 | 0.83 | -1.02 | -0.22 | 0.70 | -0.67 | 0.25 | 1.17 | -0.78 | 0.05 | 0.81 |
| ETU30 | -1.70 | -0.22 | 0.68 | -1.66 | -0.24 | 0.70 | -0.78 | 0.12 | 1.09 | -0.82 | 0.06 | 0.71 |
| ETU70 | -1.76 | -0.28 | 0.57 | -1.24 | -0.30 | 0.74 | -0.69 | 0.14 | 0.87 | -1.13 | -0.02 | 0.80 |
|  | CP |  |  | 0.9CP |  |  | 0.5CP |  |  |  |  |  |
| AWGN | -1.26 | -0.30 | 0.47 | -1.18 | -0.19 | 0.89 | -0.93 | 0.20 | 1.02 |  |  |  |
| EPA5 | -1.50 | -0.41 | 0.58 | -1.25 | -0.28 | 0.66 | -1.05 | 0.09 | 0.97 |  |  |  |
| ETU30 | -1.65 | -0.51 | 0.33 | -1.41 | -0.33 | 0.66 | -1.45 | -0.12 | 1.09 |  |  |  |
| ETU70 | -1.50 | -0.42 | 0.33 | -1.52 | -0.59 | 0.49 | -1.39 | -0.15 | 0.92 |  |  |  |

Table 3. simulation results of CSI-RSRP accuracy under {30GHz, SINR = -6dB, 48PRB, D=3, 60KHz SCS}

|  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -CP |  |  | -0.9CP |  |  | -0.5CP |  |  | 0 |  |  |
| TDL-A | -1.44 | -0.32 | 0.84 | -1.26 | -0.23 | 0.76 | -0.81 | 0.04 | 1.13 | -0.95 | 0.03 | 1.11 |
| TDL-C | -1.49 | -0.33 | 0.86 | -1.27 | -0.21 | 0.93 | -0.96 | 0.14 | 1.13 | -1.10 | 0.14 | 1.02 |
|  | CP |  |  | 0.9CP |  |  | 0.5CP |  |  |  |  |  |
| TDL-A | -1.41 | -0.23 | 0.88 | -1.50 | -0.24 | 0.83 | -0.94 | 0.04 | 0.97 |  |  |  |
| TDL-C | -1.55 | -0.30 | 0.79 | -1.37 | -0.26 | 0.84 | -1.18 | 0.05 | 1.08 |  |  |  |

Table 4. simulation results of CSI-RSRP for {30GHz, SINR = -6dB, 48PRB, D=3, 120KHz SCS}

|  | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% | 5% | 50% | 95% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -CP |  |  | -0.9CP |  |  | -0.5CP |  |  | 0 |  |  |
| TDL-A | -1.53 | -0.30 | 0.78 | -1.62 | -0.24 | 0.90 | -1.23 | 0.04 | 1.30 | -1.26 | 0.04 | 1.11 |
| TDL-C | -1.55 | -0.41 | 0.76 | -1.55 | -0.37 | 0.71 | -1.03 | 0.03 | 1.01 | -1.10 | 0.07 | 1.04 |
|  | CP |  |  | 0.9CP |  |  | 0.5CP |  |  |  |  |  |
| TDL-A | -1.44 | -0.35 | 0.70 | -1.35 | -0.28 | 0.76 | -1.39 | 0.02 | 1.10 |  |  |  |
| TDL-C | -1.55 | -0.35 | 0.78 | -1.44 | -0.31 | 0.67 | -1.18 | -0.03 | 1.04 |  |  |  |

Following observations can be made based on simulation results above.

Observation 1. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±2 dB in both FR1 and FR2.

Observation 2. The CSI-RSRP accuracy is comparable between timing offset of CP and 0.9CP.

Observation 3. The CSI-RSRP accuracy is comparable between corresponding positive timing offset and negative timing offset.

# Summary

In this contribution we provided updated simulation results for CSI-RSRP measurement accuracy. Following observations are made.

Observation 1. If timining delay between two cells is smaller than CP, the absolute accuary of CSI-RSRP measurement with 5 samples is within ±2 dB in both FR1 and FR2.

Observation 2. The CSI-RSRP accuracy is comparable between timing offset of CP and 0.9CP.

Observation 3. The CSI-RSRP accuracy is comparable between corresponding positive timing offset and negative timing offset.

- References

- R4-2101768	Simulation results for CSI-RSRP measurement accuracy, vivo

- R4-2017230	Updated link-level simulation assumptions for CSI-RS based L3 measurements, CATT

- R4-2103725	Simulation assumptions for CSI-RS based L3 measurement, CATT

# Annex

CDF curves for CSI-RSRP measurement accuracy for different timing offset are provided in the following figures.

For 15kHz SCS,

For 30kHz SCS,

For 60kHz SCS,

For 120kHz SCS,