3GPP TSG-RAN WG4 Meeting # 99-e                                                             R4-2107908

Electronic Meeting, May. 19-27, 2021

Title:	Reply LS on exception requirements for Intermodulation due to Dual uplink (IMD)

Response to:	R5-211609

Release:	Release 15

Work Item:

Source:	TSG RAN WG4

To:	TSG RAN WG5

Cc:

Contact Person:	Shengxiang, Guo

guoshengxiang@xiaomi.com

Attachments:	-

# 1	Overall description

RAN4 thanks RAN5 for the LS in R5-211609 on exception requirements for Intermodulation due to dual uplink (IMD). RAN4 has discussed the clarifications issues in the LS and would like to answer to RAN5 as follow:

Clarification on Q1: If the EN-DC IMD exceptions are applicable only when the IMD product falls into the victim carrier, and if SA requirements apply otherwise in the case of 2UL.

Answer: Yes, SA requirements shall be applied for dual UL carrier frequency combinations when no IMD product (up to 5th orders) falls into the victim’s Rx CBW and no other EN-DC exception requirements are defined, i.e. no exception due to 1) harmonics (UL harmonic or Receiver harmonic mixing), 2) cross-band isolation, 3) counter-intermodulation (C-IM).

Clarification on Q2: Clarify the criteria that need to be fulfilled in order for MSD=0 to apply.

Answer: MSD=0 could be only applied when carrier frequencies and bandwidths are selected for each active UL band such that there is no any interference falling into Rx CBW under all the conditions in Question 1. However, whether it is meaningful to do this analysis is up to RAN5.

# 2	Actions

To TSG RAN WG5

ACTION: 	RAN4 respectfully asks RAN5 to take account the above RAN4 information.

# 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 Meeting#100e 	 16th – 27th Aug 2021	Electronic Meeting

TSG-RAN4 Meeting#101e 	   1st – 12th Nov 2021	Electronic Meeting