3GPP TSG-RAN WG4 Meeting #99-e		R4-2108607

Electronic, 19nd – 27th May, 2021

Agenda item:	6.3.6

Source:	Intel Corporation

Title:	TP to TS 38.176-1: FRC and PRACH test preambles

Document for:	Approval

# 1	Introduction

As discussed in [1], we proposed the following TP to TS 38.176-1 to define FRC and PRACH test preambles for conducted IAB performance requirements

# 2	Text Proposal to TS 38.176-1

Start of the text proposal

# A.1	IAB-DU Fixed Reference Channels

## A.1.x	Fixed Reference Channels for PUSCH performance requirements (QPSK, R = 193/1024)

The parameters for the reference measurement channels are specified in table A.1.x-1 and table A.1.x-2 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.1.x-1 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.1.x-2 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers.

-	FRC parameters are specified in table A.1.x-3 for FR1 PUSCH with transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer.

Table A.1.x-1: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | D-FR1-A.1.x-1 | D-FR1-A.1.x-2 | D-FR1-A.1.x-3 | D-FR1-A.1.x-4 | D-FR1-A.1.x-5 | D-FR1-A.1.x-6 | D-FR1-A.1.x-7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 1352 | 2856 | 5768 | 1320 | 2792 | 5768 | 14856 |
| Transport block CRC (bits) | 16 | 16 | 24 | 16 | 16 | 24 | 24 |
| Code block CRC size (bits) | - | - | 24 | - | - | 24 | 24 |
| Number of code blocks - C | 1 | 1 | 2 | 1 | 1 | 2 | 4 |
| Code block size including CRC (bits) (Note 2) | 1368 | 2872 | 2920 | 1336 | 2808 | 2920 | 3744 |
| Total number of bits per slot | 7200 | 14976 | 30528 | 6912 | 14688 | 30528 | 78624 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| Note 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0= 0 and l =10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [8]. Note 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [9]. |  |  |  |  |  |  |  |

Table A.1.x-2: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers (QPSK, R=193/1024)

| Reference channel | D-FR1-A.1.x-8 | D-FR1-A.1.x-9 | D-FR1-A.1.x-10 | D-FR1-A.1.x-11 | D-FR1-A.1.x-12 | D-FR1-A.1.x-13 | D-FR1-A.1.x-14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |
| Code rate | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 | 193/1024 |
| Payload size (bits) | 2728 | 5640 | 11528 | 2600 | 5512 | 11528 | 29736 |
| Transport block CRC (bits) | 16 | 24 | 24 | 16 | 24 | 24 | 24 |
| Code block CRC size (bits) | - | 24 | 24 | - | 24 | 24 | 24 |
| Number of code blocks - C | 1 | 2 | 4 | 1 | 2 | 4 | 8 |
| Code block size including CRC (bits) (Note 2) | 2744 | 2856 | 2912 | 2616 | 2792 | 2912 | 3744 |
| Total number of bits per slot | 14400 | 29952 | 61056 | 13824 | 29376 | 61056 | 157248 |
| Total symbols per slot | 7200 | 14976 | 30528 | 6912 | 14688 | 30528 | 78624 |
| Note 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [8]. Note 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [9]. |  |  |  |  |  |  |  |

Table A.1.x-3: FRC parameters for FR1 PUSCH performance requirements, transform precoding enabled, additional DM-RS position = pos1 and 1 transmission layer (QPSK, R=193/1024)

| Reference channel | D-FR1-A.1.x-15 | D-FR1-A.1.x-16 |
| --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 30 |
| Allocated resource blocks | 25 | 24 |
| DFT-s-OFDM Symbols per slot (Note 1) | 12 | 12 |
| Modulation | QPSK | QPSK |
| Code rate | 193/1024 | 193/1024 |
| Payload size (bits) | 1352 | 1320 |
| Transport block CRC (bits) | 16 | 16 |
| Code block CRC size (bits) | - | - |
| Number of code blocks - C | 1 | 1 |
| Code block size including CRC (bits) (Note 2) | 1368 | 1336 |
| Total number of bits per slot | 7200 | 6912 |
| Total symbols per slot | 3600 | 3456 |
| Note 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [8]. Note 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [9]. |  |  |

## A.1.x	Fixed Reference Channels for PUSCH performance requirements (16QAM, R = 658/1024)

The parameters for the reference measurement channels are specified in table A.1.x-1 and table A.1.x-2 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.1.x-1 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer.

-	FRC parameters are specified in table A.1.x-2 for FR1 PUSCH with transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers.

Table A.1.x-1: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (16QAM, R=658/1024)

| Reference channel | D-FR1-A.1.x-1 | D-FR1-A.1.x-2 | D-FR1-A.1.x-3 | D-FR1-A.1.x-4 | D-FR1-A.1.x-5 | D-FR1-A.1.x-6 | D-FR1-A.1.x-7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 9224 | 19464 | 38936 | 8968 | 18960 | 38936 | 100392 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 5 | 2 | 3 | 5 | 12 |
| Code block size including CRC (bits) (Note 2) | 4648 | 6520 | 7816 | 4520 | 6352 | 7816 | 8392 |
| Total number of bits per slot | 14400 | 29952 | 61056 | 13824 | 29376 | 61056 | 157248 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| Note 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [8]. Note 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [9]. |  |  |  |  |  |  |  |

Table A.1.x-2: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 2 transmission layers (16QAM, R=658/1024)

| Reference channel | D-FR1-A.1.x-8 | D-FR1-A.1.x-9 | D-FR1-A.1.x-10 | D-FR1-A.1.x-11 | D-FR1-A.1.x-12 | D-FR1-A.1.x-13 | D-FR1-A.1.x-14 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Code rate | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 | 658/1024 |
| Payload size (bits) | 18432 | 38936 | 77896 | 17928 | 37896 | 77896 | 200808 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 3 | 5 | 10 | 3 | 5 | 10 | 24 |
| Code block size including CRC (bits) (Note 2) | 6176 | 7816 | 7816 | 6008 | 7608 | 7816 | 8392 |
| Total number of bits per slot | 28800 | 59904 | 122112 | 27648 | 58752 | 122112 | 314496 |
| Total symbols per slot | 7200 | 14976 | 30528 | 6912 | 14688 | 30528 | 78624 |
| Note 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [8]. Note 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [9]. |  |  |  |  |  |  |  |

## A.1.x	Fixed Reference Channels for PUSCH performance requirements (64QAM, R = 567/1024)

The parameters for the reference measurement channels are specified in table A.1.x-1 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.1.x-1 for FR1 PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layer.

Table A.1.x-1: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, additional DM-RS position = pos1 and 1 transmission layer (64QAM, R=567/1024)

| Reference channel | D-FR1-A.1.x-1 | D-FR1-A.1.x-2 | D-FR1-A.1.x-3 | D-FR1-A.1.x-4 | D-FR1-A.1.x-5 | D-FR1-A.1.x-6 | D-FR1-A.1.x-7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 15 | 15 | 15 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 25 | 52 | 106 | 24 | 51 | 106 | 273 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 | 12 | 12 | 12 | 12 | 12 |
| Modulation | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| Code rate | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 | 567/1024 |
| Payload size (bits) | 12040 | 25104 | 50184 | 11528 | 24576 | 50184 | 131176 |
| Transport block CRC (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 | 24 | 24 | 24 | 24 | 24 |
| Number of code blocks - C | 2 | 3 | 6 | 2 | 3 | 6 | 16 |
| Code block size including CRC (bits) (Note 2) | 6056 | 8400 | 8392 | 5800 | 8224 | 8392 | 8224 |
| Total number of bits per slot | 21600 | 44928 | 91584 | 20736 | 44064 | 91584 | 235872 |
| Total symbols per slot | 3600 | 7488 | 15264 | 3456 | 7344 | 15264 | 39312 |
| Note 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, additional DM-RS position = pos1, l0 = 2 and l = 11 for PUSCH mapping type A, l0 = 0 and l = 10 for PUSCH mapping type B as per table 6.4.1.1.3-3 of TS 38.211 [8]. Note 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [9]. |  |  |  |  |  |  |  |

## A.1.x	PRACH test preambles

Table A.1.x-1 Test preambles for Normal Mode in FR1

| Burst format | SCS (kHz) | Ncs | Logical sequence index | v |
| --- | --- | --- | --- | --- |
| 0 | 1.25 | 13 | 22 | 32 |
| A1, A2, A3, | 15 | 23 | 0 | 0 |
| B4, C0, C2 | 30 | 46 | 0 | 0 |

# A.2	IAB-MT Fixed Reference Channels

## A.2.x	Fixed Reference Channels for PDSCH performance requirements (16QAM)

The parameters for the reference measurement channels are specified in table A.2.x-1 for FR1 PDSCH performance requirements and FR1 PMI reporting performance requirements

Table A.2.x-1: Fixed Reference Channels for FR1 PDSCH (16QAM)

| Reference channel | M-FR1-A.2.x-1 | M-FR1-A.2.x-2 | M-FR1-A.2.x-3 | M-FR1-A.2.x-4 | M-FR1-A.2.x-5 |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth (MHz) | 40 | 40 | 40 | 40 | 40 |
| Subcarrier spacing (kHz) | 30 | 30 | 30 | 30 | 30 |
| Allocated resource blocks | 106 | 106 | 106 | 106 | 106 |
| Number of consecutive PDSCH symbols | 12 | 12 | 12 | 12 | 12 |
| MCS table | 64QAM | 64QAM | 64QAM | 64QAM | 64QAM |
| MCS index | 13 | 13 | 13 | 13 | 13 |
| Modulation | 16QAM | 16QAM | 16QAM | 16QAM | 16QAM |
| Target Coding Rate | 490/1024 | 490/1024 | 490/1024 | 490/1024 | 490/1024 |
| Number of MIMO layers | 1 | 3 | 4 | 1 | 2 |
| Number of DMRS REs | 12 | 24 | 24 | 24 | 24 |
| Overhead for TBS determination | 0 | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot (bits) | 26632 | 73776 | 98376 | 24576 | 49176 |
| Transport block CRC per Slot (bits) | 24 | 24 | 24 | 24 | 24 |
| Number of Code Blocks per Slot | 4 | 9 | 12 | 3 | 6 |
| Binary Channel Bits Per Slot (bits) | 55968 | 152640 | 203520 | 50880 | 101760 |

## A.2.x	Fixed Reference Channels for PDSCH performance requirements (64QAM)

The parameters for the reference measurement channels are specified in table A.2.x-1 for FR1 PDSCH performance requirements.

Table A.2.x-1: Fixed Reference Channels for FR1 PDSCH (64QAM)

| Reference channel | M-FR1-A.2.x-1 |
| --- | --- |
| Channel bandwidth (MHz) | 40 |
| Subcarrier spacing (kHz) | 30 |
| Allocated resource blocks | 106 |
| Number of consecutive PDSCH symbols | 12 |
| MCS table | 64QAM |
| MCS index | 19 |
| Modulation | 64QAM |
| Target Coding Rate | 517/1024 |
| Number of MIMO layers | 2 |
| Number of DMRS REs | 12 |
| Overhead for TBS determination | 0 |
| Information Bit Payload per Slot (bits) | 83976 |
| Transport block CRC per Slot (bits) | 24 |
| Number of Code Blocks per Slot | 10 |
| Binary Channel Bits Per Slot (bits) | 167904 |

## A.2.x	Fixed Reference Channels for PDSCH performance requirements (256QAM)

The parameters for the reference measurement channels are specified in table A.2.x-1 for FR1 PDSCH performance requirements.

Table A.2.x-1: Fixed Reference Channels for FR1 PDSCH (256QAM)

| Reference channel | M-FR1-A.2.x-1 |
| --- | --- |
| Channel bandwidth (MHz) | 40 |
| Subcarrier spacing (kHz) | 30 |
| Allocated resource blocks | 106 |
| Number of consecutive PDSCH symbols | 12 |
| MCS table | 256QAM |
| MCS index | 24 |
| Modulation | 256QAM |
| Target Coding Rate | 0.82 |
| Number of MIMO layers | 1 |
| Number of DMRS REs | 12 |
| Overhead for TBS determination | 0 |
| Information Bit Payload per Slot (bits) | 92200 |
| Transport block CRC per Slot (bits) | 24 |
| Number of Code Blocks per Slot | 11 |
| Binary Channel Bits Per Slot (bits) | 111936 |

## A.2.x	Fixed Reference Channels for PDCCH performance requirements

The parameters for the reference measurement channels are specified in table A.2.x-1 for FR1 PDCCH performance requirements.

Table A.2.x-1: Fixed Reference Channels for FR1 PDCCH

| Reference channel | M-FR1-A.2.x-1 | M-FR1-A.2.x-2 | M-FR1-A.2.x-3 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 | 30 | 30 |
| CORESET frequency domain allocation | 102 | 102 | 90 |
| CORESET time domain allocation | 1 | 1 | 1 |
| Aggregation level | 2 | 4 | 8 |
| DCI Format | 1_0 | 1_1 | 1_1 |
| Payload (without CRC) (bits) | 41 | 53 | 53 |

## A.2.x	Fixed Reference Channels for CSI reporting performance requirements

This clause defines the DL signal applicable to the reporting of channel state information

Tables A.2.x-1 specifies the mapping of CQI index to Information Bit payload, which complies with the CQI definition specified in clause 5.2.2.1 of TS 38.214 [12] and with MCS definition specified in clause 5.1.3 of TS 38.214 [12].

The parameters for the reference measurement channels are specified in table A.2.x-1 for FR1 FR1 PMI reporting performance requirements

Table A.2.x-1: Fixed Reference Channels for CSI reporting

| Reference channel |  |  |  | M-FR1-A.2.x-1 | M-FR1-A.2.x-2 | M-FR1-A.2.x-3 | M-FR1-A.2.x-4 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MCS table |  |  |  | 256QAM |  |  |  |
| Number of allocated PDSCH resource blocks |  |  |  | 106 | 106 | 106 | 106 |
| Number of consecutive PDSCH symbols |  |  |  | 12 | 12 | 12 | 12 |
| Number of PDSCH MIMO layers |  |  |  | 1 | 2 | 3 | 4 |
| Number of DMRS REs (Note 1) |  |  |  | 24 | 24 | 24 | 24 |
| Overhead for TBS determination |  |  |  | 0 | 0 | 0 | 0 |
| Available RE-s for PDSCH |  |  |  | 12720 | 12720 | 12720 | 12720 |
| CQI index | Spectral efficiency | MCS index | Modulation |  |  |  |  |
| 0 | OOR | OOR | OOR | N/A | N/A | N/A | N/A |
| 1 | 0.1523 | 0 | QPSK | 2976 | 5896 | 8976 | 11784 |
| 2 | 0.3770 | 1 |  | 4744 | 9480 | 14344 | 18976 |
| 3 | 0.8770 | 3 |  | 11016 | 22536 | 33816 | 45096 |
| 4 | 1.4766 | 5 | 16QAM | 18960 | 37896 | 56368 | 75792 |
| 5 | 1.9141 | 7 |  | 24576 | 49176 | 73776 | 98376 |
| 6 | 2.4063 | 9 |  | 30728 | 61480 | 92200 | 122976 |
| 7 | 2.7305 | 11 | 64QAM | 34816 | 69672 | 104496 | 139376 |
| 8 | 3.3223 | 13 |  | 42016 | 83976 | 127080 | 167976 |
| 9 | 3.9023 | 15 |  | 49176 | 98376 | 147576 | 196776 |
| 10 | 4.5234 | 17 |  | 57376 | 114776 | 172176 | 229576 |
| 11 | 5.1152 | 19 |  | 65576 | 131176 | 196776 | 262376 |
| 12 | 5.5547 | 21 | 256QAM | 69672 | 139376 | 213176 | 278776 |
| 13 | 6.2266 | 23 |  | 79896 | 159880 | 237776 | 319784 |
| 14 | 6.9141 | 25 |  | 88064 | 176208 | 262376 | 352440 |
| 15 | 7.4063 | 27 |  | 94248 | 188576 | 278776 | 376896 |
| Note 1: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data Note 2: PDSCH is only scheduled on slots which are full DL |  |  |  |  |  |  |  |

Table A.2.x-1: Fixed Reference Channels for FR1 PMI reporting

| Reference channel | M-FR1-A.2.x-1 | M-FR1-A.2.x-2 |
| --- | --- | --- |
| Channel bandwidth (MHz) | 40 | 40 |
| Subcarrier spacing (kHz) | 30 | 30 |
| Allocated resource blocks | 106 | 106 |
| Number of consecutive PDSCH symbols | 12 | 12 |
| MCS table | 64QAM | 64QAM |
| MCS index | 13 | 13 |
| Modulation | 16QAM | 16QAM |
| Target Coding Rate | 490/1024 | 490/1024 |
| Number of MIMO layers | 1 | 2 |
| Number of DMRS REs | 24 | 24 |
| Overhead for TBS determination | 0 | 0 |
| Information Bit Payload per Slot (bits) | 24576 | 49176 |
| Transport block CRC per Slot (bits) | 24 | 24 |
| Number of Code Blocks per Slot | 3 | 6 |
| Binary Channel Bits Per Slot (bits) | 50880 | 101760 |

End of the text proposal

# References

- R4-2103994 “WF on Rel-16 NR IAB demodulation requirements”, Nokia, Nokia Shanghai Bell, RAN4 #98e, February 2021