3GPP TSG-RAN Meeting # 99-e		R4-2109073

Electronic Meeting, May. 19 – 27, 2021

Title:	Draft LS on RRM relaxation in power saving

Response to:

Release:	Rel-16

Work Item:	NR_UE_pow_sav-Core

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact person:	Yanze Fu

fuyanze@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

RAN4 had been discussing requirements for clarification of applicability of IE highPriorityMeasRelax. Companies have different understandings of previous agreements in RAN4. After discussion, RAN4 agreed to the measurement requirements as follows and there are misalignments between RAN2 and RAN4.

For inter-frequency measurements for UE fulfilling only low mobility criterion:

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

For inter-RAT measurements for UE fulfilling only low mobility criterion:

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every K2*Thigher_priority_search  where Thigher_priority_search is described in clause 4.2.2.7 and, K2 = 60. Otherwise if the UE is not configured with highPriorityMeasRelax then the UE shall search for E-UTRA inter-RAT frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7.

In clause 4.2.2.7 of 38.133:

4.2.2.7	General requirements

The UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds, where Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information.

For a UE configured with early measurement reporting, while T331 is running, Nlayers is the combined total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information and carriers configured for idle mode CA measurements.

Note:	combined total number means that if a carrier is a high priority carrier and additionally a carrier configured for idle mode CA measurements, it only counts as one carrier.

2	Actions

To RAN WG2

ACTION: 	RAN4 respectfully asks RAN2 to take the above RAN4 agreements into account in their future work.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 #100-e	Aug 16 - Aug 27, 2021	Online

TSG-RAN4 #101-e	Nov 01 - Nov 12, 2021	Online