3GPP TSG-RAN WG4 Meeting #98-bis-e	R4-2109593

Electronic meeting, 19th – 27th May, 2021

Source:	Ericsson

Title:	Simulation results for NR-U PUSCH demodulation requirement

Agenda item:	6.1.7.4.2

Document for:	information

# Introduction

In RAN4#98-e meeting, companies got agreements on PUSCH simulation assumptions [1]. This contribution delivers our simulation results for both PUSCH and CG-UCI multiplexing on PUSCH based on our analysis and proposals [2].

# Simulation assumptions and results for PUSCH

Table 2-1. Simulation assumptions for PUSCH demodulation

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | {0, 2, 3, 1} |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | The first interlace |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2-2. Ideal simulation results of 20MHz bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | Additional DM-RS position | MCS | Subcarrier spacing (kHz) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | typeA | typeB |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | pos1 | 20 | 15 | 9.5 | 9.5 |
|  |  |  |  |  |  |  | 30 | 9.4 | 9.4 |

Table 2-3. Impairment simulation results of 20MHz bandwidth

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | Additional DM-RS position | MCS | Subcarrier spacing (kHz) | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | typeA | typeB |
| 1 | 2 | Normal | TDLA30-10 Low | 70 % | pos1 | 20 | 15 | 12 | 12 |
|  |  |  |  |  |  |  | 30 | 11.9 | 11.9 |

# Simulation assumptions and results for CG-UCI multiplexing on PUSCH

Table 3-1 simulation assumptions for CG-UCI multiplexing on PUSCH demodulation

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| Channel model |  | TDLA30-10 low |
| Antenna configuration |  | 1Tx2Rx low |
| MCS |  | 20 |
| HARQ | Maximum number of HARQ transmissions | 1 |
|  | RV sequence | 0 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID0 = 0, nSCID = 0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | 15kHz SCS: {0, 10, 20, 30, 40, 50, 60, 70, 80, 90, 100} 30kHz SCS: {0, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50} |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
|  | Number of CG-UCI information bit payload | 18 |
| UCI | scaling | 1 |
|  | betaOffsetCG-UCI-r16 | 8 |
|  | UCI partition for frequency hopping | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. |  |  |

Pattern of CG-UCI information bits

- Use a bit pattern {c0,c1,…,c17} that consists of :

  - HARQ process number: [c0,c1,c2,c3] = [0 0 0 1]

  - RV sequence: [c4,c5] = [0 0]

  - NDI: [c6] = toggle for every new transmission, e.g. 0 for even transmissions and 1 for odd ones

  - COT sharing information field: [c7,c8,…c17] = [0,0,0,0,0,0,0,0,0,0,0]

Table 3-2 Ideal simulation results for 20MHz BW

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix | PUSCH mapping type | Additional DM-RS position | SCS [kHz] | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | Type A | pos1 | 15 | 2.3 |
|  |  |  |  |  |  | 30 | 2.3 |
|  | 2 | Normal | TDLA30-10 Low | Type B | pos1 | 15 | 2.3 |
|  |  |  |  |  |  | 30 | 2.4 |

Table 3-3 Impairment simulation results for 20MHz BW

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix | PUSCH mapping type | Additional DM-RS position | SCS [kHz] | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-10 Low | Type A | pos1 | 15 | 4.3 |
|  |  |  |  |  |  | 30 | 4.3 |
|  | 2 | Normal | TDLA30-10 Low | Type B | pos1 | 15 | 4.3 |
|  |  |  |  |  |  | 30 | 4.4 |

# Conclusion

In this contribution, we deliver our simulation results based on the WF in the last meeting and our discussion paper [2].

# References

[1]	R4-2103988, “Way forward on NR-U BS demodulation requirements for general part and PUSCH”, RAN4#98-e, Huawei, HiSilicon

[2]	R4-2104548, “Discussion on NR-U PUSCH demodulation requirements”, RAN4#98-bis-e, Ericsson

[3]