# 3GPP TSG-RAN WG4 Meeting #99-e	R4-2109691

# Electronic Meeting, 19th - 27th May, 2021

Source: 	vivo

Title: 				 TP on operating bands and channel arrangement for SL enhancement

Agenda Item:	9.14.1

Document for:	Approval

- Introduction

In the last meeting, the TR 38.785 for SL enhancement in Rel-17 [1] was approved. In this text proposal, the operating bands for SL enhancement and system parameters is captured for the TR based on our companion paper [2].

- Reference

[1]		R4-2104969, TR38.785 v0.1.0 TR Update for SL enhancement in Rel-17, LGE, RAN4#98bis-e.

[2]   R4-2109692, Discussion on system parameters for SL enhancement, vivo, RAN4#99-e.

- Annex: TP for channel arrangement in TR38.785

============================ Start of TP ============================

# 7	Operating bands and channel arrangement for SL enhancement

## 7.1	Operating bands

### 7.1.1	Operating bands

NR V2X is designed to operate in the operating bands in FR1 defined in Table 7.1.1-1.

Table 7.1.1-1 V2X operating bands in FR1

| V2X Operating Band | Sidelink (SL) Transmission operating band |  |  | Sidelink (SL) Reception operating band |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| n381 | 2570 MHz | - | 2620 MHz | 2570 MHz | - | 2620 MHz | HD | PC5 |
| n47 | 5855 MHz | - | 5925 MHz | 5855 MHz | - | 5925 MHz | HD | PC5 |
| n142 | 788 MHz | - | 798 MHz | 758 MHz | - | 768 MHz | HD | PC5 |
| n79 | 4400 MHz | - | 5000 MHz | 4400 MHz | - | 5000 MHz | HD | PC5 |
| Note 1: When this band is used for V2X SL service, the band is exclusively used for NR V2X in particular regions. Note 2: This band is only used for SL transmission for public safety services when UE is out of coverage of LTE/NR. |  |  |  |  |  |  |  |  |

### 7.1.2	Operating bands for inter-band con-current operation in FR1

7.1.3 Operating bands for intra-band con-current operation in FR1

The intra-band con-current operating bands in Rel-17 are defined in Table 7.1.3-1.

Table 7.1.3-1 Intra-band con-current operating bands

| V2X con-current operating Band | NR or V2X Operating Band | Interface |
| --- | --- | --- |
| V2X_n38 | n38 | Uu |
|  | n38 | PC5 |
| V2X_n79 | n79 | Uu |
|  | n38 | PC5 |

## 7.2	Channel bandwidth

### 7.2.1	Channel bandwidth

Table 7.2.1-1 V2X Communication channel bandwidth

|  |  | V2X band / V2X channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X Operating Band | SCS kHz | 5 MHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n38 | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n47 | 15 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes |  |  |  |  |  |
| n14 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |
|  | 30 |  | Yes |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |
| n79 | 15 |  |  |  |  | Yes |  |  |  |  |  |
|  | 30 |  |  |  |  | Yes |  |  |  |  |  |
|  | 60 |  |  |  |  | Yes |  |  |  |  |  |

### 7.2.2	Channel bandwidth for inter-band con-current operation

### 7.2.3 Channel bandwidth for intra-band V2X operation

| V2X configuration | Operating band | Channel bandwidths for NR Uu carrier (MHz) | Channel bandwidths for NR SL carrier (MHz) |
| --- | --- | --- | --- |
| V2X_n38 | n38 for Uu/SL | 10,20,30,40 | 10,20,30,40 |
| V2X_n79 | n79 for Uu/SL | 40,50,60,80,100 | 40 |

## 7.3	Channel arrangement enhancement

### 7.3.1	Channel raster

#### 7.3.1.1	NR-ARFCN and channel raster

The NR-ARFCN and channel raster defined in subclause 5.4.2.1 in TS38.101-1 are applied to each licensed operating band for NR SL enhancement.

For NR V2X UE, the reference frequency can be shifted by configuration.

FREF_V2X = FREF + Δshift + N * 5 kHz

where

Δshift = 0 kHz or 7.5 kHz indicated in IE (frequencyShift7p5khz), and

N can be set as one of following values {-1, 0, 1}, which are signalled by the network in higher layer parameters or configured by pre-configuration parameters.

#### 7.3.1.2	Channel raster to resource element mapping

Channel raster to resource element mapping defined in subclause 5.4.2.2 in TS38.101-1 are applied to each licensed operating band for NR SL enhancement.

#### 7.3.1.3	Channel raster entries for each operating band

The channel raster entries defined in subclause 5.4.2.3 in TS38.101-1 are applied to each licensed operating band for NR SL enhancement.

### 7.3.2	Synchronization raster

There is no synchronization raster definition for each licensed operating band for NR SL enhancement.

============================ End of TP ============================