3GPP TSG-RAN WG4 Meeting AH-1807 LTE/NR perf	R4-1808669

Montreal, Canada, 2th – 6th July 2018

Agenda Item:	4.2.1.1.7.1

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Proposals on MU for OTA receiver co-location blocking requirement

Document for:	Approval

# 1.	Introduction

The MU and TT for OTA receiver directional requirements were discussed in RAN4#87 and the way forward was agreed [1], with two open issues relating to the MU. Our proposals to conclude on the two open issues relating to the MU are provided in [2].

This contribution provides our proposals on the MU for the OTA receiver co-location blocking requirement, according to the agreed way forward in [1] and proposals in [2].

# 2.	Discussion

It was agreed in [1] that the MU for OTA receiver directional requirements should be calculated as:

MU=MUconductedwanted2+MUconductedint2+MUEIS2-MUTestEquipment2-2*MUmatching2+ACLR effect

And we propose in [2] to use the EIS measurement uncertainty from the source ‘Impedance mismatch in the transmitting chain’ recorded in TR 37.842 [3] as the mismatch uncertainty for each of the radiated measurement methods for the frequency range below 4.2 GHz, and PA is not needed for frequency range below 6GHz for high-end signal generators.

The interfering signal for the OTA receiver co-location blocking requirement is an out-of-band CW signal. Thus, the impact of interferer broadband noise should be considered instead of ACLR effect as stated in TS 37.145-1 [4]. Hence, the equation above should be modified for the OTA receiver co-location blocking requirement as:

MU=MUconductedwanted2+MUconductedint2+MUEIS2-MUTestEquipment2-2*MUmatching2+broadband noise effect

Therefore, the MU for the OTA receiver co-location blocking requirement can be calculated as shown in Tables 1 to 4 below for each of the radiated measurement methods, respectively.

Table 1: MU for Indoor Anechoic Chamber

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedint (Interferer signal level error in [4]) | 2.0 | 2.0 |
| MUEIS (Combined standard uncertainty [3]) | 0.62 | 0.64 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0.10 | 0.16 |
| Broadband noise effect (Impact of interferer broadband noise [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 2.55 | 2.67 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 5.01 | 5.23 |

Table 2: MU for Compact Antenna Test Range

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedint (Interferer signal level error in [4]) | 2.0 | 2.0 |
| MUEIS (Combined standard uncertainty [3]) | 0.68 | 0.71 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0.09 | 0.23 |
| Broadband noise effect (Impact of interferer broadband noise [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 2.57 | 2.68 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 5.04 | 5.25 |

Table 3: MU for One Dimensional Compact Range Chamber

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedint (Interferer signal level error in [4]) | 2.0 | 2.0 |
| MUEIS (Combined standard uncertainty [3]) | 0.66 | 0.73 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0 | 0.01 |
| Broadband noise effect (Impact of interferer broadband noise [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 2.57 | 2.71 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 5.04 | 5.31 |

Table 4: MU for Near Field Test Range

| Test System Uncertainty | Standard uncertainty ui [dB] f ≦ 3GHz | Standard uncertainty ui [dB] 3GHz < f ≦ 4.2 GHz |
| --- | --- | --- |
| MUconductedwanted (Wanted signal level error in [4]) | 0.7 | 1.0 |
| MUconductedint (Interferer signal level error in [4]) | 2.0 | 2.0 |
| MUEIS (Combined standard uncertainty [3]) | 0.63 | 0.63 |
| MUTestEquipment (Uncertainty of the RF signal generator in [3]) | 0.46 | 0.46 |
| MUmatching (Impedance mismatch in the transmitting chain in [3]) | 0.20 | 0.20 |
| Broadband noise effect (Impact of interferer broadband noise [4]) | 0.4 | 0.4 |
| Combined standard uncertainty (1σ) | 2.54 | 2.66 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) | 4.99 | 5.21 |

Consequently, the test system specific measurement uncertainty values for the OTA receiver co-location blocking test can be obtained in Table 5 below.

# Table 5: Test system specific measurement uncertainty values for the OTA receiver co-location blocking test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 5.01 | 5.23 |
| Compact Antenna Test Range | 5.04 | 5.25 |
| One Dimensional Compact Range Chamber | 5.04 | 5.31 |
| Near Field Test Range | 4.99 | 5.21 |
| Common maximum accepted test system uncertainty | 5.0 | 5.3 |

# 3.	Conclusion

This contribution has provided our proposals on the MU for the OTA receiver co-location blocking requirement, according to the agreed way forward in [1] and proposals in [2].

Proposal:

Adopt the following table as the MU for the OTA receiver co-location blocking test.

# Table 5: Test system specific measurement uncertainty values for the OTA receiver co-location blocking test

|  | Expanded uncertainty ue [dB] |  |
| --- | --- | --- |
|  | f ≦ 3GHz | 3GHz < f ≦ 4.2 GHz |
| Indoor Anechoic Chamber | 5.01 | 5.23 |
| Compact Antenna Test Range | 5.04 | 5.25 |
| One Dimensional Compact Range Chamber | 5.04 | 5.31 |
| Near Field Test Range | 4.99 | 5.21 |
| Common maximum accepted test system uncertainty | 5.0 | 5.3 |

# References

[1]	R4-1808366, “WF on MU and TT for receiver requirements”, Ericsson.

[2]	R4-1808658, “Proposals on MU and TT for OTA receiver directional requirements”, Nokia, Nokia Shanghai Bell.

[3]		3GPP TR 37.842 v13.2.0, “E-UTRA and UTRA; Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

[4]	3GPP TR 36.141 v15.2.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.