3GPP TSG-RAN WG4-AH-1807 Meeting	R4-1808742

Montreal, Canada, 2 – 6 July 2018

Agenda item:	5.4.1.5

Title: 	Discussion on NR UE CSI feedback performance requirements

Source: 	Intel Corporation

Document for:	Discussion

# Introduction

In RAN4#86 Meeting (Feb. 2018), we have provided our preliminary view on the scope of the NR UE performance requirements, and raised a number of questions to trigger the discussion on the topic of NR UE CSI feedback performance requirements [1]. In RAN4#87 Meeting (May 2018), the work plan [2] and a WF [3] were approved for NR UE demodulation and CSI feedback performance requirements. In this contribution, we further discuss the test coverage and configurations of the NR UE CSI feedback performance requirements.

# Discussion

In our parallel contribution [4], we provide our views on the NR UE performance requirements scenarios and focus on the general scope of requirements and target scenarios. In this contribution, we address more detailed aspects of NR UE CSI reporting performance requirements. In our view, the CSI reporting performance requirements which are needed to be specified are listed in Table 1 below.

Table 1: NR UE CSI reporting performance requirements

| Question | Comments |
| --- | --- |
| Types of requirements and test purposes | The CSI tests should ensure sufficient test coverage of the key NR CSI reporting features. The test should focus on the mandatory set of features. The following types of CSI reporting requirements shall be considered: - Channel quality indicator (CQI) reporting - Precoding matrix index (PMI) reporting - Rank indication (RI) reporting - CSI-RS resource indicator (CRI) reporting - Strongest layer indication (LI) reporting - CRI/L1-RSRP reporting |
| Key features to be tested | CSI reporting tests should aim to verify the following features: General - Define test cases to cover different CSI reporting configuration: periodic, semi-persistent, and aperiodic. CQI Tests - Define Wideband CQI tests: - Define CQI tests that a single CQI is reported for each codeword for the entire CSI reporting band. - Define Subband CQI tests: - Define CQI tests that one CQI for each codeword is reported for each subband in the CSI reporting band; - For subband mode, there are different possible/configurable subband sizes depending on the total number of PRBs in the carrier bandwidth. Define CQI tests for different subband sizes (in terms of number of PRBs). - Define CQI tests considering two 4-bit CQI tables, respectively: - Maximum modulation order 64QAM; - Maximum modulation order 256QAM. - Need to consider different target BLER requirements for CQI tests for URLLC. PMI Tests - Define Wideband PMI tests: - Define PMI tests that a single PMI is reported for the entire CSI reporting band. - Define Subband PMI tests: - Except cases with 2 antenna ports, define PMI tests that a single wideband indication is reported for the entire CSI reporting band and one subband indication is reported for each subband in the CSI reporting band; - For cases with 2 antenna ports, define PMI tests that a PMI is reported for each subband in the CSI reporting band; - Define PMI tests considering different subband sizes (in terms of number of PRBs). - Define PMI test for up to 32 CSI-RS antenna ports - Define tests cases for different codebooks RI Tests - Define RI tests: - Define accuracy RI reporting tests that are determined by the relative increase of the throughput obtained by transmitting based on the reported rank compared to that based on a fixed rank. Other Issues: - Study and compare the throughput performance difference with and without accurate strongest layer indicator applied, in order to justify if the throughput performance difference is large enough for tests. - CRI/L1-RSRP reporting is introduced in NR CSI feedback for beam management purpose. The time domain behavior of the reporting is indicated by the higher layer parameter “reportConfigType” and can be set to aperiodic, semi-persistent, or periodic. It is necessary to firstly study the viable assumptions and metric for L1-RSRP reporting tests. One option could be to align with the assumptions and metric determined by the RRM RSRP performance test. Since the discussion of L1-RSRP test methodology is on-going from the RRM perspective, such as if delay and/or accuracy requirements are needed and if it would be included in the core and/or performance parts. From CSI feedback performance requirement perspective, it needs to align with the outcome of the RRM discussion to avoid duplicated work. - For interference measurement, the CSI-IM resource based measurement should be prioritized. - Different “MeasRestrictionConfig-time-channel” and “MeasRestrictionConfig-time-interference” settings need to be considered to specify parameters to enable configuration of time domain measurement restriction for channel and interference. |

In RAN4#86bis Meeting, initial agreements on the scope of NR UE CSI performance requirements were reached [5]:

- Define the following NR UE performance requirements - CSI reporting requirements - CQI reporting - PMI reporting - RI reporting - FFS for: CRI reporting; CRI/L1-RSRP reporting; LI reporting - Other open issues (for information) - CQI reporting - Test metrics - Wideband/Subband CQI tests - CQI tables - PMI reporting - Test metrics - Wideband/Subband PMI tests - Number of APs - Codebook types - RI reporting

In RAN4#87 Meeting, agreements related to the NR UE CSI performance requirements were reached in a WF [3] as listed below:

- First focused on the mandatory features

- Overall Test Configuration & Scope

  - First priority focused on:

    - NZP CSI-RS for channel measurement
CSI-IM for interference measurement

    - CSI report types: periodic CSI report and aperiodic CSI report

    - CSI-RS resources types: periodic and aperiodic

- PMI reporting

  - In Rel-15 focused on Type-I single panel codebook

Moreover, in RAN4#87 Meeting, the work plan of NR UE performance requirements [2] was approved to divide the performance requirements definition into three phase as shown in Fig.1 below:

Fig.1 Three phases for the work plan of NR UE performance requirements

Specifically, for NR UE CSI performance requirements, Fig.1 indicates that:

Phase 1: Completion 1st bullet of test cases (RAN4 July Ad-hoc, RAN4#88)

- CSI: CQI (static), PMI, RI test cases

Phase 2: Completion 2nd bullet of test cases (RAN4#88bis, RAN4#89)

- CSI: CQI (fading), CRI/L1-RSRP if any

In our view:

Proposal 1:	Considering the time limit, we propose to prioritize the CQI/PMI/RI test definition for NR UE CSI reporting requirements, and focus on the key performance requirement tests for essential CSI feedback functionalities. Meanwhile, additional studies on LI reporting can be considered with second priority.

Proposal 2:	The discussion of L1-RSRP test/requirement is on-going in the RRM session, such as whether/how to include delay and/or accuracy requirements from the RRM perspective. In the Demod session and from CSI feedback performance requirement perspective, it needs to align with the outcome of the RRM discussion to avoid duplicated work.

Because CRI/L1-RSRP reporting is mandatory for FR2, which is used for beam management with mmWave analog beamforming, it would be also sufficient for CRI test. In this case, the CRIs will be chosen based on L1-RSRP, instead of, such as, CRI/CQI reporting. Therefore, we propose that:

Proposal 3:	Do not introduce CSI reporting CRI requirements for FR2.

According to Rel.15 NR UE feature list in [6], the candidate values set is {1:8} for the max # of CSI-RS resource in a resource set. However, it is unclear now if it is mandatory for Rel.15 NR UE to support multiple CRI process, at least for FR1. In case that it is mandatory to support single CSI-RS resource in a resource set, it would make CSI reporting CRI test for FR1 out of the focus of mandatory features for Rel.15 NR UE requirements. Therefore, we propose that:

Proposal 4:	Postpone the discussion of Rel.15 NR CSI reporting CRI test/requirement for FR1 until the feature list is finalized in RAN1.

Moreover, according to Rel-15 NR UE feature list in [6], the mandatory PMI codebook type and Tx antenna configurations are defined as following:

- Type I single panel codebook based PMI

- 2Tx codebook for FR1 and FR2

- 4Tx codebook for FR1

- 8Tx codebook for FR1 when configured as wideband CSI report

Therefore, for CQI/PMI/RI tests definition to cover the key performance requirement tests for essential mandatory CSI feedback functions, Table 2 lists the initial simulation assumption parameters for the prioritized CQI/PMI/RI test definition to choose from.

Table 2 Test parameters for CSI reporting performance requirements

| Parameters | FR1 | FR2 |
| --- | --- | --- |
| CBW[MHz]/SCS[KHz] | 10MHz/15KHz 20MHz/30KHz 40MHz/30KHz | 100MHz/60KHz 200MHz/120KHz |
| Modulation | QPSK, 16QAM, 64QAM, 256QAM | QPSK, 16QAM, 64QAM |
| Rank | 1, 2, 3, 4 | 1, 2 |
| PTRS presence | OFF | ON |
| CSI-RS Type | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement | - NZP CSI-RS for channel measurement - CSI-IM for interference measurement |
| CSI type | Type I single-panel | Type I single-panel |
| PMI frequency granularity | Wideband | Wideband |
| CQI frequency granularity | Wideband | Wideband |
| Number of Tx CSI-RS ports | 2, 4, 8 | 2 |
| Number of Rx ports | 2, 4 | 2 |

Proposal 5:	Considering use of the initial simulation parameters listed in Table 2 for the prioritized CQI/PMI/RI tests.

Proposal 6:	Specifically, we propose the initial simulation assumptions listed in Table 3 for FR1 static CQI test.

Table 3 Test parameters for FR1 CQI reporting performance requirements

| Parameters | Test 1 | Test 2 |
| --- | --- | --- |
| CBW[MHz]/SCS[KHz] | 10MHz/15KHz | 20MHz/30KHz |
| PTRS presence | OFF | OFF |
| NZP-CSI-RS and ZP-CSI-RS | Antenna ports {3000, 3001, 3002, 3003} | Antenna ports {3000, 3001} |
| NZP-CSI-RS periodicity / slot offset | 5/1 | 5/1 |
| ZP CSI-RS periodicity / slot offset | 5/0 | 5/0 |
| CSI type | Type I single-panel | Type I single-panel |
| PMI frequency granularity | Wideband | Wideband |
| Antenna configuration | 4×2 (N1, N2) = (2, 1) | 2×2 |
| Propagation condition | Static [TS36.101 Clause B.1] | Static [TS36.101 Clause B.1] |
| CQI frequency granularity | Wideband | Wideband |
| Max number of HARQ transmissions | 1 | 1 |
| Reporting periodicity | TBD | TBD |
| Reporting delay | TBD | TBD |
| SNR | TBD | TBD |

# Conclusion

In this contribution, we further discuss the test coverage and configurations of the NR UE CSI feedback performance requirements. We make the following proposals:

Proposal 1:	Considering the time limit, we propose to prioritize the CQI/PMI/RI test definition for NR UE CSI reporting requirements, and focus on the key performance requirement tests for essential CSI feedback functionalities. Meanwhile, additional studies on LI reporting can be considered with second priority.

Proposal 2:	The discussion of L1-RSRP test/requirement is on-going in the RRM session, such as whether/how to include delay and/or accuracy requirements from the RRM perspective. In the Demod session and from CSI feedback performance requirement perspective, it needs to align with the outcome of the RRM discussion to avoid duplicated work.

Proposal 3:	Do not introduce CSI reporting CRI requirements for FR2.

Proposal 4:	Postpone the discussion of Rel.15 NR CSI reporting CRI test/requirement for FR1 until the feature list is finalized in RAN1.

Proposal 5:	Considering use of the initial simulation parameters listed in Table 2 for the prioritized CQI/PMI/RI tests.

Proposal 6:	Specifically, we propose the initial simulation assumptions listed in Table 3 for FR1 static CQI test.

# Reference

[1] R4-1801758, “Views on NR UE performance requirements”, Intel Corporation, Feb. 2018

[2] R4-1808029, “Work plan for UE performance requirements”, Samsung, May 2018

[3] R4-1808027, “Way forward on NR UE demodulation and CSI requirements”, Ericsson, May 2018

[4] R4-1808744, “Views on NR UE Demodulation and CSI reporting requirements”, Intel Corporation, June 2018

[5] R4-1805549, “Way Forward on NR UE performance requirements”, Intel Corporation, Ericsson, Samsung, NTT DOCOMO, Qualcomm, LGE, Spirent, April 2018

[6] RP-181483, “Rel-15 NR UE feature list”, June 2018