3GPP TSG RAN WG4 Meeting AH-1807		R4-1808761

Montreal, Canada, 2nd – 6th July 2018

Source: 	NTT DOCOMO, INC.

Title: 	Remaining issues on requirements of intra-frequency measurement without gap

Agenda item:	5.2.4.1.1

Document for:	Discussion

- Introduction

In the last several RAN4 meetings, requirements of intra-frequency measurement have been discussed, but there are some remaining issues including values of scaling factor N related to Rx beam sweeping and requirements in case of CA. In this contribution, we provide our views on remaining issues for intra-frequency measurement requirements.

- Discussion

Scaling factor N related to Rx beam sweeping

Latest specification for requirements of intra-frequency measurement is following.

TS38.133 [1] # Table 9.2.5.1-2: Time period for PSS/SSS detection, (Frequency range FR2) DRX cycle; TPSS/SSS_sync. No DRX; max[ 600ms, ceil( [5] x Kp) x N1 x SMTC period ]Note 1. DRX cycle≤ 320ms; max[ 600ms, ceil(1.5 x [5] x Kp) x N1 x max(SMTC period,DRX cycle) ]. DRX cycle>320ms; Ceil( [5] x Kp) x N1 x DRX cycle. …; …. Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. # Table 9.2.5.1-4: Time period for time index detection (Frequency range FR2) DRX cycle; TSSB_time_index. No DRX; max[ 200ms, ceil( [5] x Kp) x N2 x SMTC period ]Note 1. DRX cycle≤ 320ms; max[ 200ms, ceil (1.5 x[5] x Kp) x N2 x max(SMTC period,DRX cycle) ]. DRX cycle>320ms; Ceil( [5] x Kp) x N2 x DRX cycle. …; …. Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. # Table 9.2.5.2-2: Measurement period for intrafrequency measurements without gaps(Frequency FR2) DRX cycle; T SSB_measurement_period. No DRX; max[ 400ms, ceil( 5 x Kp) x N3 x SMTC period ]Note 1. DRX cycle≤ 320ms; max[ 400ms, ceil(1.5x 5 x Kp) x N3 x max(SMTC period,DRX cycle) ]. DRX cycle>320ms; ceil(5 xKp ) x N3 x DRX cycle. Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Regarding cell identification delay requirements for FR2, values of scaling factor N related to Rx beam sweeping have not been concreated yet. As we mentioned in [2], too long delay in cell detection and measurement would cause degradation of system performance due to difference between measurement report and actual condition when the report is provided. This means that UE might report some degrading beams or cells to NW and might miss some strong beams or cells due to limited number of cells/beams that UE is capable of monitoring. In terms of measurement period, RSRP values that UE is going to report would have already changed during measurement, and it means RSRP difference between reported value and actual latest value would increase if measurement period is too long. Therefore, from system performance point of view, too large values of N resulting too long delay requirements should be avoided. Based on the discussion in the last several meetings, some companies proposed N = 8 as scaling factor for PSS/SSS detection and measurement to obtain flexibility on UE implementation, but N would not need to reflect actual number of Rx beams. If UE has larger number of Rx beams, UE could complete measurements with fewer samples per beam since UE would have benefit from higher Rx beamforming gain. Therefore, N should be a just scaling factor to relax the requirements, and it would be important to consider final delay requirements derived from determined values, i.e. actual value of sample number × N × SMTC periodicity. Table 1 shows calculated values with respect to N and SMTC periodicity. If N would be 5 or larger, each PSS/SSS detection, SSB index acquisition, and RSRP measurement delay would be 1 sec or longer even in case of SMTC periodicity = 40 ms.

Table 1: Calculated values with respect to N and SMTC periodicity (5 × N × SMTC periodicity ms)

| SMTC periodicity | 5 ms | 10 ms | 20 ms | 40 ms | 80 ms | 160 ms |
| --- | --- | --- | --- | --- | --- | --- |
| N = 4 | 100 | 200 | 400 | 800 | 1600 | 3200 |
| N = 5 | 125 | 250 | 500 | 1000 | 2000 | 4000 |
| N = 6 | 150 | 300 | 600 | 1200 | 2400 | 4800 |
| N = 7 | 175 | 350 | 700 | 1400 | 2800 | 5600 |
| N = 8 | 200 | 400 | 800 | 1600 | 3200 | 6400 |

As agreed in [3], since UE could not perform RLM and RRM measurement simultaneously in FR2, NW would configure longer SMTC periodicity than actual SSB transmission periodicity TSSB to provide frequent opportunity for RLM. In addition, if we assume overlapping case between SMTC window and measurement gap and/or CA case, actual delay value would be much longer. In Figure 1, one of typical cases of partial overlapping is illustrated, i.e. SMTC periodicity = 1/2 × MGRP, and calculated delay value in this case is shown in Table 2. Based on this investigation, value of N should be the minimum value in the candidates, i.e. N = 4, and N = 4 would be reasonable value even if flexibility on UE implementation would be taken into consideration.

Figure 1: Typical case of partial overlapping between SMTC and measurement gap

Table 2: Calculated values with respect to N and SMTC periodicity in the case depicted in Figure 1

(5 x N x 2 x SMTC periodicity ms)

| SMTC periodicity | 5 ms | 10 ms | 20 ms | 40 ms | 80 ms | 160 ms |
| --- | --- | --- | --- | --- | --- | --- |
| N = 4 | 200 | 400 | 800 | 1600 | 3200 | 6400 |
| N = 5 | 250 | 500 | 1000 | 2000 | 4000 | 8000 |
| N = 6 | 300 | 600 | 1200 | 2400 | 4800 | 9600 |
| N = 7 | 350 | 700 | 1400 | 2800 | 5600 | 11200 |
| N = 8 | 400 | 800 | 1600 | 3200 | 6400 | 12800 |

Regarding SSB time index acquisition, UE would have some knowledge about Rx beam UE could utilize for that cell since UE has already completed PSS/SSS detection. Thus, value of scaling factor N2 could be small, e.g. 1 or 2. In the previous meetings, some companies had argued that they had concern on N2 = 1 and some margin would be needed. Therefore, scaling factor for SSB time index acquisition could be N2 = 2.

Proposal 1: For FR2, values of scaling factor for PSS/SSS detection delay, N1, SSB time index acquisition delay, N2, and measurement period, N3, would be expressed as following.

N1 = 4, N2 = 2, N3 = 4

Requirements in case of CA

At RAN4#86bis meeting, it was agreed that PSell/PSCell measurement was not relaxed even if some SCell measurements are configured. However, scaling factor considering different SMTC configurations on each carrier was not decided. In addition, some companies insisted their concerns on applying same requirements for FR2 as that for FR1 because UE might not always have more than 1 cell searcher for FR2. Thus, requirements of intra-frequency measurement with SCell configured for FR2 were still unclear. Latest specification was captured in [1] as following.

TS38.133 [1] The UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_without_index if UE is not indicated to report SSB based RRM measurement result with the associated SSB index, or the UE has been indicated that the neighbour cell is synchronous with the serving cell. Otherwise UE shall be able to identify a new detectable intra frequency cell within Tidentify_intra_with_index. The UE shall be able to identify a new detectable intra frequency SS block of an already detected cell within Tidentify_intra_without_index. Tidentify_intra_without_index = Kca (TPSS/SSS_sync + T SSB_measurement_period) ms Tidentify_intra_with_index = Kca (TPSS/SSS_sync + T SSB_measurement_period + TSSB_time_index) ms Where: TPSS/SSS_sync: it is the time period used in PSS/SSS detection given in table 9.2.5.1-1, 9.2.5.1-2, 9.2.5.1-5 (deactivated Scell) or 9.2.5.1-6 (deactivated SCell) TSSB_time_index: it is the time period used to acquire the index of the SSB being measured given in table 9.2.5.1-3 or 9.2.5.1-4 or 9.2.5.1-7 (deactivated SCell) or 9.2.5.1-8 (deactivated SCell) T SSB_measurement_period: equal to a measurement period of SSB based measurement given in table 9.2.5.2-1, table 9.2.5.2-2 table 9.2.5.2-3 (deactivated Scell) or 9.2.5.2-4(deactivated SCell) Kca: For FR1, Kca =1 for measurements on frequencies corresponding to PCell or PSCell, and Kca =number of configured SCells for measurements on frequencies corresponding to FR1 only SCells Editor’s note : Kca for SCells on FR1 assumes that all Scell SMTC are overlapping(definition FFS). Kca definition may be revised for non overlapping Scell SMTCs. Editor’s note:Kca is FFS if any FR2 serving cells are configured

FR1

For FR1, delay requirements on PCell/PSCell are not relaxed, i.e. Kca = 1 for PCell/PSCell, and Kca for SCell would be number of configured SCells if all SMTC on configured SCells are fully overlapped. However, discussion on scaling factor in case of different SMTC configurations on configured SCells would be still ongoing. If Kca = 1 is assumed, measurement for PCell/PSCell would not be affected from other SCell measurements, and hence scaling factor for SCell measurement could be considered without taking PCell/PSCell measurement into account. Therefore, scaling factor might be derived based on similar scaling rule as inter-frequency measurement requirement as discussed in [4].

FR2

For FR2, some companies provided their concerns on number of searchers for FR2, and requirements are still FFS. Since PCell/PSCell measurement would be more important than SCell measurement, even for FR2, PCell/PSCell measurement should be prioritized.

Observation 1: For FR2, PCell/PSCell measurement should be prioritized than SCell measurement.

If UE could have more than one searcher for FR2, same requirements as that for FR1 could be applied for FR2, that is to say, requirements on PCell/PSCell would not need to be relaxed. However, scaling factor Kca could not be 1 if UE only has one searcher for FR2 since there would be no SMTC occasion utilized for SCell measurement in case of full overlap between PCell/PSCell and SCells.

Observation 2: For FR2, scaling factor Kca could be 1 for PCell/PSCell measurement if UE is assumed to have more than one searcher. Otherwise, Kca should be derived based on SMTC configurations on each carrier.

Based on above discussion, we made following proposal.

Proposal 2: For FR2, scaling factor Kca could be derived based on following manner.

- If UE could be assumed to have more than one searcher for FR2, PCell/PSCell measurement would not be relaxed, i.e Kca = 1 for PCell/PSCell, and Kca for configured SCells could be derived based on SMTC configurations on each SCell.

- If UE would be assumed to have only one searcher for FR2, Kca for PCell/PSCell and SCell measurement should be derived based on SMTC configurations on each cell.

  - Measurement for PCell/PSCell should be prioritized than SCells, i.e. Kca for PCell/PSCell < Kca for SCell

- Conclusion

In this contribution, we provided our views on cell identification delay requirements for FR2, and we made following observations and proposals.

Proposal 1: For FR2, value of scaling factor N for PSS/SSS detection delay, N1, SSB time index acquisition delay, N2, and measurement period, N3, would be expressed as following.

N1 = 4, N2 = 2, N3 = 4

Observation 1: For FR2, PCell/PSCell measurement should be prioritized than SCell measurement.

Observation 2: For FR2, scaling factor Kca could be 1 for PCell/PSCell measurement if UE is assumed to have more than one searcher. Otherwise, Kca should be derived based on SMTC configurations on each carrier.

Proposal 2: For FR2, scaling factor Kca could be derived based on following manner.

- If UE could be assumed to have more than one searcher for FR2, PCell/PSCell measurement would not be relaxed, i.e Kca = 1 for PCell/PSCell, and Kca for configured SCells could be derived based on SMTC configurations on each SCell.

- If UE would be assumed to have only one searcher for FR2, Kca for PCell/PSCell and SCell measurement should be derived based on SMTC configurations on each cell.

  - Measurement for PCell/PSCell should be prioritized than SCells, i.e. Kca for PCell/PSCell < Kca for SCell

# Reference

- 3GPP, Draft TS38.133, v15.2.0

- 3GPP, R4-1802536, NTT DOCOMO, “Remaining issues on intra frequency measurement requirements without measurement gap,” Feb. 2018

- 3GPP, R4-1805541, NTT DOCOMO, “WF on collision issue among RLM-RS, SMTC and measurement gap,” Apr. 2018

- 3GPP, R4-1805521, Mediatek, Huawei, HiSilicon, “Wayforward on requirement of intra-frequency measurement without gap,” Apr. 2018