3GPP TSG-RAN WG4 Meeting AH1807	R4-1808803

Montreal, Canada, 2 – 6 July 2018

Agenda Item:	5.4.1.6

Source:	Ericsson

Title:	Demodulation performance impacts due to simplified propagation channel models

Document for:	Discussion

# 1	Introduction

RAN4#87 discussed the simplification of TDL channel models for NR UE demodulation and CSI reporting requirements and agreed with the way forward [1].

Channel model simplifications - Simplify the existing 38.901 TDL channel models by choosing strongest paths - Option 1: Choose strongest paths that contribute to [95%] of total power. - Examples of simplified profiles for Option1 provided in the next slides - Option 2: Choose [7] strongest paths that contribute for NLOS PDPs - Apply normalization of the normalized DS after removing the weak paths (DS RMS = 1). - Use equidistant delay modelling grid for TDL channel models after DS scaling with grid step ΔT ≤ 1/BW. - BW = [200] MHz - Paths that end up with the same delay will be combined into a single path by adding their respective powers - Note: Initial simulations for July AH can be done based on non-simplified TDL models

In this contribution, we compare the option 1 and option 2 with regard to the UE demodulation performance.

# 2	Simplified TDL propagation channel models

## 2.1	Simplification procedure

As RAN4#87 discussed, the resolution of delay path grids for TDL channel models in TR38.901 [2] is 1 pico-second (1ps). To avoid the complexity of the test setting, RAN4 discussed to assume the simplification of the channel model.

The simplification method consists of:

Step 1: Remove the path if the path power is less than threshold. Trimming method 1: Choose strongest paths that contribute to [X=95%] of total power. Trimming method 2: Choose [Y=7] strongest paths that contribute for NLOS PDPs. Step 2: Apply the desired delay spread after removing the weak paths. Step 3: Decide the path grid based on resolution, and round the path delay position close to the grid. Set the grid step size to [5] ns. Step 4: If two or more paths are rounded to the same grid τ, the path power at τ is set as a sum of all the path power values (in linear) rounded to τ. Step 5: Normalize the power so that the relative power of the strongest path becomes 0dB.

## 2.2	Simplified TDL-A (DS=30ns)

Figure 1 compares two trimming methods for the simplified TDL-A channel model according to the method in 2.1 for desired delay spread of 30ns. It is observed from our calculation that 95% of total path corresponds to 14 strong paths. Comparing two methods (7 strong paths vs. 14 strong paths), the left figure is more frequency-selective compared with 14 paths cases. In our observation, TDL-A is less frequency-selective, if we respect the original frequency correlation characteristics, we need to keep larger paths. As a compromise, we observe 11 strong paths (Figure 2) is a good compromise regarding the number of paths and frequency correlation characteristics.

Table 1 shows the path positions and their relative powers for simplified TDL-A channel models.

Figure 1	Example of simplified TDL-A (Desired DS=30ns); Method 2 with Y=7 paths (left) and Method 1 with X=95% (right).

Figure 2	Example of simplified TDL-A (Desired DS=30ns); Method 2 with Y=11 paths (X=90% in Method 1).

Table 1	Tap delay and relative power for the simplified TDL-A with Desired DS=30ns

| Y = 7 paths |  | Y = 11 paths |  | Y = 14 paths |  |
| --- | --- | --- | --- | --- | --- |
| X = 81% |  | X = 90% |  | X = 95% |  |
| Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 30 | 0 | 20 | 0 | 0 | -15.4 |
| 35 | -8 | 25 | -6 | 15 | 0 |
| 40 | -10.2 | 30 | -5.2 | 20 | -6 |
| 45 | -6 | 35 | -7.6 | 25 | -4.3 |
| 55 | -9.5 | 95 | -8.6 | 30 | -9.5 |
| 140 | -8.6 | 125 | -12.8 | 75 | -8.6 |
|  |  | 150 | -13.3 | 85 | -14.4 |
|  |  |  |  | 100 | -12.8 |
|  |  |  |  | 120 | -13.3 |
|  |  |  |  | 160 | -14.7 |
|  | DS=29.1ns |  | DS=30.1ns |  | DS=30.1ns |

## 2.3	Simplified TDL-C (DS=300ns)

We have done the same exercise for TDL-C DS=300ns case. Figure 3 compares two trimming methods for simplified TDL-C channel models with desired DS of 300ns. We observed 15 strong paths corresponds to keep 95% of total power. The figure shows TDL-C with 7 strong paths is too frequency selective compared with the original TDL-C model. According to our observation selecting 11 strong paths could be a good compromise with regard to the number of paths and the frequency correlation characteristic as shown in Figure 4.

Table 2 gives the delay tap positions and their relative powers.

Figure 3	Example of simplified TDL-C (Desired DS=300ns); Method 2 with Y=7 paths (left) and Method 1 with X=95% (right).

Figure 4	Example of simplified TDL-C (Desired DS=300ns); Method 2 with Y=11 paths (corresponding to X=87% in Method 1).

Table 2	Tap delay and relative power for the simplified TDL-C with Desired DS=300ns

| Y = 7 paths |  | Y = 11 paths |  | Y = 15 paths |  |
| --- | --- | --- | --- | --- | --- |
| X = 70% |  | X = 87% |  | X = 95% |  |
| Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] | Tap delay [ns] | Relative power [dB] |
| 0 | -4.4 | 0 | -4.4 | 0 | -4.4 |
| 265 | -1.2 | 180 | -1.2 | 150 | -1.2 |
| 275 | -2.5 | 190 | -2.5 | 155 | -2.5 |
| 280 | -3.5 | 195 | -3.5 | 160 | -3.5 |
| 800 | 0 | 200 | -5.2 | 165 | -5.2 |
| 810 | -2.2 | 555 | 0 | 455 | 0 |
| 820 | -3.9 | 560 | -2.2 | 460 | -2.2 |
|  |  | 570 | -3.9 | 465 | -3.9 |
|  |  | 690 | -7.1 | 470 | -7.4 |
|  |  | 1070 | -5.1 | 565 | -7.1 |
|  |  | 1135 | -6.8 | 585 | -10.7 |
|  |  |  |  | 665 | -11.1 |
|  |  |  |  | 875 | -5.1 |
|  |  |  |  | 930 | -6.8 |
|  |  |  |  | 1545 | -8.7 |
|  | DS=299.8ns |  | DS=300.3ns |  | DS=300.3ns |

# 3	Simulation results

We have compared different channel models with two PDSCH demodulation configurations: QPSK and 256QAM as shown in Table 3. Figure 5 and Figure 6 show our ideal simulation results with different channel models. Note the metric PDSCH BLER is given by the number of NACKs over the number of PDSCH transmissions including up to 4 HARQ retransmissions)

For TDL-A (Figure 5), QPSK result shows X=95% and X=80% are deviated from the original PDP, and 256QAM result shows X=95% case is also deviated but not so large deviation compared with X=90% and X=81%.

For TDL-C (Figure 6), 256QAM result shows X=70% model is deviated significantly compared with the original PDP model.

Observation: Simplified TDL-A/TDL-C models with 7 strongest paths shows significant deviation compared with the simulation results with original channel models.

Table 3	Simulation parameters for PDSCH

|  | Case 1 | Case 2 |
| --- | --- | --- |
| Propagation channel | Model 1: TDL-A DS=30ns, Doppler=10Hz Model 2: TDL-C DS=300ns, Doppler=100Hz |  |
| Frequency range | FR1 | FR1 |
| UE system bandwidth | 100MHz | 100MHz |
| SCS | 30kHz | 30kHz |
| Antenna configuration and MIMO layers | 2x2 Low, 2 layers | 2x2 Low, 2 layers |
| MCS | MCS2 (QPSK 0.33) | MCS23 (256QAM 0.75) |
| Max HARQ transmission | 4 | 4 |

Figure 5	Simulation results for TDL-A.

Figure 6	Simulation results for TDL-C.

# 4	Conclusion

Observation: Simplified TDL-A/TDL-C models with 7 strongest paths shows significant deviation compared with the simulation results with original channel models.

Proposal: RAN4 considers the simplified TDL-A channel model with X=87% in Method 1 and TDL-C channel model with X=90% in Method 1.

# 5	References

R4-1808028, “Way forward on channel models for NR UE demodulation requirements”, Intel.

3GPP TR38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”.

# Appendix

Table 7.7.2-1. TDL-A

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | -13.4 | Rayleigh |
| 2 | 0.3819 | 0 | Rayleigh |
| 3 | 0.4025 | -2.2 | Rayleigh |
| 4 | 0.5868 | -4 | Rayleigh |
| 5 | 0.4610 | -6 | Rayleigh |
| 6 | 0.5375 | -8.2 | Rayleigh |
| 7 | 0.6708 | -9.9 | Rayleigh |
| 8 | 0.5750 | -10.5 | Rayleigh |
| 9 | 0.7618 | -7.5 | Rayleigh |
| 10 | 1.5375 | -15.9 | Rayleigh |
| 11 | 1.8978 | -6.6 | Rayleigh |
| 12 | 2.2242 | -16.7 | Rayleigh |
| 13 | 2.1718 | -12.4 | Rayleigh |
| 14 | 2.4942 | -15.2 | Rayleigh |
| 15 | 2.5119 | -10.8 | Rayleigh |
| 16 | 3.0582 | -11.3 | Rayleigh |
| 17 | 4.0810 | -12.7 | Rayleigh |
| 18 | 4.4579 | -16.2 | Rayleigh |
| 19 | 4.5695 | -18.3 | Rayleigh |
| 20 | 4.7966 | -18.9 | Rayleigh |
| 21 | 5.0066 | -16.6 | Rayleigh |
| 22 | 5.3043 | -19.9 | Rayleigh |
| 23 | 9.6586 | -29.7 | Rayleigh |

Table 7.7.2-2. TDL-B

| Tap # | Normalized delay | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0.0000 | 0 | Rayleigh |
| 2 | 0.1072 | -2.2 | Rayleigh |
| 3 | 0.2155 | -4 | Rayleigh |
| 4 | 0.2095 | -3.2 | Rayleigh |
| 5 | 0.2870 | -9.8 | Rayleigh |
| 6 | 0.2986 | -1.2 | Rayleigh |
| 7 | 0.3752 | -3.4 | Rayleigh |
| 8 | 0.5055 | -5.2 | Rayleigh |
| 9 | 0.3681 | -7.6 | Rayleigh |
| 10 | 0.3697 | -3 | Rayleigh |
| 11 | 0.5700 | -8.9 | Rayleigh |
| 12 | 0.5283 | -9 | Rayleigh |
| 13 | 1.1021 | -4.8 | Rayleigh |
| 14 | 1.2756 | -5.7 | Rayleigh |
| 15 | 1.5474 | -7.5 | Rayleigh |
| 16 | 1.7842 | -1.9 | Rayleigh |
| 17 | 2.0169 | -7.6 | Rayleigh |
| 18 | 2.8294 | -12.2 | Rayleigh |
| 19 | 3.0219 | -9.8 | Rayleigh |
| 20 | 3.6187 | -11.4 | Rayleigh |
| 21 | 4.1067 | -14.9 | Rayleigh |
| 22 | 4.2790 | -9.2 | Rayleigh |
| 23 | 4.7834 | -11.3 | Rayleigh |

Table 7.7.2-3. TDL-C

| Tap # | Normalized delays | Power in [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -4.4 | Rayleigh |
| 2 | 0.2099 | -1.2 | Rayleigh |
| 3 | 0.2219 | -3.5 | Rayleigh |
| 4 | 0.2329 | -5.2 | Rayleigh |
| 5 | 0.2176 | -2.5 | Rayleigh |
| 6 | 0.6366 | 0 | Rayleigh |
| 7 | 0.6448 | -2.2 | Rayleigh |
| 8 | 0.6560 | -3.9 | Rayleigh |
| 9 | 0.6584 | -7.4 | Rayleigh |
| 10 | 0.7935 | -7.1 | Rayleigh |
| 11 | 0.8213 | -10.7 | Rayleigh |
| 12 | 0.9336 | -11.1 | Rayleigh |
| 13 | 1.2285 | -5.1 | Rayleigh |
| 14 | 1.3083 | -6.8 | Rayleigh |
| 15 | 2.1704 | -8.7 | Rayleigh |
| 16 | 2.7105 | -13.2 | Rayleigh |
| 17 | 4.2589 | -13.9 | Rayleigh |
| 18 | 4.6003 | -13.9 | Rayleigh |
| 19 | 5.4902 | -15.8 | Rayleigh |
| 20 | 5.6077 | -17.1 | Rayleigh |
| 21 | 6.3065 | -16 | Rayleigh |
| 22 | 6.6374 | -15.7 | Rayleigh |
| 23 | 7.0427 | -21.6 | Rayleigh |
| 24 | 8.6523 | -22.8 | Rayleigh |