3GPP TSG-RAN WG4 Meeting AH1807	R4-1809249

## Montreal, Canada, 02nd – 06th July, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | CRNum | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.843 - OTA Co-location Spurious Emissions in a Near Field Test Range |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MVG Industries |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Adding measurement procedure for OTA Co-location Spurious Emissions in a Near Field Test setup |  |  |  |  |  |  |  |  |
| Summary of change: |  | The addition of text for OTA Co-location Spurious Emissions measurement procedure for co-location requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test procedure is needed for OTA Co-location Spurious Emissions measurement with respect to the Near Field test method. If not included, then Near Field test method is excluded from this OTA requirement for conformance requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.6.X.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

#### 10.6.X.4	Near Field

##### 10.6.X.4.1	General

This method measures the OTA co-location spurious emissions in a Near Field Test Range. An example of a Near Field measurement setup for co-location requirements is shown in Figure 10.6.X.4.1-1

# Figure 10.6.X.4.1-1: Example of a Near Field measurement setup for co-location measurements

##### 10.6.X.3.2	Calibration

Calibration for wanted signal power level is the same as in subclause 10.2.2.4.2.

Additionally, the losses in the signal chain between co-location reference antenna conducted output(s) and measurement equipment need to be calibrated out:

- Path loss calibration

- Measure the loss in signal chain from each output of co-location reference antenna to input of measurement equipment, call this L ANT↔C which is the equivalent of 20log|S21| from the use of a network analyser.

##### 10.6.X.3.3 	Procedure

- Place AAS BS and co-location reference antenna as specified in subclause 4.15, 3GPP TS 37.145-2 [24].

- Connect Measurement antenna and co-location reference antenna to the measurement equipment.

- OTA co-location spurious emissions are measured at the co-location reference antenna conducted output(s).

- The measurement device (spectrum analyser) characteristics shall be:

-  Detection mode: True RMS.

- Set the AAS BS to transmit:

-	For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, either simultaneously or sequentially, in boresight direction (reference direction), at manufacturer's declared rated output power, Prated,t,TRP.

-	For AAS BS declared to be capable of multi-carrier and/or CA operation, set the AAS BS to transmit full beam power (rated beam EIRP), both polarizations, in boresight direction (reference direction) on all carriers configured using the applicable test configuration and corresponding power setting.

-	Check that specified beam power (EIRP) is obtained for each polarization. Re-align if the specified beam power is not achieved.

- This can be done by running a full EIRP measurement for the declared beam. Refer to 10.2.2.4.3 for the EIRP measurement procedure

- Measure the mean power spectral density from all co-location reference antenna conducted output(s) with each wanted signal polarization under test.

##### Additional factor to be considered in the measurement is that the test requirement is very close to thermal noise floor, and the measurement setup needs to be able to tolerate both very high and low signal levels. This will impact measurement uncertainty.

##### 10.6.X.4.4 	MU assessment

###### 10.6.X.4.4.1 	MU Budget

###### 10.6.X.4.4.2 	MU Value

#### 10.6.X.x	Summary

{editors note: Last section at this level summarises MU estimates for all chambers and concludes final agreed MU}