3GPP TSG-RAN WG4 Meeting NR#2                                   R4-1706649

Qingdao, China, 27th – 29th June, 2017

Agenda item:			3.3.3.1

Source:			Samsung

Title:	High Power UE for 3.5GHz Band

Document for:			Discussion

# 1	Introduction

In RAN4 #83 meeting, the related issues on high power UE for 3.5GHz are discussed, with the following WF agreed [1]:

It is proposed to specify both PC2 and PC3 UE for 3.5GHz NR frequency range in Rel-15 to accommodate different operator demands. The high power UE (PC2 UE) specific work plan in Rel-15 is shown in slide 3.

In this contribution, we would like to provide our technical inputs for related 3.5GHz high power UE.

# Co-existence Study Necessity

As stated in the above 3.5GHz HPUE work plan, the co-existence study under the deployment scenarios should be clarified firstly, which could potentially be the basis for other RF requirements. Obviously, the study for LTE B41 PC2 HPUE could be a comparable baseline, and the necessity of introducing new co-existence study is given as below.

By review the study for LTE B41 PC2 HPUE, the following scenarios studied are summarized in the following table 1 (i.e., Table 5.8.1.3-1 [2]), while the summarized simulation results for ACLR is provided in table 2 (i.e., Table 5.8.2-1 [2]), and the following key differences are highlighted for 3.5GHz HPUE afterwards.

Table 1: Inter-site distances and Propagation model

| Environment | ISD (KM) | ISD (miles) |
| --- | --- | --- |
| Urban | .75 | .47 |
| Suburban | 2.8 | 1.74 |
| Rural | 6 | 3.73 |
| Rural | 8 | 5 |

Table 2: Summary of ACLR simulations (additional E-UTRA ACLR needed)

|  | 20 MHz |  | 10 MHz |  |
| --- | --- | --- | --- | --- |
| ISD | Avg | 5th percentile | Avg | 5th percentile |
| 750 meter | 0.36 | 0.83 | 0.16 | 0.14 |
| 2.8 km | 0.65 | 0.61 | 0.18 | 0.00 |
| 6 km | 0.31 | 0.42 | 0.72 | 0.00 |
| 8 km | 0.15 | 0.09 | 0.37 | 0.00 |

Interference Scenarios:

Another big difference is the interference scenarios for 3.5GHz HPUE NR deployment. Different from B41 LTE HPUEs, which are introduced after business deployment of legacy PC3 B41 HPUE, the co-existence study’s aim is to make sure the interference brought by HPUE to adjacent network with PC3 UEs can be roughly maintained in the similar level. However, for NR 3.5GHz HPUE, the scenario could be quite different, since we have noted that 3.5GHz NR HPUE could probably be deployed firstly without corresponding normal power class UEs utilized yet, e.g., in China. For other countries without the need for HPUE, e.g., Japan, there is no co-existence issue of HPUE with normal power class UE in NR 3.5GHz band.

Proposal 1: RAN4 should firstly clarify the necessity of the coexistence scenario of HPUE interfering UL signals of normal power class UEs in adjacent band/cell, which corresponding to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters.

In other words, except operators confirmed that the coexistence scenario of HPUE interfering UL signals of normal PC UEs in adjacent band/cell, the above-mentioned co-existence study similarly done in B41 HPUE is not necessary. If this scenario cannot be confirmed, the following procedure should be considered:

Step-1: NR 3.5GHz UL to UL coexistence in which both interference and victim system use normal power class TX power.

Step-2: NR 3.5GHz UL to UL coexistence in which both interference and victim system use HPUE TX power.

Step-3: Compare the victim system performance degradation obtained from Step-1 and 2, choose HPUE UE ACLR value so that the victim system degradation due to HPUE in Step-2 is the same as Step-1.

Proposal 2: If the above three steps (Step-1: PC3 to PC3; Step-2: PC2 to PC2; Step-3: compare Step-1 and 2) are adopted, the original 30dBc should be enough to guarantee coexistence study.

Other technical inputs:

If the coexistence study is agreed to be conducted, the technical inputs captured in Appendix should be considered.

# SAR for HPUE with NR-LTE DC

The SAR related issues for NR-LTE DC has been discussed for replying RAN2’s LS in RAN4#82, however, more studies are needed on related issues. Here we would like to introduce the procedure and details of SAR measurements.

For DUEs with simultaneous multi-band transmission, generally, the impacts from all operational bands should be taken into account. Specifically for NR-LTE DC, considering the frequency separation of NR and LTE bands normally is large enough (i.e., larger than the valid frequency range of probe calibration which is typical narrow as +/- 50MHz for electric field probes), the procedure presented as below should be followed [6]. It should be noted that each of the four alternatives listed below can be followed:

1) summation of peak spatial-averaged SAR values – simplest but most conservative method to find upper bound (Section 6.3.2.2 [6]); 2) selection of highest assessed maximum SAR values – accurate estimate when separate maxima are far apart and do not effect each other by more than 5 % (Section 6.3.2.3 [6]); 3) calculation of multi-band SAR from existing area and zoom scans – accurate and fast method, always applicable (Section 6.3.2.4 [6]); 4) volumetric scanning – most accurate, always applicable (6.3.2.5 [6]).

Considering the details of SAR procedure is somehow out of the scope of RAN4 discussion, we would like to provide the general summary for the relationship among these four methods:

- Alternative 1) is the most conservative one and identified as the reference method.

- Alternative 2) is only applicable when the separately measured SAR zoom scan distributions have little or no overlap.

- Alternative 3) is conducted based on existing area and zoom scans for each operational frequency with interpolation and extrapolation for generation of volumetric SAR data is utilized, thus resulting a rapid way of obtaining the multi-band SAR.

- Alternative 4) is the most accurate method to assess the multi-band SAR in which the points in the volumetric grid which encompasses the zoom scans at all frequencies will be measured.

- It should be noted that except Alternative 2), all the other three alternatives are applicable all the time.

Based on the above procedure analysis, the next question we would like to answer is whether or not the following relationship (provided in [7]) is valid theoretically:

10GHz

∑ ( SAR,i )  ≤  SAR,l

i=100KHz

Where:

SAR,i is the SAR caused at the frequency i

SAR,l is the regulatory SAR limit given in the regulatory tables

Obviously, the above equation follows the most conservative test method, i.e., Alternative 1), in which all the peak spatial-average SAR values at each frequency points. However, the following observations should also be noted:

- It should be noted that for simultaneous transmissions with NR-LTE DC operation, the resultant peak spatial-average SAR values are not necessarily the same for NR and LTE bands, which is obviously the reason why Alternative 1) is regarded as the most conservative approach.

- Even for Alternative 1), there are at least two variances:

Alternative 1)-a 1) For each test condition where simultaneous operation is intended, add the peak spatial-average SAR values at each frequency, f1, f2, etc (see Note below). 2) If the maximum summed SAR value is within -3 dB of the compliance limit, additional measurements at lower and upper frequencies shall be conducted for this test condition. These additional values shall be used to determine the maximum SAR. 3) The maximum summed SAR value in step 1 or 2 is the multi-band SAR. Alternative 1)-b (captured in the Note of [7]) An acceptable variation of step 1 is to add the highest peak spatial-average SAR values regardless of test condition. In other words, add the highest peak spatial-average SAR at the frequency f1 (among all test conditions at that frequency) to the highest peak spatial-average SAR value at the frequency f2 (among all test conditions), and so on for any other frequencies. Steps 2 and 3 must still be followed using this method. This method is more conservative than the method of step 1.

Considering the peak spatial-average SAR values at LTE and NR bands may not be achieved at the same test condition (or modes), the above equation could be more comparable to Alternative-1)-b, which is even more conservative than Alternative 1)-a.

Observation 1: Based on current SAR measurement procedure for simultaneous transmission at LTE and NR bands, to apply the accumulative equation at each operational band is the most conservative method.

# Conclusion

In this contribution, we would like to provide our technical inputs for related 3.5GHz high power UE.

Proposal 1: RAN4 should firstly clarify the necessity of the coexistence scenario of HPUE interfering UL signals of normal power class UEs in adjacent band/cell, which corresponding to the coexistence of two commercial networks operating in adjacent channel and with similar deployment parameters.

Proposal 2: If the above three steps (Step-1: PC3 to PC3; Step-2: PC2 to PC2; Step-3: compare Step-1 and 2) are adopted, the existing ACLR 30dBc should be enough to guarantee coexistence performance.

Observation 1: Based on current SAR measurement procedure for simultaneous transmission at LTE and NR bands, to apply the accumulative equation at each operational band is the most conservative method.

# Reference

[1] R4-1706068, “WF on high power UE for 3.5GHz”, CMCC, Skyworks, Qorvo, Broadcom, MediaTek, Vodafone, CATT, Huawei, HiSilicon, ZTE.

[2] 3GPP TR 36.886, “Evolved Universal Terrestrial Radio Access (E-UTRA); Band 41 High Power UE (HPUE)”.

[3] Report ITU-R M.2135-1, “Guidelines for evaluation of radio interface technologies for IMT-Advanced”.

[4] H. Xia, "A simplified analytical model for predicting path loss in urban and suburban environments," in IEEE Transactions on Vehicular Technology, vol. 46, no. 4, pp. 1040-1046, Nov 1997.

[5] Report ITU-R SM.2028-1, “Monte Carlo simulation methodology for the use in sharing and compatibility studies between different radio services or systems”.

[6] BS EN 62209-2:2010, “Human exposure to radio frequency fields from handheld and bodymounted wireless communication devices — Human models, instrumentation, and procedures”.

[7] R4-1700446, “NSA and NR – UE maximum output power”, InterDigital.

# Appendix

If the coexistence study is agreed to be conducted, the following technical inputs should be considered.

Frequency Band and Propagation Model:

One direct difference of frequency band from 2.6GHz to 3.5GHz, as expected, is the different pathloss models and frameworks to be utilized:

For rural model, in LTE B41 HPUE discussion, the modified Hata model from Report ITU-R SM.2028-1 is adopted, while it should be noted that the modified Hata model is explicitly NOT advised for above 3GHz frequency [5]. Based on this, the widely used ITU model from M.2135 can be utilized as below:

| Scenario |  | Path loss (dB) Note: fc is given in GHz and distance in m! | Shadow fading std (dB) | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| Rural Macro (RMa) | LoS | PL1 = 20 log10(40d fc/3) + min(0.03h1.72,10) log10(d) – min(0.044h1.72,14.77) + 0.002 log10(h)d PL2 = PL1 (dBP) + 40 log10(d/dBP) | = 4 = 6 | 10 m < d < dBP (4) dBP < d < 10 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m (Applicability ranges of h, W, hBS, hUT are same as UMa NLoS) |
|  | NLoS | PL = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d) 3) + 20 log10(fc) – (3.2 (log10 (11.75 hUT)) 2 4.97) | = 8 | 10 m < d < 5 000 m, hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 5 m (The applicability ranges of h, W, hBS, hUT are same as UMa NLoS) |

(4)	Break point distance dBP  = 2π hBS hUT fc/c, where fc is the centre frequency in Hz, c = 3.0  108 m/s is the propagation velocity in free space, and hBS and hUT are the antenna heights at the BS and the UT, respectively.

On the other hand, for urban and suburban environment, in LTE B41 HPUE analysis, the propagation model derived from TR 36.942 (also TR 25.942) is utilized, which from our understanding comes from the Xia’s literature [4]. However, if we further investigate the original literature, the applicability of the proposed model is questionable, since the model is mainly designed for personal communications services in 1990 era, when the operational frequency is much lower than 3.5GHz. To keep the consistence with the above rural scenarios, we suggest to use urban and suburban model also from ITU-R report M.2135-1, i.e.,

| Scenario |  | Path loss (dB) Note: fc is given in GHz and distance in m! | Shadow fading std (dB) | Applicability range, antenna height default values |
| --- | --- | --- | --- | --- |
| Urban Macro (UMa) | LoS | PL = 22.0 log10(d) + 28.0 + 20 log10(fc) | = 4 = 4 | 10 m < d < d′BP (1) d′BP < d < 5 000 m(1) hBS = 25 m(1), hUT = 1.5 m(1) |
|  | NLoS | PL = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d) 3) + 20 log10(fc) – (3.2 (log10 (11.75 hUT))2 4.97) | = 6 | 10 m < d < 5 000 m h = avg. building height W = street width hBS = 25 m, hUT = 1.5 m, W = 20 m, h = 20 m. The applicability ranges: 5 m < h < 50 m 5 m < W < 50 m 10 m < hBS < 150 m 1 m < hUT < 10 m |
| Suburban Macro (SMa, optional) | LoS | PL1 = 20 log10(40d fc/3) + min(0.03h1.72,10) log10(d) – min(0.044h1.72,14.77) + 0.002 log10(h)d PL2 = PL1 (dBP) + 40 log10(d/dBP) | σ = 4 σ = 6 | 10 m < d < dBP (4) dBP < d < 5 000 m hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 10 m (The applicability ranges of h, W, hBS, hUT are same as in UMa NLoS) |
|  | NLoS | PL = 161.04 – 7.1 log10 (W) + 7.5 log10 (h) – (24.37 – 3.7(h/hBS)2) log10 (hBS) + (43.42 – 3.1 log10 (hBS)) (log10 (d) 3) + 20 log10(fc) – (3.2 (log10 (11.75 hUT))2 4.97) | = 8 | 10 m < d < 5 000 m hBS = 35 m, hUT = 1.5 m, W = 20 m, h = 10 m (Applicability ranges of h, W, hBS, hUT are same as in UMa NLoS) |

(1)	Break point distance d′BP  = 4 h′BS h′UT fc/c, where fc is the centre frequency (Hz), c = 3.0  108 m/s is the propagation velocity in free space, and h′BS and h′UT are the effective antenna heights at the BS and the UT, respectively. The effective antenna heights h′BS and h′UT are computed as follows:

h′BS = hBS – 1.0 m, h′UT = hUT – 1.0 m

where: hBS and hUT  are the actual antenna heights, and the effective environment height in urban environments is assumed to be equal to 1.0 m.

If scenarios of rural, urban and suburban are agreed to be simulated for co-existence study, the pathloss models from ITU-R M.2135-1 is adopted.

The Necessity of Rural Deployment Scenarios:

Although the deployment of NR in 3.5GHz depends on operators’ demand, we still expect its corresponding cell coverage will be well reduced: (a) the propagation loss in the frequency band of 3.5GHz will be increased; (b) the bandwidth usage for NR will probably be increased thus resulting the decreased power density in the receiver. Considering this fact, the rural scenario could be de-prioritized.

The necessity of rural deployment scenarios in co-existence study should depends on operators’ demand, with the fact that much reduced cell coverage expected for 3.5GHz NR into account.