3GPP TSG-RAN WG4 Meeting NR #2                 	R4-1706793

Qingdao, China, 27 – 29, June, 2017

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	UE Measurement capability

Agenda item:	3.5.4

Document for:	Discussion

# 1	Introduction

In last RAN4 meeting in Hangzhou, China, discussions on the UE measurement capability continued. In the agreed WF [R4-1704806] following was agreed related to UE measurement capabilities:

- Companies are encouraged to discuss whether RAN4 should involved in discussion of definitions of intra-frequency and inter-frequency.

- Further evaluate the capabilities for number of interfrequency ad inter-RAT frequency layers to measure

- Further evaluate the need for FDD/TDD differentiation in the number of carriers to measure

- Further discussion on the methodology for determining capability for number of cells, number of beams etc to measure

In this paper, we will discuss these topics in more details.

# 2	Intra-frequency and Inter-frequency discussion

When discussing intra-frequency and inter-frequency measurement it seems natural to look at how intra-frequency and inter-frequency measurements are defined legacy systems. Using LTE as baseline, the understanding of the intra-frequency cells is that those are cells deployed using the same center frequency as the serving cell. Otherwise, the cells are seen as inter-frequency cells.

RAN1 has decided that the cells are detected and identified by SS-Block. Additionally, RAN1 has also introduced SS-Block-RSRP for cell measurements. RAN1 has also decided that there can be one or more SS-blocks within the cell BW.

As RAN4 will be responsible for defining the UE requirements – including both intra-frequency and inter-frequency requirements – it seems rather natural that RAN4 should be involved in the discussion how to define intra-frequency and inter-frequency.

Observation 1: RAN4 should be involved in the discussion how to define intra-frequency and inter-frequency.

As the definition of intra-frequency and inter-frequency measurements may or may not be the same in NR as known from e.g. LTE, we think RAN4 should discuss and agree on a common understanding of intra-frequency and inter-frequency rather soon to ensure common understanding among all companies.

Proposal 1: RAN4 should discuss and agree on a common understanding of intra-frequency and inter-frequency.

# 3	Intra-frequency and Inter-frequency layers

Looking first at what RAN2 has been discussing related to NR and EN-DC (i.e. NR NSA Option 3), RAN2 has decided following:

- SN handles the mobility within the SCG and decides on the PSCell.

- SN does not make any decisions regarding mobility within MCG

- SN can initiate change of within the SN (including change of PSCell in NR)

- MN can also decide change  the entire SCG (i.e. SN that is used by the UE, including PSCell and all SCG SCells)

- MN and SN can configure same measurement objects, but some coordination is expected.

To initially setup EN-DC, it needs to be possible for MN to configure measurements on an NR carrier while operating in LTE. I.e. MN need to be able to configure UE with NR inter-RAT measurements. Based on such configuration the UE will initiate measurements on the given NR measurement objects. Assuming similar use of measurement events, triggers and reporting the UE will at some point report NR cells to MN based on the inter-RAT measurement results.

Observation 2: MN (LTE) will have to be able to configure NR inter-RAT measurements.

From this it is clear that new NR inter-RAT measurement requirements are needed in 36.133 for UE supporting NR. Next question is how many NR inter-RAT layers the UE shall be able to support when operating in LTE and how to decide on such requirement. Shall the NR layers be in addition to the number of layers that UE is currently required to support (i.e. number of inter-RAT layers is increased) or shall the support of NR Inter-RAT layers lead to a relaxation in the existing requirements such that the overall number of inter-RAT layers is not increased (too much)?

Once the NR PSCell has been configured, the SN can configure its own measurement configuration for NR Scell management (including change of PSCell) . For every serving NR cell, RAN4 will need to discuss and decide on the requirements related to number of intra-frequency cells the UE should be able to monitor.

Additionally, RAN4 will need to discuss the number of inter-frequency and inter-RAT layers the UE be able to monitor and how these numbers are supposed to be determined across LTE and NR RAT in terms of total number of carriers.

Considering only EN-DC (i.e. NR NSA option 3), RAN4 would not need to discuss inter-RAT frequency layers from NR point of view. This means RAN4 can focus on inter-frequency layers that UE needs to be able to monitor from NR. From RAN4 point of we think the combined UE measurement configuration should be used as base for the requirements – i.e. the combined number of inter-frequency and inter-RAT layers received from the MN measurement configuration and the SN measurement configuration.

Proposal 2: The combined UE measurement configuration (i.e. measurements configured by MN and SN) should be used as base for the requirements.

Next question is then how to combine two measurement configurations? E.g. when are two measurement configurations same or similar enough to count as one?

When it comes to the NR cell detection and measurement it is clear from the current RAN1 decisions, that the UE would at least need to be informed about the location of the center carrier frequency of the SS-Block. I.e. one could conclude that an NR Object refer to the center carrier frequency of the SS-Block.

Proposal 3: An NR measurement object refers to the center carrier frequency of the SS-Block

Once an NR Object is configured this would lead to that the UE will start to monitor the Object. As RAN2 has decided it is possible that both MN and SN configures the same measurement Object, so  it is reasonable to assume that the physical measurement of the Object only needs to be carried out once by the UE, despite the fact that the UE is configured with same Object from both MN and SN.

Observation 3: Physical measurement of an Object is only needed once although an Object is configured twice.

A measurement configuration includes more parameters than just the Object to be measured. On the other hand – most of these parameters are more related to postprocessing of the physical measurement – e.g. adding offsets to measurement results, evaluating triggering of events (including which cells may trigger an event), starting/stopping timers etc. Post-processing of course impact the UE to some extent, but this impact is less than carrying out the actual physical measurement (e.g. RF chain need not be activated, possible gap impact).

When it comes to actual physical measurements of configured Objects and how to count the number of inter-frequency and inter-RAT layers, we propose that if an Object is configured from both MN and SN it only count as one Object in the total number of effective carrier to monitor requirement.

Proposal 4: If an Object is configured from both MN and SN it only counts as one Object in requirement for the total number of effective carriers to monitor.

Rest of the measurement configuration for the Object configuration may be different based on whether the Object is configured from MN or from SN. The impact from having different parameterization will be on the total number of reporting criteria.

# 4	FDD/TDD differentiation

In LTE RAN4 have defined clear requirements regarding the number of carriers the UE shall be able to monitor per RAT group and per capability (FDD/TDD). There are requirement for UEs supporting only FDD, UEs supporting only TDD and then also UEs supporting both FDD and TDD.

Common understanding is that at least a large majority of the bands for NR will be TDD bands. This somehow indicates that the UE might have to be able to monitor more TDD carrier than FDD carriers. One approach to address the FDD/TDD differentiation, is not to have any UE FDD/TDD differentiation in number of carriers to monitor. Instead RAN4 will simply define a generic number of carrier that the UE should be able to monitor. The weighting between FDD and TDD would then be governed by the actual measurement configuration.

Proposal 5: In NR there is no FDD/TDD differentiation in the number of inter-frequency carriers the UE shall be able to monitor.

# 5	Number of Inter-frequency and Inter-RAT layers

In last meeting, the number of inter-frequency carriers the UE should be able to monitor was discussed without any agreements. Looking at the history of LTE the initial number of inter-frequency carriers for an FDD capable UE to monitor was 3. Same for TDD while for an FDD/TDD capable UE the number was 6. Over the time, it became clear that at some point these requirements would not be able to meet the needs from the operators – hence RAN4 defined new requirements in Rel12 under the IncMon WI.

Based on these earlier identified needs during the IncMon WI we believe that for NR the number of inter-frequency carriers the UE shall be able to monitor needs to be increased compared to Rel-8 LTE. The requirements would most likely depend on the outcome of the discussion related to proposal 5. If RAN4 decides to have FDD/TDD differentiation in the requirements we propose that for each duplex mode, we propose that the UE shall be able to monitor at least 6 inter-frequency carriers. If RAN4 decides not to have differentiation the number could be slightly lower than 12 inter-frequency carriers. In the end some input from operators might be useful.

Proposal 6: If FDD/TDD differentiation is applied, for each duplex mode the UE shall be able to monitor 6 inter-frequency carriers.

Proposal 7: If no FDD/TDD differentiation is applied, the number of carriers the UE shall be able to monitor could be less than 12.

# 6	Further Discussion

Related to the methodology for determining the UE capability of number of cells and beams to measure we propose to use input from the system level simulations. We have more detailed discussion related to this in other papers.

# 7	Conclusion

RAN4 has discussed the UE measurement capability for some time. In this paper, we continued the discussion by firstly looking at the actual definition of intra- and inter-frequency carrier. Then we looked at the measurement configurations and how to address the fact that same objects may be configured by MN and SN. Finally we discussed the FDD/TDD differentiation and number of inter-f and inter-RAT layers the UE would need to be able to monitor.

We make a number of proposals:

Proposal 1: RAN4 should discuss and agree on a common understanding of intra-frequency and inter-frequency.

Proposal 2: The combined UE measurement configuration (i.e. measurements configure by MN and SN) should be used as base for the requirements

Proposal 3: An NR measurement object refers to the center carrier frequency of the SS-Block

Proposal 4: If an Object is configured from both MN and SN it only count as one Object in requirement for the total number of effective carriers to monitor.

Proposal 5: In NR there is no FDD/TDD differentiation in the number of inter-frequency carriers the UE shall be able to monitor.

Proposal 6: If FDD/TDD differentiation is applied, for each duplex mode the UE shall be able to monitor 6 inter-frequency carriers.

Proposal 7: If no FDD/TDD differentiation is applied, the number of carriers the UE shall be able to monitor could be less than 12.

# 8	References

- R4-1704806, NR RRM way forward, RAN2, Ericsson, Nokia, ZTE