3GPP TSG RAN WG4 Meeting NR ad-hoc #3		R4-1709441

Nagoya, Japan, 18th – 21th September 2017

Source: 	NTT DOCOMO, INC.

Title: 	Link level evaluation results on NR-PSS/SSS detection

Agenda item:	3.5.6.2.1

Document for:	Discussion

- Introduction

In RAN4 NR-AH#2 meeting, evaluation assumptions for system level and link level simulations to discuss RRM requirements were agreed [1]. In the last RAN4 meeting, we showed initial evaluation results regarding NR-PSS/SSS detection [2]. In this contribution, we provide our additional evaluation results based on the updated link level simulation assumption [3].

- Evaluation assumptions

Table 1 to 4 show the simulation assumptions for PSS/SSS detection based on agreed simulation assumption in [3]. We select some specific parameters for initial evaluation as highlighted by red.

Table 1: Cell-specific Parameters for PSS/SSS Detection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| Carrier frequency |  | 4GHz; 30GHz | The same frequency as for Cell 1 is used |
| Subcarrier spacing | KHz | 15 kHz; 30 kHz for 4GHz; 120 kHz; 240 kHz for 30GHz | The same values as for Cell 1 are used |
| RB Utilization | % | 100 | 100 |
| Power boosting for PSS and SSS | dB | 0 | 0 |
| Power boosting for Data and Control | dB | 0 | 0 |
| Data Modulation | - | QPSK | QPSK |
| CP Length | - | Normal | Normal |
| Number of transmitted SS block within a SS burst set period (K) | - | 1 (baseline) FFS: 4 | 1(baseline) FFS: 4 |
| SS burst set periodicity | ms | 5 | 5 |
| Relative Delay of 1st Path (synchronous) | μs | 0 | CP/2 |
| Relative Delay of 1st Path (asynchronous) | ms | 0 | 1.5 |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 |
| Es/Noc for K=1 | dB | 6.4 | 1.25; FFS: 0.25; -0.75 (SINR: -6dB; FFS: -8dB, -7dB) |
| Es/Noc for K=4 | dB | 2, 6,4, -2 | 2, 1, 1, 1 (SINR: -2.12, -5.97, -4.45, -1.12); -1, 5, -3, 2 dB (SINR: -5.12, -1.97, -8.45, -0.12) |
| Number of Tx antennas | - | 1 | 1 |
| PSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| SSS Sequence ID | - | To be stated by companies together with the results (examples shows in Table 3, 4) | To be stated by companies together with the results (examples shows in Table 3, 4) |
| Propagation Condition | - | AWGN, EPA5, ETU30 TDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 CDL-A/B/C/D with 100ns for 4 GHz and 30ns for 30 GHz Note1 (secondary priority) |  |
| Ioc Model Note3 | - | AWGN noise |  |
| NOTE 1: The channel models of CDL and TDL are used for simplified link level evaluations, which are defined in TR 38.901. |  |  |  |

Table 2: UE-specific Parameters for Cell Detection

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 by the UE | Yes |
| False detect threshold | Required as in a real UE implementation |
| UE having apriority knowledge of system being synchronous or asynchronous | No |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both PSS and SSS sequence IDs. |
| CP length detection | Not simulated |
| Receive antennas | 2, 4 (uncorrelated(except for CDL model) Option 1: With receiver beamforming Option 2: Without receiver beamforming |

Table 3: Example Cell ID Combinations that can be simulated

| Case # | Cell 1 (Interferer Cell) |  | Cell 2 (Desired Cell) |  | Scenario |
| --- | --- | --- | --- | --- | --- |
| 1 | psc1 | ssc1 | psc2 | ssc2 | Synchronous |
| 2 | psc1 | ssc1 | psc1 | ssc3 | Synchronous |
| 3 | psc1 | ssc1 | psc2 | ssc2 | Asynchronous |
| 4 | psc1 | ssc1 | psc1 | ssc3 | Asynchronous |

Table 4: Example PSS, SSS indices that can be simulated simulations

| PSS Indices |  | SSS Indices |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Label | Code index | Label | Code index (m0, m1) | Cell ID | (,) |
| psc1 | 43 | ssc1 | (20, 0) | 337 | (112, 1) |
| psc2 | 86 | ssc2 | (40, 0) | 674 | (224, 2) |
|  |  | ssc3 | (35,.0) | 673 | (224, 1) |

- Evaluation results

Evaluation results on PSS/SSS detection are shown in this section. Table 5 and Table 6 show the one-shot detection rate and the 90%-ile PSS/SSS acquisition time for the case of 15 kHz SCS on 4 GHz frequency carrier, respectively. As we already provided in [2], we could observe that four cases with different cell ID combinations show almost the same cell detection performance. Therefore, we additionally evaluate cell detection performance by using different channel model, CDL-A, in the case of only Case 4.

Table 7 and Table 8 show the one-shot detection rate and the 90%-ile PSS/SSS acquisition time for Case 4 with different SCSs, respectively. It is observed that cell detection with larger SCS such as 120/240 kHz in 30 GHz carrier frequency needs more number of PSS/SSS samples to achieve reliable cell detection in fading channel due to short CP/symbol length compared with multipath delay.

Table 5. One-shot detection rate for 15 kHz SCS

| SINR | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | CDL-A |
| -6 dB | 1.00 | 0.69 | 0.61 | 1.00 | 0.69 | 0.60 | 1.00 | 0.69 | 0.60 | 1.00 | 0.69 | 0.59 | 0.66 |
| -7 dB | 1.00 | 0.63 | 0.52 | 1.00 | 0.63 | 0.51 | 0.99 | 0.63 | 0.51 | 0.99 | 0.63 | 0.52 | 0.59 |
| -8 dB | 0.99 | 0.57 | 0.44 | 0.99 | 0.56 | 0.43 | 0.94 | 0.57 | 0.41 | 0.93 | 0.56 | 0.42 | 0.53 |

Table 6. The 90%-ile PSS/SSS acquisition time for 15 kHz SCS

| SINR | Case 1 |  |  | Case 2 |  |  | Case 3 |  |  | Case 4 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | AWGN | EPA5 | ETU30 | CDL-A |
| -6 dB | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 3 |
| -7 dB | 1 | 3 | 4 | 1 | 3 | 4 | 1 | 3 | 4 | 1 | 3 | 4 | 3 |
| -8 dB | 1 | 3 | 4 | 1 | 3 | 5 | 1 | 3 | 5 | 1 | 3 | 5 | 4 |

Table 7. One-shot detection rate (Case 4, SINR = -6dB)

| SCS | AWGN | EPA5 | ETU30 | CDL-A |
| --- | --- | --- | --- | --- |
| 15 kHz | 1.00 | 0.69 | 0.59 | 0.66 |
| 30 kHz | 1.00 | 0.69 | 0.54 | 0.64 |
| 120 kHz | 1.00 | 0.66 | 0.27 | 0.61 |
| 240 kHz | 1.00 | 0.45 | 0.18 | 0.61 |

Table 8. The 90%-ile PSS/SSS acquisition time (Case 4, SINR = -6dB)

| SCS | AWGN | EPA5 | ETU30 | CDL-A |
| --- | --- | --- | --- | --- |
| 15 kHz | 1 | 2 | 3 | 3 |
| 30 kHz | 1 | 2 | 4 | 3 |
| 120 kHz | 1 | 3 | 8 | 3 |
| 240 kHz | 1 | 5 | 12 | 3 |

- Discussion

Based on evaluation results shown in Table 6, cell detection could be completed with at most 5 samples even in the case that SINR of desired cell corresponds to -8 dB. Furthermore, cell detection based on PSS/SSS could be achieved with fewer samples than LTE because NR-PSS/SSS consists of longer sequence mapped to larger number of subcarriers than LTE-PSS/SSS. Therefore, required number of samples for NR cell detection could be less than LTE cell detection.

Observation 1: NR-PSS/SSS cell detection could be achieved with fewer samples than those required for LTE PSS/SSS detection since NR-PSS/SSS consists of longer sequence mapped to larger number of subcarriers compared with LTE-PSS/SSS.

Proposal 1: Required number of samples for cell detection to derive cell detection delay requirements should be less than LTE case.

As shown in results of the previous section, larger number of samples is required for larger SCSs such as 120/240 kHz in 30 GHz carrier frequency in some channel models, especially in case of ETU30 channel compared with 15/30 kHz SCS in 4 GHz carrier frequency. ETU channel model would not be suitable for evaluation on 30 GHz carrier frequency, and hence the discussion should be based on simulation results in CDL channel model. Based on the results in CDL model, one-shot detection performance and required number of samples for PSS/SSS detection in different SCS are almost same.

Observation2: EPA5 and ETU30 would not be suitable channel model for link level simulation on 30 GHz carrier frequency case, and hence simulation results in CDL models should be discussed at least for 30 GHz carrier frequency.

In addition, regarding evaluation assumption, there would be no need to evaluate the case of K = 4 since in cell detection procedure (i.e., before identifying the cell) UE has no idea on which SS/PBCH block is actually transmitted, i.e., PSS/SSS detection relies only on single SS/PBCH block in every SS burst set.

Observation3: There would be no need to evaluate the case of K =4 since UE has no idea on which SS/PBCH block is actually transmitted in cell detection procedure.

- Conclusion

In this contribution, we showed our initial evaluation results regarding PSS/SSS detection delay based on the agreed link level simulation assumptions. Based on the evaluation results, we presented our views on the requirements for cell identification requirements and made following observations and proposals.

Observation 1: NR-PSS/SSS cell detection could be achieved with fewer samples than those required for LTE PSS/SSS detection since NR-PSS/SSS consists of longer sequence mapped to larger number of subcarriers compared with LTE-PSS/SSS.

Proposal 1: Required number of samples for cell detection to derive cell detection delay requirements should be less than LTE case.

Observation2: EPA5 and ETU30 would not be suitable channel model for link level simulation on 30 GHz carrier frequency case, and hence simulation results in CDL models should be discussed at least for 30 GHz carrier frequency.

Observation3: There would be no need to evaluate the case of K =4 since UE has no idea on which SS/PBCH block is actually transmitted in cell detection procedure.

# Reference

- 3GPP RAN4 #NR-AH2, Chairman’s note, June 2017.

- 3GPP, RAN4-1707933, NTT DOCOMO, “Link level evaluation results on NR-PSSSSS detection”, August 2017

- 3GPP, R4-1708698, Huawei, HiSilicon, NTT DOCOMO, “Updated link level simulation assumption of cell detection in NR,” August 2017.