3GPP TSG-RAN5 Meeting#83	DRAFT_R5-194351

# Reno, USA, 13th – 17th May. 2019

Title:							On the noise impact for FR2 priority 1 and 2 TRx TCs

Source:			Anritsu

Agenda Item:			5.3.5.17

Document for:			Endorsement

- Discussion

# Till now, following values are agreed [10][15].

| # Test case | # Applicable Frequency Range[GHz] | # Agreement |
| --- | --- | --- |
| # MOP # (EIRP, PC3) | # 23.45GHz ≤ f ≤ 32.125 GHz | # ΔSNR=0.1dB, Relaxation=0dB |
|  | # 32.125 GHz ≤ f ≤ 40.8 GHz | # ΔSNR=0.3dB, Relaxation=0dB |
| # MOP # (TRP, PC3) | # 23.45GHz ≤ f ≤ 32.125 GHz | # ΔSNR=0.1dB, Relaxation=0dB |
|  | # 32.125 GHz ≤ f ≤ 40.8 GHz | # ΔSNR=0.3dB, Relaxation=0dB |
| # Off power | # 23.45 GHz ≤ f ≤ 32.125 GHz | # Test cannot be carried out without relaxation. Relaxation amount is in the range 28 - 42.7dB for 400MHz BW if threshold SNR is 10dB # (waiting for input from OEM vendor, operators …) |
|  | # 32.125 GHz < f ≤ 40.8 GHz | # Test cannot be carried out without relaxation. Relaxation amount is in the range 35 - 45.9dB for 400MHz BW if threshold SNR is 10dB # (waiting for input from OEM vendor, operators …) |
| # SEM | # 23.45 GHz ≤ f ≤ 32.125 GHz | # ΔSNR=0.62dB, Relaxation=0dB |
|  | # 32.125 GHz < f ≤ 40.8 GHz | # TBD |

# SNR estimation for some remaining FR1 and FR2 TCs are provided in Table 1.

Table 1: SNR for some Priority 1 and 2 Tx test cases

| Test case | Applicable Frequency Range [GHz] | UL signal level [dBm] | Estimated SNR for total component [dB] | ΔSNR=10log10(1+1/SNR), SNR in linear and defined for total component power | Proposed relaxation of test requirement |
| --- | --- | --- | --- | --- | --- |
| SEM | 23.45 GHz ≤ f ≤ 32.125 GHz | -13dBm/1MHz | 8.1(EIRPMAX > 33dBm) 17.8(EIRPMAX ≤ 33dBm) | 0.41 0.1 | 1.9 0 |
|  | 32.125 GHz < f ≤ 40.8 GHz |  | 1.26.0(EIRPMAX > 33dBm) 19.9 (EIRPMAX ≤ 33dBm) | 0.411.0 0.1 | 8.8 0 0 |
| Rx spurious | 6 GHz ≤ f ≤ 20 GHz | -47dBm/1MHz | 3.3 | 0.41~1.0 | 6.7~2.7 |
|  | 20 GHz < f ≤ 40 GHz |  | -6.2 | 0.41~1.0 | 16.2~12.2 |
|  | 40 GHz < f ≤ 80 GHz |  | -19.6 | 0.41~1.0 | 29.6~25.6 |
| General Tx spurious | 6 GHz ≤ f ≤ 20 GHz | -13dBm/1MHz | ≥ 17 | 0.1 | 0 |
|  | 20GHz ≤ f ≤ 80 GHz |  | ≥ 10 | 0.41 | 0 |
| NOTE ChBW = 10log10(400MHz/ChBW), 9.03dB for ChBW=50MHz. NOTE 2 : According to the agreement in [7], the SNR estimation is only valid between 23.45 and 30.3 GHz. SNR needs to be revisited once the new band and its core requirement is introduced between 30.3GHz and 32.125GHz. NOTE 3 : CP : Min Peak EIRP – Multi-band relaxation – Directivity(2x8 array), ACP : CP – ACLR NOTE 4 : Different SNR from different condition of maximum EIRP came from the LNA availability . 14dB PAPR is assumed in this estimation to judge the LNA availability. |  |  |  |  |  |

Assumptions

| #1 | Assumption | Description |
| --- | --- | --- |
| #2 | Frequency ranges under consideration | All Rel-15 FR2 bands for in-band measurement. 6GHz – 80GHz for spurious measurement. |
| #3 | Size of QZ for IFF | 30 cm |
| #5 | Power range for EIRP measurements considered at the conducted reference plane | +43 dBm – Pathloss tolerable (PC3 max beam peak) |
| #6 | Temperature variation impact | +18 to +28 degrees C |
| #7 | UE power class | PC3 |
| #9 | Characterization for QoQZ for spurious measurements | N/A |

# All the SNR is provided based on the “single” test setup.

# Considering the above estimation and the discussion until now, followings are proposed.

# Proposal 1 : For PC3 SEM FR2b, no relaxation for the test requirement,  ΔSNR  = 1.0 dB for EIRPmax ≤ 43dBm

# Proposal 2 : For PC3 General Tx Spurious, no relaxation for the test requirement, ΔSNR = 0.41 dB for 6GHz ≤ f  ≤ 80GHz

# Proposal 3 : For PC3 Rx Spurious, ΔSNR = 1.0 dB and relaxation 2.7dB(6 GHz ≤ f ≤ 20 GHz), 12.2dB(20 GHz < f ≤ 40 GHz), 25.6dB(40 GHz < f  ≤ 80 GHz), or ΔSNR = 0.41 dB and relaxation 6.7dB(6 GHz ≤ f ≤ 20 GHz), 16.2dB(20 GHz < f ≤ 40 GHz), 29.6dB(40 GHz < f  ≤ 80 GHz)

# For spherical coverage, we need the discussion of MU definition (whether consider correlation of MUs between grids) [16] and power level assumption for deriving noise impact is needed at first. We discuss this aspect in the separated paper.

# Observation 1 : For PC3 Spherical Coverage, discussion on MU definition (whether consider correlation of MUs between grids) and power level assumption for deriving noise impact is needed

# For ACLR, seeing the all input from TE vendors [12],[13] and [14], the test cannot be carried out without relaxation with current assumed minimum TRP. More elaboration needed on the assumed minimum TRP. e.g. the appropriateness of current assumption of minimum TRP( peak EIRP – MBR – ACLR), also need to consider the effect from MPR which is missing in the current assumption. We discuss this aspect in the separated paper.

# Observation 2 : For PC3 ACLR, more careful checking required for assumed minimum TRP

- Conclusion

# Proposal 1 : For PC3 SEM FR2b, no relaxation for the test requirement,  ΔSNR  = 1.0 dB for EIRPmax ≤ 43dBm

# Proposal 2 : For PC3 General Tx Spurious, no relaxation for the test requirement, ΔSNR = 0.41 dB for 6GHz ≤ f  ≤ 80GHz

# Proposal 3 : For PC3 Rx Spurious, ΔSNR = 1.0 dB and relaxation 2.7dB(6 GHz ≤ f ≤ 20 GHz), 12.2dB(20 GHz < f ≤ 40 GHz), 25.6dB(40 GHz < f  ≤ 80 GHz), or ΔSNR = 0.41 dB and relaxation 6.7dB(6 GHz ≤ f ≤ 20 GHz), 16.2dB(20 GHz < f ≤ 40 GHz), 29.6dB(40 GHz < f  ≤ 80 GHz)

# Observation 1 : For PC3 Spherical Coverage, discussion on MU definition (whether consider correlation of MUs between grids) and power level assumption for deriving noise impact is needed

# Observation 2 : For PC3 ACLR, more careful checking required for assumed minimum TRP

# 3.	References

[1] R5-185806, “Testability issue of low PSD test cases in FR2 (2)”, Anritsu, RAN5 NR#3 AdHoc

[2] R5-187273, “Consideration on MU and TT for FR2 low PSD tests”, Anritsu, RAN5#80,

[3] R5-188229, “SNR estimation for priority 1 and 2 FR2 Tx test cases”, Anritsu, RAN5#81

[4] R5-190446, “On the power level to derive SNR for FR2 TRx tests”, Anritsu, RAN5 NR#4 AdHoc

[5] R5-190460, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5 NR#4 AdHoc

[6] R5-190601, “On relaxation of test requirements for Low and High PSD test cases”, Keysight, RAN5 NR#4 AdHoc

[7] R5-190980, “Way forward for SNR estimation assumptions”, Anritsu/Keysight/R&S, RAN5 NR#4 AdHoc

[8] R5-191794, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5#82

[9] R5-192125, “On the SNR for FR2 TRx test cases”, Rohde & Schwarz, RAN5#82

[10] R5-192663, “WF on Noise Impact on RF Measurements for FR2”, Keysight/Anritsu/R&S, RAN5#82

[11] R5-193105, “On the impact from noise for EIRP spherical coverage”, Anritsu, RAN5 NR#5 AdHoc

[12] R5-193104, “SNR estimation update for FR2 priority 1 and 2 TRx TCs”, Anritsu, RAN5 NR#5 AdHoc

[13] R5-192984, “On the SNR for FR2 TRx test cases”, Rohde & Schwarz, RAN5 NR#5 AdHoc

[14] R5-193187, “On noise impact”, Keysight Technology, RAN5 NR#5 AdHoc

[15] R5-193444, “Agreement of noise impact during RAN5 NR#5 AdHoc”, Anritsu /R&S/Keysight, RAN5 NR#5 AdHoc

[16] R5-193483, “Discussion on TT and the impact on UE fail rate for EIRP CDF”, Ericsson et.al, RAN5 NR#5 AdHoc