3GPP TSG-RAN5 Meeting #86-e		R5-200321r1
Electronic Meeting, 17th – 28th February 2020

Agenda item:	5.3.2.17

Source:	Keysight Technologies

Title:	NR FR2 Demodulation MU Table

Document for:	Discussion for Endorsement

# Introduction

This contribution provides a consolidated MU Table for Mode 1 (conditions with external noise) and for Mode 2 (noise free conditions) demodulation test cases.

# Discussion

In RAN5#84, three different contributions discussed MU elements for demodulation test cases [1][2][3]. This contribution proposes a consolidated MU table for Mode 1 demodulation test cases based on previous discussions and endorsements in Table 1.

Table 1: Uncertainty Contributions for Mode 1 Demodulation Test Cases

| UID |  | Uncertainty source | Uncertainty value |  | Distribution of the probability |  | Divisor |  | Standard uncertainty (σ) [dB] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Signal-to-noise ratio uncertainty |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Positioning misalignment |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 2 |  | Measure distance uncertainty |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
| 3 |  | Quality of Quiet Zone |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 4 |  | Mismatch |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 5 |  | Standing wave between the DUT and measurement antenna |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 6 |  | gNB emulator SNR uncertainty |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 7 |  | Phase curvature |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 8 |  | Amplifier uncertainties |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 9 |  | Random uncertainty |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 10 |  | Influence of the XPD |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 11 |  | Insertion Loss Variation |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
| 12 |  | RF leakage (from measurement antenna to the receiver/transmitter) |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 13 |  | Multiple measurement antenna uncertainty |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 14 |  | DUT repositioning |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | Mismatch |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |  |  |
| 16 |  | Amplifier Uncertainties |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |  |  |
| 17 |  | Misalignment of positioning System |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |  |  |
| 18 |  | Uncertainty of the Network Analyzer |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |  |  |
| 19 |  | Uncertainty of the absolute gain of the calibration antenna |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |  |  |
| 20 |  | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |  |  |
| 21 |  | Phase centre offset of calibration antenna |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |  |  |
| 22 |  | Quality of quiet zone for calibration process |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |  |  |
| 23 |  | Standing wave between reference calibration antenna and measurement antenna |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |  |  |
| 24 |  | Influence of the calibration antenna feed cable |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |  |  |
| 25 |  | Insertion Loss Variation |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |  |  |
|  |  | Systematic uncertainties |  |  |  |  |  | Value |  |  |  |
| 26 |  | Impact on non-ideal isolation between branches for the wireless cable mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Signal-to-Noise ratio uncertainty |  |  |  |  |  |  |
| Other contributors affecting test result |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  | gNB emulator fading model impairments |  |  |  | [Normal] | [2.00] |  |  |  |  |  |  |  |
| 28 |  | Test System Frequency Flatness Uncertainty for AWGN |  |  |  | [Actual] | [1.00] |  |  |  |  |  |  |  |
| 29 |  | Result variation due to finite test time |  |  |  | [Actual] | [1.00] |  |  |  |  |  |  |  |

The consolidated MU table for Mode 2 demodulation test cases based on the EIS MU table in [4], specifically Table B.19.2-2, in Table 2.

This contribution is merely intended to define the baseline tables and does not address specific MU elements which needs to be addressed separately.

Table 2: Uncertainty Contributions for Mode 2 Demodulation Test Cases

| UID |  | Uncertainty source |  | Uncertainty value |  | Distribution of the probability |  | Divisor |  |  |  | Standard uncertainty (σ) [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Positioning misalignment |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 2 |  | Measure distance uncertainty |  |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
| 3 |  | Quality of Quiet Zone |  |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 4 |  | Mismatch |  |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 5 |  | Standing wave between the DUT and measurement antenna |  |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 6 |  | gNB uncertainty on absolute level |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 7 |  | Phase curvature |  |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 8 |  | Amplifier uncertainties |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 9 |  | Random uncertainty |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 10 |  | Influence of the XPD |  |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 11 |  | Insertion Loss Variation |  |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
| 12 |  | RF leakage (from measurement antenna to the receiver/transmitter) |  |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 13 |  | Multiple measurement antenna uncertainty |  |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 14 |  | DUT repositioning |  |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
| Stage 1: Calibration measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 |  | Mismatch |  |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 16 |  | Amplifier Uncertainties |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 17 |  | Misalignment of positioning System |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 18 |  | Uncertainty of the Network Analyzer |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 19 |  | Uncertainty of the absolute gain of the calibration antenna |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 20 |  | Positioning and pointing misalignment between the reference antenna and the measurement antenna |  |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
| 21 |  | Phase centre offset of calibration antenna |  |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
| 22 |  | Quality of quiet zone for calibration process |  |  |  | [Actual] |  | [1.00] |  |  |  |  |  |  |
| 23 |  | Standing wave between reference calibration antenna and measurement antenna |  |  |  | [U-shaped] |  | [1.41] |  |  |  |  |  |  |
| 24 |  | Influence of the calibration antenna feed cable |  |  |  | [Normal] |  | [2.00] |  |  |  |  |  |  |
| 25 |  | Insertion Loss Variation |  |  |  | [Rectangular] |  | [1.73] |  |  |  |  |  |  |
|  | Systematic uncertainties |  |  |  |  |  |  |  |  | Value |  |  |
| 26 |  | Systematic error related to beam peak search |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 |  | Impact on non-ideal isolation between branches for the wireless cable mode |  |  |  |  |  |  |  |  |  |  |  |  |
| Other contributors affecting test result |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  | Result variation due to finite test time |  |  |  | [Actual] |  | [1.00] |  |  |  |  |

It is proposed to use Table 1 as a baseline for Mode 1 NR FR2 demodulation test cases.

Proposal 1: Use Table 1 as a baseline for Mode 1 NR FR2 demodulation test cases for baseband-combining implementation

Proposal 2: Capture Mode 1 NR FR2 demodulation test cases MU with external-combining implementation by specifying the difference (applicable contributors, values, divisors) from baseband-combining case.

It is proposed to use Table 2 as a baseline for Mode 2 NR FR2 demodulation test cases.

Proposal 3: Use Table 2 as a baseline for Mode 2 NR FR2 demodulation test cases

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: Use Table 1 as a baseline for Mode 1 NR FR2 demodulation test cases

Proposal 2: Capture Mode 1 NR FR2 demodulation test cases MU with external-combining implementation by specifying the difference (applicable contributors, values, divisors) from baseband-combining case.

Proposal 3: Use Table 2 as a baseline for Mode 2 NR FR2 demodulation test cases

# References

- R5-196582, On FR2 Demodulation MU, Anritsu, 3GPP TSG-RAN5 #84 Meeting, August 2019

- R5-196820, On MU Elements for Demodulation test cases, Keysight Technologies, 3GPP TSG-RAN5 #84 Meeting, August 2019

- R5-196851, Discussion on FR2 Demod MU contributors, Rohde & Schwarz, 3GPP TSG-RAN5 #84 Meeting, August 2019

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases, V16.2.0 (2019-12)