3GPP TSG-RAN5 Meeting #90-e	R5-210836

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.903 | CR | 0208 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on FR2 Blocking Test MU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some FR2 TRx MUs for Blocking Tests is still FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update FR2 Blocking Test(ACS, IBB) MU descriptions and values |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FR2 TRx MUs for Blocking Test remains missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.2.1.35, 2.2.35, B.21, B.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated discussion paper : R5-210838 and R5-210841 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

B.2.1.34	Influence of ETC on EIRP/EIS

This systematic error contributor describes the uncertainty of EIRP/EIS measurements with the ETC enclosure surrounding the DUT, e.g., due to lensing or diffraction effects of the enclosure. This term is assessed by using the absolute difference of EIRP means captured during the Quality of Quiet Zone validation in NTC environment and the EIRP means captured during the Quality of Quiet Zone validation in ETC environment. For NTC validation, the NTC path loss calibration, i.e., the path loss without the ETC enclosure, is applied while for the ETC validation, a path loss calibration is performed with the ETC enclosure surrounding the calibration antenna. Alternate approaches to further improve or completely compensate this systematic error are FFS.

B.2.1.35	Influence of offset antenna for blocker signal

This MU term describes the additional uncertainty caused by using offset antenna for blocker signal for FR2 blocking test cases. The cause of additional MU using offset antenna is the difference of UE antenna’s gain between beam peak direction and offset beam peak direction, which will cause the error for the ACS or IBB performance requirement which is given by the power ratio of the wanted signal power and blocker signal power. Such difference of the UE antenna gain can be compensated by increasing the blocker signal power by the measured EIS difference at beam peak direction and at offset beam peak. Despite this compensation, there still is a residual error corresponding to the antenna gain difference due to different frequency of wanted and blocker signal. Table 2.1.35-1 summarizes the residual error after compensation for various offset angle assumption.

Table 2.1.35-1 Residual error when offset antenna is used for FR2 blocking test

| offset angle[deg] | Mean error[dB] | Std.dev[dB] | Mean error[dB] | Std.dev[dB] |
| --- | --- | --- | --- | --- |
|  | 2x8 Assumption |  | 1x4 Assumption |  |
| 0 | 0.000 | 0.000 | 0.000 | 0.000 |
| 0.5 | 0.001 | 0.001 | 0.000 | 0.000 |
| 1 | 0.004 | 0.003 | 0.001 | 0.001 |
| 1.5 | 0.010 | 0.006 | 0.002 | 0.002 |
| 2 | 0.018 | 0.011 | 0.004 | 0.003 |
| 2.5 | 0.028 | 0.018 | 0.006 | 0.004 |
| 3 | 0.041 | 0.026 | 0.009 | 0.006 |
| 3.5 | 0.057 | 0.036 | 0.013 | 0.009 |
| 4 | 0.075 | 0.048 | 0.016 | 0.011 |
| 4.5 | 0.096 | 0.062 | 0.021 | 0.015 |
| 5 | 0.120 | 0.078 | 0.026 | 0.018 |
| 5.5 | 0.147 | 0.096 | 0.031 | 0.022 |
| 6 | 0.178 | 0.117 | 0.038 | 0.026 |
| 6.5 | 0.213 | 0.142 | 0.044 | 0.031 |
| 7 | 0.252 | 0.170 | 0.051 | 0.036 |
| NOTE 1 : For MU assessment of the test system, the MU values should be taken from the row corresponding to the test system’s offset antenna angle. NOTE 2 : Mean error should be counted as systematic offset and Std.dev should be counted as a random uncertainty in the MU budget table. NOTE 3 : For PC3 UE testing, the values for 2x8 Assumption should be used. |  |  |  |  |

<<Unchaged sections skipped>>

B.2.2.34	Influence of ETC on EIRP/EIS

See B.2.1.34.

B.2.2.35	Influence of offset antenna for blocker signal

See B.2.1.35.

<<Unchaged sections skipped>>

B.21	Adjacent Channel Selectivity

Following tables summarize the MU threshold for Adjacent Channel Selectivity measurement. The origin MU values for different test setups with varies parameters can be found in following subclauses.

Table B.19-1: MU threshold for Adjacent Channel Selectivity

| Power Class | Frequency | MBW | Power | Threshold MU value (NOTE 1) |
| --- | --- | --- | --- | --- |
| PC3 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | 8.04 |
|  | 32.125GHz < f <= 40.8GHz |  |  | 8.04 |
| PC1 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | FFS |
|  | 32.125GHz < f <= 40.8GHz |  |  | FFS |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.21.2-2 for PC3 UEs |  |  |  |  |

B.21.1	Uncertainty budget format and assessment for DFF

FFS

B.21.2	Uncertainty budget format and assessment for IFF

Table B.21.2-1:

void

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D

-	The uncertainty assessment has been derived for the case of Quiet Zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz, 40.8GHz}, [P = maximum output power].

-	The uncertainty assessment for ACS is provided in Table B.21.2-2 for PC3 UEs.

Table B.21.2-2: Uncertainty assessment for Adjacent Channel Selectivity measurement (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) for PC3 UEs

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| 1 | Wanted Signal Absolute Power Uncertainty (NOTE1) | 5.19 | NOTE3 | 1.96 | 2.65 |
| 2 | Blocker Signal Absolute Power Uncertainty (NOTE1) | 5.19 | NOTE3 | 1.96 | 2.65 |
| 3 | Influence of offset antenna (Std.Dev) (NOTE2) | 0.00 | Actual | 1.00 | 0.00 |
|  | Systematic uncertainties |  |  |  | Value |
| 28 | Blocker ACLR Effect |  |  |  | 0.70 |
| 29 | Influence of offset antenna (Mean Error) (NOTE2) |  |  |  | 0.00 |
| Total measurement uncertainty |  |  |  |  | Value |
| Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 8.04 |
| NOTE 1: EIS expanded uncertainity in Table B.19.2-2 NOTE 2: These values should be taken from Table 2.1.35-1 if offset antenna is used. For MTSU derivation purpose, these values are set to 0.0. NOTE 2: Divisor of 1.96 is applied as the uncertainty value is based on 95% confidence level k=1.96. |  |  |  |  |  |

B.22	In-Band Blocking

Following tables summarize the MU threshold for In-band Blocking measurement. The origin MU values for different test setups with varies parameters can be found in following subclauses.

Table B.19-1: MU threshold for In-band Blocking

| Power Class | Frequency | MBW | Power | Threshold MU value (NOTE 1) |
| --- | --- | --- | --- | --- |
| PC3 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | 8.04 |
|  | 32.125GHz < f <= 40.8GHz |  |  | 8.04 |
| PC1 | 23.45GHz <= f <= 32.125GHz | BW <= 400MHz | P = Max Output Power | FFS |
|  | 32.125GHz < f <= 40.8GHz |  |  | FFS |
| NOTE 1: Total Expanded MU for IFF for Quiet Zone size ≤ 30cm in Table B.22.2-2 for PC3 UEs |  |  |  |  |

B.22.1	Uncertainty budget format and assessment for DFF

FFS

B.22.2	Uncertainty budget format and assessment for IFF

Table B.22.2-1:

void

The uncertainty assessment tables are organized as follows:

-	For the purpose of uncertainty assessment, the radiating antenna aperture of the DUT is denoted as D

-	The uncertainty assessment has been derived for the case of Quiet Zone size ≤ [30 cm], f = {23.45GHz, 32.125GHz, 40.8GHz}, [P = maximum output power].

-	The uncertainty assessment for IBB is provided in Table B.22.2-2 for PC3 UEs.

Table B.22.2-2: Uncertainty assessment for In-band Blocking measurement (f=23.45GHz, 32.125GHz, 40.8GHz, Quiet Zone size ≤ 30 cm) for PC3 UEs

| UID | Uncertainty source | Uncertainty value | Distribution of the probability | Divisor | Standard uncertainty (σ) [dB] |
| --- | --- | --- | --- | --- | --- |
| 1 | Wanted Signal Absolute Power Uncertainty (NOTE1) | 5.19 | NOTE3 | 1.96 | 2.65 |
| 2 | Blocker Signal Absolute Power Uncertainty (NOTE1) | 5.19 | NOTE3 | 1.96 | 2.65 |
| 3 | Influence of offset antenna (Std.Dev) (NOTE2) | 0.00 | Actual | 1.00 | 0.00 |
|  | Systematic uncertainties |  |  |  | Value |
| 28 | Blocker ACLR Effect |  |  |  | 0.70 |
| 29 | Influence of offset antenna (Mean Error) (NOTE2) |  |  |  | 0.00 |
| Total measurement uncertainty |  |  |  |  | Value |
| Expanded uncertainty (23.45GHz <= f <= 32.125GHz) (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  | 8.04 |
| NOTE 1: EIS expanded uncertainity in Table B.19.2-2 NOTE 2: These values should be taken from Table 2.1.35-1 if offset antenna is used. For MTSU derivation purpose, these values are set to 0.0. NOTE 2: Divisor of 1.96 is applied as the uncertainty value is based on 95% confidence level k=1.96. |  |  |  |  |  |

B.23

<<End of change>>