3GPP TSG-RAN5 Meeting #90-e	R5-210082r2

# Electronic meeting, 22nd February – 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0451 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of FR2 DL 256QAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_DL256QAM_FR2-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test requirements related to the feature of 256QAM DL transmission in FR2 are not specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of test requirements related to the feature of 256QAM DL transmission in FR2. - Maximum input level requirement for 256QAM added in clause 7.4.3 - Max input level test requirement for 256QAM added in clause 7.4.5 - Fixed Reference Channel for 256QAM added as new clause A.3.3.5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 256QAM DL transmission in FR2 cannot be tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4, 7.4.3, 7.4.5, A.3.3.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Single CR is submitted covering changes to both receiver clause 7 (agenda 5.3.26.3.2) and Annex A (agenda 5.3.26.3.3). New DL reference measurement channel for 256QAM is introduced in both clauses. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Remove changes to CA related clauses which are introduced in R5-210492 (CR 0460) R2: Add editor’s note that relaxation of DL power for 256 QAM is FFS |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 7.4	Maximum input level

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- Measurement uncertainty is FFS.

- UL power level configuration is TBD

- Relaxation of DL power for 256 QAM is FFS.

7.4.1	Test purpose

Maximum input level tests the UE's ability to receive data with a given average throughput for a specified reference measurement channel, under conditions of high signal level, ideal propagation and no added noise.

A UE unable to meet the throughput requirement under these conditions will decrease the coverage area near to a g-NodeB.

7.4.2	Test applicability

The minimum conformance requirements in this test case are not testable due to maximum input level unachievable in IFF OTA test setup. Other test setups have not been analysed. Thus the test case will not be tested as part of UE conformance testing.

NOTE: As a result TC 7.4 has not been included in the test case applicability table 4.1.2-1, TS 38.522. This does not preclude the test from being used for R&D or other purposes if deemed useful to all types of NR UE release 15 and forward.

7.4.3	Minimum conformance requirements

The maximum input level is defined as the maximum mean power, for which the throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

The maximum input level is defined as a directional requirement. The requirement is verified in beam locked mode in the direction where peak gain is achieved.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with parameters specified in Table 7.4.3-1. The requirement is verified with the test metric of EIS (Link=RX beam peak direction, Meas=Link angle).

Table 7.4.3-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in transmission bandwidth configuration | dBm | -25 (NOTE 2) -27 (NOTE 3) |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in subclause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 2: Reference measurement channel is specified in Annex A.3.3.2: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A. NOTE 3: Reference measurement channel is specified in Annex A.3.3.5: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern as described in Annex A. |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-2 [3] clause 7.4.

7.4.4	Test description

7.4.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 7.4.4.1-1. The details of the uplink and downlink reference measurement channels (RMC) are specified in Annexes A.2 and A.3. The details of the OCNG patterns used are specified in Annex A.5. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | 120kHz |  |
| Test Parameters for Channel Bandwidths |  |  |  |
| Downlink Configuration |  | Uplink Configuration |  |
| Modulation | RB allocation | Modulation | RB allocation |
| CP-OFDM QPSK | NOTE1 | DFT-s-OFDM QPSK | NOTE2 |
| NOTE 1: The specific configuration of downlink RB allocation is defined in Table 7.3.2.4.1-2. NOTE 2: The specific configuration of uplink RB allocation is defined in Table 7.3.2.4.1-3. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to AnnexG.

4.	The DL and UL Reference Measurement channels are set according to Table 7.4.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message content are defined in clause 7.4.4.3.

7.4.4.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format [1_1] for C_RNTI to transmit the DL RMC according to Table 7.4.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

2.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format [0_1] for C_RNTI to schedule the UL RMC according to Table 7.4.4.1-1. Since the UL has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

3.	Set the Downlink signal level for θ-polarization to the value as defined in Table 7.4.5-1.

4.	Set the UE in the Rx beam peak direction found with a 3D EIS scan as performed in Annex K.1.2. Allow at least BEAM_SELECT_WAIT_TIME (NOTE) for the UE Rx beam selection to complete.

5.	Send Uplink power control commands to the UE (less or equal to 1dB step size should be used), to ensure that the UE output power is within [TBD] dB of the target power level in Table 7.4.5-1, for at least the duration of the throughput measurement.

6.	SS activates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.2 using condition Rx Only.

7.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.2.

8.	SS deactivates the UE Beamlock Function (UBF) by performing the procedure as specified in TS 38.508-1 [10] clause 4.9.3.

9.	Repeat steps from 3 to 8, for the downlink signal from φ-polarization.

10.	Compare the results for both the θ-polarization and φ-polarization against the requirement. If either result meets the requirements, pass the UE.

NOTE:	The BEAM_SELECT_WAIT_TIME default value is defined in Annex K.1.1.

7.4.4.3	Message contents

Message contents are according to TS 38.508-1 [10] subclause 4.6.

7.4.5	Test requirement

The throughput measurement derived in test procedure shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A with parameters specified in Tables 7.4.5-1.

Table 7.4.5-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -51 (NOTE 2,3) for band n257, n258 and n261 -59 (NOTE 2,3) for band n260 -53 (NOTE 3,4) for band n257, n258 and n261 -61 (NOTE 3,4) for band n260 |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below the PUMAX,f,c as defined in subclause 6.2.4, with uplink configuration specified in Table 7.3.2.3.1-2. NOTE 2: Reference measurement channel is specified in Annex A.3.3.2: QPSK, R=1/3 variant with one sided dynamic OCNG Pattern as described in Annex A. NOTE 3: The test requirements deviate from minimum requirements by 26dB relaxation for 24.25 ~ 29.5 GHz and 34 dB relaxation for 37 ~ 40 GHz. NOTE 4: Reference measurement channel is specified in Annex A.3.3.5: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern as described in Annex A. |  |  |  |  |  |

-------- Skipped unchanged clauses --------

### A.3.3.4	FRC for receiver requirements for 64QAM

Table A.3.3.4-1 Fixed Reference Channel for Receiver Requirements (SCS 60 kHz, TDD)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 |
| Allocated resource blocks |  | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame |  | 23 | 23 | 23 |
| MCS index |  | 19 | 19 | 19 |
| Modulation |  | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2} for i from {1,…,79} | Bits | 20496 | 40976 | 81976 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2} for i from {1,…,79} | CBs | 3 | 5 | 10 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2} for i from {1,…,79} | Bits | 40986 | 81972 | 163944 |
| Max. Throughput averaged over 1 frame | Mbps | 49.190 | 98.343 | 196.742 |
| Note 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 3: SS/PBCH block is transmitted in slot 0 with periodicity 20 ms Note 4: Slot i is slot index per 2 frames Note 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. |  |  |  |  |

Table A.3.3.4-2 Fixed Reference Channel for Receiver Requirements (SCS 120 kHz, TDD)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 47 | 47 | 47 | 47 |
| MCS index |  | 19 | 19 | 19 | 19 |
| Modulation |  | 64QAM | 64QAM | 64QAM | 64QAM |
| Target Coding Rate |  | 1/2 | 1/2 | 1/2 | 1/2 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 9992 | 20496 | 40976 | 81976 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 2 | 3 | 5 | 10 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 19872 | 40986 | 81972 | 163944 |
| Max. Throughput averaged over 1 frame | Mbps | 47.962 | 98.381 | 196.685 | 393.485 |
| Note 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. Note 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). Note 3: SS/PBCH block is transmitted in slot with periodicity 20 ms Note 4: Slot i is slot index per 2 frames Note 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. |  |  |  |  |  |

#### A.3.3.5	FRC for receiver requirements for 256QAM

Table A.3.3.5-1 Fixed Reference Channel for Receiver Requirements (SCS 60 kHz, TDD)

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 |
| Allocated resource blocks |  | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame |  | 23 | 23 | 23 |
| MCS index |  | 24 | 24 | 24 |
| Modulation |  | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | Bits | 44040 | 88064 | 176208 |
| Transport block CRC | Bits | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | CBs | 6 | 11 | 21 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,79} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,79} | Bits | 54648 | 109296 | 218592 |
| Max. Throughput averaged over 1 frame | Mbps | 101.292 | 202.547 | 405.278 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. |  |  |  |  |

Table A.3.3.5-2 Fixed Reference Channel for Receiver Requirements (SCS 120 kHz, TDD)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 50 | 100 | 200 | 400 |
| Subcarrier spacing configuration |  | 3 | 3 | 3 | 3 |
| Allocated resource blocks |  | 32 | 66 | 132 | 264 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 47 | 47 | 47 | 47 |
| MCS index |  | 24 | 24 | 24 | 24 |
| Modulation |  | 256QAM | 256QAM | 256QAM | 256QAM |
| Target Coding Rate |  | 4/5 | 4/5 | 4/5 | 4/5 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 21504 | 44040 | 88064 | 176208 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | CBs | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | CBs | 3 | 6 | 11 | 21 |
| Binary Channel Bits Per Slot |  |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,159} | Bits | N/A | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,159} | Bits | 26496 | 54648 | 109296 | 218592 |
| Max. Throughput averaged over 1 frame | Mbps | 101.069 | 206.988 | 413.901 | 828.178 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: PTRS is configured on symbols containing PDSCH with 1 port, per 2PRB in frequency domain, per symbol in time domain. Overhead for TBS calculation is assumed to be 6. |  |  |  |  |  |

-------- End of changes --------